Automotive Plumbing Catalog

June 2017

Metal Tubing Tube Nuts Weld-On Studs Tube Unions

Clips and Brackets Bulkhead Nuts Tees

Thread Adapters Bump Tube Adapters

Transmission Line Adapters Copper Washers

Banjo Bolts Banjo Adapters

Weld-On Banjo Heads Bleed Screws

Fitting Caps and Plugs Assortment Kits

Tube Straighteners Cutters

Tube Benders Flaring Tools Specialty Tools

Fuel Line and Fittings

## Tube and Hose	BrakeQuip

90/10 Cunifer Tubing

For the ultimate combination of corrosion resistance and ease of use, choose the EziBend family of tubing. EziBend tubing meets UNS C70600 requirements for material composition and is compliant with

US Department of Transportation and Federal Motor Vehicle Safety Standards.

Volvo, Audi, Porsche, Rolls Royce, Lotus, Aston Martin, and other performance automobile makers use C70600 tubing. In the replacement market, C70600 tubing is the preferred tubing for brake lines, fuel, and transmission systems.

Durability

Unlike other similarly-priced tubing that attempts to protect the core material with a plating or coating, EziBend tubing is fabricated from a special alloy that is inherently rust and corrosion proof. There are no coatings to fail. EziBend will remain corrosion-proof after tube formation, scratches during installation, and nicks from road debris.

Ease of use

EziBend is strong, but easy to work with. It straightens, cuts, flares, and bends smoothly. And the same properties that make EziBend simple to shape allow it to achieve a superior sealing surface against fittings.

EziBend 2

- 0.028” wall thickness

- Polishes to a brilliant finish

- Seamless construction

- Ready for retail in pre-packaged coils

Original EziBend

- 0.028” wall thickness

- Polishes to a brilliant finish

- Made in the USA

- Unpackaged coils

| Part Number | OD | Coil Length |  | Part Number | OD | Coil Length |
| --- | --- | --- | --- | --- | --- | --- |
| T187-25SE | 3/16” | 25ft |  | T187-25CN | 3/16” | 25ft |
| T187-100SE | 3/16” | 100ft |  | T187-100CN | 3/16” | 100ft |
| T250-25SE | 1/4” | 25ft |  | T250-25CN | 1/4” | 25ft |
| T250-100SE | 1/4” | 100ft |  | T250-100CN | 1/4” | 100ft |
| T312-25SE | 5/16” | 25ft |  | T312-25CN | 5/16” | 25ft |
| T375-25SE | 3/8” | 25ft |  | T375-25CN | 3/8” | 25ft |

# On the right side of the standards

In cunifer tubing, nickel improves strength and corrosion resistance while allowing the tube to remain ductile. EziBend and EziBend 2 stood out in independent lab tests against three of the largest cunifer suppliers in the US and the UK.

| ”Wholesale” Tubing (US) |  | ”Parts Store” Tubing (US) | Leading UK Tubing | UNS C70600 | EziBend | EziBend 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Nickel Content | 8.85% | 8.92% | 8.89% | 9.0%–11.0% | 10.30% | 10.40% |

2

Innovative Solutions for Automotive Tubes and Hoses	Tube and Hose

316L Stainless Tubing

The pinnacle of automotive plumbing, our 316L marine-grade tubing has the highest level of corrosion protection combined with the stunning appearance of polished stainless steel.

Corrosion resistance

316L contains molybdenum, a corrosion resistor that is not found in 304 grade stainless.

Proper wall thickness

Thick-walled 304 stainless tubing meant for heavy industrial applications will damage automotive tube-working tools. Our 0.028”-walled tubing is strong enough for automotive, marine, and light industrial applications, yet easy to work using DIY automotive tools.

Soft-annealed

BrakeQuip 316L is soft-annealed, a heat treatment process which makes straightening, cutting, bending, and flaring far easier than with non-annealed stainless.

See coil straighteners and benders on page 14 and 15,

flaring tools on page 16, and stainless fittings on page 20.

| Part Number | OD | Coil Length |
| --- | --- | --- |
| T187-20SS | 3/16” | 20ft |
| T250-20SS | 1/4” | 20ft |
| T312-20SS | 5/16” | 20ft |
| T375-20SS | 3/8” | 20ft |

See inside back cover for brake and power steering hoses.

Remote Reservoir Hose

Part Number	ID	OD

BQ320	1/4”	7/16”

BQ321	5/16”	1/2”

BQ322	3/8”	9/16”

BQ313-SV Heat Shield

One size fits all. 5ft rolls.

Remote Reservoir

Spring Guards

Part Number			ID		OD			Material BQ202	11.5mm	16. 5mm		Black Nylon BQ231		3/16”			Stainless Steel

BQ232	1/4”	Stainless Steel

Remote Reservoir Hose

Master Cylinder

3

Other Tube and Hose Products

##### BrakeQuip can manufacture custom tubing OD, wall thickness, and coil length for large volume opportunities. For more information, contact sales@brakequip.com.

| Part Number | Thread | Stud Length | Flare | Hex | Tube Size |
| --- | --- | --- | --- | --- | --- |
| BQ4276 | 3/8-20 BSF | 14mm | 45° | 3/8 | 3/16” |
| BQ5555 | 3/8-24 UNF | 9.1mm | 45° | 3/8 | 3/16” |
| BQ5555SS | 3/8-24 UNF | 9.1mm | 45° | 3/8 | 3/16” |
| BQ4285 | 3/8-24 UNF | 17mm | 45° | 3/8 | 3/16” |
| BQ4277 | 7/16-20 UNF | 18mm | 45° | 7/16 | 1/4” |
| BQ6118 | 7/16-20 UNF | 17.5mm | 45° | 11mm | 3/16” |
| BQ5842 | 7/16-24 UNS | 9.1mm | 45° | 11mm | 3/16” |
| BQ5842SS | 7/16-24 UNS | 9.1mm | 45° | 11mm | 3/16” |
| BQ2577 | 7/16-24 UNS | 12.5mm | 45° | 7/16 | 1/4” |
| BQ2577SS | 7/16-24 UNS | 12.5mm | 45° | 7/16 | 1/4” |
| BQ2578 | 1/2-20 UNF | 10.2mm | 45° | 1/2 | 5/16” |
| BQ9184 | 1/2-20 UNF | 10.2mm | 45° | 1/2 | 3/16” |
| BQ9187 | 1/2-20 UNF | 14mm | 45° | 1/2 | 1/4” |
| BQ9185 | 9/16-18 UNF | 14mm | 45° | 9/16 | 3/16” |
| BQ9186 | 9/16-18 UNF | 14mm | 45° | 9/16 | 1/4” |
| BQ9181 | 5/8-18 UNF | 10.2mm | 45° | 5/8 | 3/8” |
| BQ4510 | M9 × 1 | 14mm | 45° | 10mm | 3/16” |
| BQ4511 | M10 × 1 | 11.6mm | 45° | 11mm | 3/16” |
| BQ4511LT | M10 × 1 | 14mm | 45° | 11mm | 3/16” |
| BQ4511SS | M10 × 1 | 11.6mm | 45° | 11mm | 3/16” |
| BQ72 | M10 × 1 | 12.6mm | DIN | 11mm | 3/16” |
| BQ4512 | M10 × 1.25 | 11.6mm | 45° | 11mm | 3/16” |
| BQ9188 | M10 × 1.5 | 11.6mm | 45° | 11mm | 3/16” |
| BQ9189 | M11 × 1 | 15mm | 45° | 11mm | 3/16” |
| BQ9190 | M11 × 1.25 | 15mm | 45° | 11mm | 3/16” |
| BQ9191 | M11 × 1.5 | 13.1mm | 45° | 13mm | 3/16” |
| BQ9180 | M12 × 1 | 11.6mm | 45° | 13mm | 3/16” |
| BQ4513 | M12 × 1 | 11.6mm | 45° | 13mm | 1/4” |
| BQ4513LT | M12 × 1 | 14.6mm | 45° | 13mm | 1/4” |
| BQ4513SS | M12 × 1 | 11.6mm | 45° | 12mm | 1/4” |
| BQ9169 | M12 × 1 | 14.9mm | DIN | 13mm | 1/4” |
| BQ9182 | M12 × 1.5 | 11.6mm | 45° | 13mm | 1/4” |
| BQ9192 | M12 × 1.25 | 11.6mm | 45° | 13mm | 3/16” |
| BQ9157 | M12 × 1.25 | 11.6mm | 45° | 13mm | 1/4” |
| BQ9193 | M12 × 1.5 | 11.6mm | 45° | 13mm | 3/16” |
| BQ9194 | M13 × 1 | 15mm | 45° | 13mm | 3/16” |
| BQ9195 | M13 × 1.25 | 15mm | 45° | 13mm | 3/16” |
| BQ9196 | M13 × 1.5 | 15mm | 45° | 13mm | 3/16” |
| BQ9183 | M14 × 1.5 | 11.6mm | 45° | 15mm | 1/4” |
| BQ9197 | M14 × 1.5 | 11.6mm | 45° | 15mm | 5/16” |
| BQ9149 | M14 × 1.5 | 15mm | DIN | 16mm | 8mm |
| BQ9198 | M16 × 1.5 | 11.6mm | 45° | 17mm | 5/16” |
| BQ9166 | M16 × 1.5 | 16mm | 45° | 17mm | 3/8” |
| BQ9148 | M16 × 1.5 | 13.2mm | DIN | 16mm | 10mm |
| BQ9199 | M18 × 1.5 | 16mm | 45° | 18mm | 5/16” |

Male Tube Nuts

See page 13 for assortment kits BQ7005 (metric tube nuts) and BQ7006 (imperial tube nuts).

Stud Length

Tube Size

Hex

See page 13 for assortment kits BQ7005 (metric tube nuts) and BQ7006 (imperial tube nuts).

Female Tube Nuts

| Part Number | Thread | Thread Length | Seat | Hex | Tube Size |
| --- | --- | --- | --- | --- | --- |
| BQ55 | 3/8-24 UNF | 7.5mm | 37° | 7/16” | 3/16” |
| BQ9176 | 3/8-24 UNF | 5.6mm | 45° | 7/16” | 3/16” |
| BQ57 | 7/16-20 UNF | 5.8mm | 37° | 1/2” | 1/4” |
| BQ9177 | 1/2-20 UNF | 10mm | 45° | 5/8” | 5/16” |
| BQ9201 | 9/16-18 UNF | 7.5mm | 37° | 11/16” | 3/8” |
| BQ9239 | 9/16-18 UNF | 8.6mm | 45° | 11/16” | 3/8” |
| BQ9207 | 9/16-18 UNF | 6.1mm | o-ring pilot | 5/8” | 3/8” |
| BQ9219 | 5/8-18 UNF | 8.6mm | 45° | 13/16” | 3/8” |
| BQ9175 | M10 × 1 | 9mm | 45° | 13mm | 3/16” |
| BQ9174 | M12 × 1 | 9mm | 45° | 16mm | 3/16” |
| BQ9179 | M12 × 1 | 9mm | 45° | 16mm | 1/4” |
| BQ9156 | M12 × 1.25 | 8mm | 45° | 16mm | 1/4” |
| BQ9178 | M12 × 1.5 | 9mm | 45° | 16mm | 1/4” |
| BQ9208 | M14 × 1.5 | 10.9mm | 37° | 17mm | 10mm |
| BQ9209 | M16 × 1.5 | 10.9mm | 37° | 19mm | 10mm |
| BQ9226 | M16 × 1.5 | 11mm | 45° | 19mm | 3/8” |
| BQ9210 | M18 × 1.5 | 14.4mm | 37° | 21mm | 12mm |

Thread Length

Tube Size

See page 10 for copper seat seals (flare gaskets).

| Part Number | Thread | Type | Tube Size | Stud Length | Hex |
| --- | --- | --- | --- | --- | --- |
| BQ170 | 9/16-18 UNF | -6 JIC | 3/8” | 14.1mm | 5/8” |
| BQ171 | 5/8-18 UNF | 45° inverted | 3/8” | 15.7mm | 11/16” |
| BQ172 | 3/4-16 UNF | -8 JIC | 1/2” | 16.7mm | 13/16” |
| BQ173 | 7/8-14 UNF | -10 JIC | 5/8” | 22.6mm | 15/16” |
| BQ174 | 1 1/16-14 UNS | 45° inverted | 3/4” | 25.4mm | 1 1/16” |

BQ141 Swivel Elbow – 90°

3/8-24 UNF — 3/8-24 UNF

Weld-On Fittings

Handy for tight spots that require a connection with an inverted seat.

Aluminum

Steel

Stud Length

Tube Size

| Part Number | Thread | Type | Tube Size | Stud Length | Hex |
| --- | --- | --- | --- | --- | --- |
| BQ153 | 3/8-24 UNF | -3 JIC | 3/16” | 12.2mm | 7/16” |
| BQ226 | 9/16-18 UNF | -6 JIC | 3/8” | 14.1mm | 5/8” |
| BQ228 | 9/16-18 UNF | -6 JIC | 1/2 ” | 14.1mm | 11/16” |
| BQ227 | 9/16-18 UNF | -6 JIC | 10mm | 14.1mm | 5/8” |

BQ33

Pre-fluxed Silver Brazing Rod

pack of 8

BQ34

Pre-fluxed Aluminum Brazing Rod

pack of 5

For professional brazing of steel, stainless, brass, or any combination of these metals. Designed for brazing high-pressure fittings. No cadmium. 18” long.

Brazing Rods

Female Unions

| Part Number | Thread | Thread Length | Seat | Hex | Overall Length |
| --- | --- | --- | --- | --- | --- |
| BQ110 | 3/8-24 UNF | 5.16mm | 45° | 1/2” | 22.2mm |
| BQ115 | 3/8-24 UNF | 8mm | Bubble | 1/2” | 25mm |
| BQ111 | 7/16-24 UNS | 5.16mm | 45° | 9/16” | 22.2mm |
| BQ111SS | 7/16-24 UNS | 5.16mm | 45° | 9/16” | 22.2mm |
| BQ112 | 1/2-20 UNF | 5.94mm | 45° | 5/8” | 25mm |
| BQ119 | 9/16-18 UNF | 5.94mm | 45° | 3/4” | 27mm |
| BQ113 | 5/8-18 UNF | 6.76mm | 45° | 3/4” | 27mm |
| BQ114 | M10 × 1 | 7mm | DIN | 13mm | 25mm |
| BQ117 | M10 × 1 | 7.95mm | 45° | 13mm | 24mm |
| BQ116 | M12 × 1 | 7.95mm | 45° | 16mm | 25mm |
| BQ147 | M12 × 1 | 9mm | DIN | 16mm | 27mm |

See page 13 for assortment kit BQ7009.

45° Inverted

Bubble or DIN

Tube Clamps – white nylon

BQ249 3/16” BQ250 1/4” FL35	5/16”

FL34	3/8”

port here

7/16-24 UNS

internal threads

BQ148 Bulkhead Union 5/8-18 UNF external thread 45° inverted seat, 19mm hex

Secure with a BQ13 bulkhead nut - next page

port here

BQ222 Tandem Trailer Fitting Universal

Four ports: 3/8-24 UNF, inverted seat

Secure with BQ13 bulkhead nut – next page Block off excess ports with BQ19 plugs – page 12

BQ293SS -3 JIC Union, Stainless

3/8-24 UNF, 7/16” hex

45.9mm overall length

Requires BQ78 nut for bulkhead mount

BQ292 -3 JIC Union

BQ292SS Stainless version

3/8-24 UNF, 7/16” hex

30.7mm overall length

Male Unions

BQB06   Universal Bracket

Clips and Brackets

19mm hole, 5mm mount holes

BQB244 Universal Bracket

17mm hole

6.8mm mount holes

BQ3052

##### Universal Clip

BQ3052SS

Universal Clip - Stainless

BQ3053

Clip - suits Toyota

BQ71

Bracket Retaining Screws

pack of 50

| Part Number | Thread | Hex Size | Thickness |
| --- | --- | --- | --- |
| BQ78 | 3/8-24 UNF | 1/2” | 5 mm |
| BQ80 | 7/16-20 UNF | 5/8” | 6 mm |
| BQ83 | 1/2-20 UNF | 3/4” | 6 mm |
| BQ13 | 5/8-18 UNF | 15/16” | 8 mm |
| BQ77 | 11/16-18 UNS | 1” | 8 mm |
| BQ84 | 3/4-16 UNF | 1 1/16” | 8 mm |
| BQ75 | 7/8-14 UNF | 1 1/8” | 8 mm |
| BQ89 | 7/8-18 UNS | 1 1/8” | 8 mm |
| BQ79 | M10 × 1 | 13 mm | 5 mm |
| BQ81 | M12 × 1 | 19 mm | 6 mm |
| BQ82 | M12 × 1.5 | 19 mm | 6 mm |
| BQ87 | M16 × 1.5 | 22 mm | 7 mm |
| BQ86 | M18 × 1.5 | 25 mm | 8 mm |
| BQ85 | M20 × 1.5 | 27 mm | 8 mm |

See page 13 for assortment kit BQ7002.

Tees

| Part Number | Thread | Seat | Diameter | Mount | Mount Hole | Thickness |
| --- | --- | --- | --- | --- | --- | --- |
| BQ93A | 1/8-27 NPT | NPT | 31.75mm | Bolt | 8.2mm | 14mm |
| BQ264 BQ56A BQ56ASS BQ90A BQ94A BQ92A | 3/8-24 UNF 3/8-24 UNF 3/8-24 UNF 3/8-24 UNF 7/16-20 UNF 7/16-24 UNS | 45° 45° 45° 45° Bubble 45° | T shape 28.6mm 28.6mm 31.75mm 31.75mm 31.75mm | none Bolt Bolt Bolt Bolt Bolt | none 8.2mm 8.2mm 8.2mm 8.2mm 8.2mm | 1/2” 1/2” 1/2” 1/2” 14mm 14mm |
| BQ91A BQ95A | M10 × 1 M10 × 1 | 45° DIN | 31.75mm 31.75mm | Bolt Bolt | 8.2mm 7.2mm | 1/2” 14mm |

NPT

3/8-24 UNF

Inverted seat

M10 × 1 port 7/16-24 UNS port

Male

M10 × 1 tube nut, DIN flare

BQ265 NPT Tee

7/16-24 UNS

BQ7812 Tee with Bracket

BQ378 Swivel Tee

Two 3/8-24 UNF ports One 1/8-27 NPT port

BQ177SS Tee

3/8-24 UNF, inverted seat 47mm diameter

1/2” thick

Bulkhead Nuts

Female

| Part Number | Thread | JIC | Mount | Mount Hole | Diameter | Thickness |
| --- | --- | --- | --- | --- | --- | --- |
| BQ360 | 3/8-24 UNF | -3 | Bolt | 8.2mm | 31.75mm | 1/2” |
| BQ361 | 7/16-20 UNF | -4 | Bolt | 8.2mm | 31.75mm | 1/2” |

See page 13 for assortment kit BQ7004.

Male-Female Adapters

| Part Number | Port Thread | Port Thread Length | Port Seat | Tube Size | Stud Thread | Stud Length | Stud Fit | Hex |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BQ247 | 1/8-27 NPT | 6.92mm | NPT | — | 3/8-24 UNF | 11.2mm |  | 9/16” |
| BQ248 | 1/8-27 NPT | 6.92mm | NPT | — | 1/2-20 UNF | 11mm |  | 9/16” |
| BQ187 | 1/8-27 NPT | 6.92mm | NPT | — | M10 × 1.5 | 11mm |  | 9/16” |
| BQ48 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 1/8-27 NPT | 9.97mm | 3/16" | 1/2” |
| BQ156 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 3/8-24 UNF | 12mm | 3/16" | 9/16” |
| BQ47 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 7/16-20 UNF | 12mm | 3/16" | 9/16” |
| BQ46 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 7/16-24 UNS | 12mm | 3/16" | 9/16” |
| BQ9300 | 3/8-24 UNF | 6.19mm | 45° | 3/16” | 1/2-20 UNF | 12mm | 3/16" | 5/8” |
| BQ120 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 1/2-20 UNF | 12mm | 5/16" | 5/8” |
| BQ124 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 9/16-18 UNF | 12mm | 3/16" | 11/16” |
| BQ262 | 3/8-24 UNF | 9mm | Bubble | 3/16” | 9/16-18 UNF | 12mm | 3/16" | 11/16” |
| BQ121 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | 9/16-20 UNS | 12mm | 3/16" | 11/16” |
| BQ49 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | M10 × 1 | 12.6mm | 3/16" | 9/16” |
| BQ125 | 3/8-24 UNF | 9mm | Bubble | 3/16” | M10 × 1 | 12mm | 3/16" | 13mm |
| BQ383 | 3/8-24 UNF | 5.16mm | 45° | 3/16” | M12 × 1 | 12mm | 3/16" | 16mm |
| BQ45 | 7/16-24 UNS | 5.16mm | 45° | 1/4” | 3/8-24 UNF | 11.2mm | 3/16" | 9/16” |
| BQ9301 | 7/16-24 UNS | 6.19mm | 45° | 1/4” | 1/2-20 UNF | 12mm | 3/16" | 5/8” |
| BQ122 | 7/16-24 UNS | 5.16mm | 45° | 1/4” | 1/2-20 UNF | 12mm | 5/16" | 5/8” |
| BQ123 | 7/16-24 UNS | 5.16mm | 45° | 1/4” | 9/16-18 UNF | 12mm | 3/16" | 11/16” |
| BQ9304 | 1/2-20 UNF | 6.67mm | 45° | 3/16” | 3/8-24 UNF | 12mm | 3/16" | 11/16” |
| BQ260 | 1/2-20 UNF | 5.94mm | 45° | 5/16” | 3/8-24 UNF | 11.2mm | 3/16" | 16mm |
| BQ127 | 1/2-20 UNF | 5.94mm | 45° | 5/16” | 7/16-24 UNS | 12mm | 1/4" | 5/8” |
| BQ261 | 9/16-18 UNF | 5.94mm | 45° | 5/16” | 3/8-24 UNF | 11.2mm | 3/16" | 11/16” |
| BQ50 | M10 × 1 | 7.95mm | 45° | 3/16” | 3/8-24 UNF | 11.2mm | 3/16" | 1/2” |
| BQ40 | M10 × 1 | 7mm | DIN | 3/16” | M10 × 1 | 13mm | 3/16" | 16mm |
| BQ9305 | M10 × 1 | 7.3mm | 45° | 3/16” | M10 × 1 | 13mm | 3/16" | 15mm |
| BQ42 | M10 × 1 | 7.95mm | 45° | 3/16” | M10 × 1.25 | 12mm | 3/16" | 13mm |
| BQ43 | M10 × 1 | 7.95mm | 45° | 3/16” | M12 × 1 | 12mm | 3/16" | 16mm |
| BQ41 | M10 × 1.25 | 7.95mm | 45° | 3/16” | M10 × 1 | 12.6mm | 3/16" | 13mm |
| BQ246 | M12 × 1 | 9mm | DIN | 1/4” | 1/8-27 NPT | 9.97mm | 3/16" | 16mm |
| BQ44 | M12 × 1 | 7.95mm | 45° | 1/4” | M10 × 1 | 12.6mm | 3/16" | 16mm |
| BQ379 | M12 × 1.5 | 7.95mm | 45° | 3/16” | 1/8-27 NPT | 9.97mm | 3/16" | 14mm |
| BQ380 | M14 × 1.5 | 7.95mm | 45° | 3/16” | 1/8-27 NPT | 9.97mm | 3/16" | 17mm |

Port Thread Length

Inverted seat for 45° double flare

Stud Length

Drill-point seat for Bubble or DIN flare

When deciding if an adapter will work with a specific port, think of the Stud Fit as a "tube-size equivalent," in that the adapter will seat properly if the corresponding tube and tube nut would also seat properly. Most BrakeQuip adapters correspond to 3/16" tube but there are a few exceptions, as noted in the table.

See page 13 for assortment kit BQ7010.

Male-Male Adapters

| Part Number | JIC Stud Thread | Other Stud Thread | Other Stud Length | Hex | Overall Length |
| --- | --- | --- | --- | --- | --- |
| BQ135 | 3/8-24 UNF (-3 JIC) | 3/8-24 UNF | 16mm | 1/2” | 34.2mm |
| BQ132 | 3/8-24 UNF (-3 JIC) | 1/4-28 UNF | 16mm | 1/2” | 36mm |
| BQ168 | 3/8-24 UNF (-3 JIC) | 1/2-20 UNF | 13mm | 1/2” | 33mm |
| BQ129 | 3/8-24 UNF (-3 JIC) | M10 × 1 | 16mm | 1/2” | 34.2mm |
| BQ128 | 3/8-24 UNF (-3 JIC) | M10 × 1.25 | 16mm | 1/2” | 36mm |
| BQ130 | 3/8-24 UNF (-3 JIC) | M10 × 1.5 | 16mm | 1/2” | 60mm |
| BQ131 | 7/16-20 UNF (-4 JIC) | 3/8-24 UNF | 16mm | 1/2” | 37.1mm |
| BQ134 | 7/16-20 UNF (-4 JIC) | 7/16-24 UNS | 16mm | 9/16” | 36mm |
| BQ176 | 7/16-20 UNF (-4 JIC) | M10 × 1 | 11.6mm | 1/2” | 31.6mm |
| BQ144 | 7/16-20 UNF (-4 JIC) | M12 × 1 | 16mm | 14mm | 36mm |
| BQ149 | 7/16-20 UNF (-4 JIC) | M12 × 1.25 | 16mm | 14mm | 37mm |

BQ130 has an elongated JIC stud.

JIC Stud

BQ145 -4 JIC to M10 × 1

Non-JIC stud for use with ISO 24° cone connector: either compression or o-ring seal cone (DKO).

Male

12mm hex, 29mm overall length

Other Stud

BQ157 -3 JIC to 1/8-28

5/8" hex, 25.9mm overall length

For use with bonded washer (dowty seal)

See page 13 for assortment kit BQ7003.

Bump Tube Adapters

| Part Number | JIC Stud Thread | O-ring Pilot Thread | Hex | Overall Length |
| --- | --- | --- | --- | --- |
| 5MBT-5MJ | 1/2-20 UNF (-5 JIC) | 1/2-20 UNF | 9/16” | 37.4mm |
| 7MBT-6MJ | 9/16-18 UNF (-6 JIC) | 11/16-18 UNS | 3/4” | 41.9mm |
| 14MMBT-6MJ | 9/16-18 UNF (-6 JIC) | M14 × 1.5 | 5/8” | 38.8mm |
| 16MMBT-6MJ | 9/16-18 UNF (-6 JIC) | M16 × 1.5 | 17mm | 38.9mm |
| 18MMBT-6MJ | 9/16-18 UNF (-6 JIC) | M18 × 1.5 | 3/4” | 39.4mm |

| Part Number | Port O-ring Pilot Thread Thread | Hex Overall Length |
| --- | --- | --- |
| 16MMBT-6FI | 5/8-18 UNF M16 × 1.5 | 3/4” 30.8mm |

Transmission Line Adapters

##### Convert push-on connection transmission lines to double-flare connections to make and repair these lines in your shop.

Female

BQ252 -3 JIC Elbow

JIC Elbow

BQ182 Straight

5/8-18 UNF

inverted seat

BQ183 90°

5/8-18 UNF

inverted seat

3/8-24 UNF port, inverted seat 3/8-24 UNF stud (-3 JIC)

for Nissan

See page 13 for assortment kit BQ7000.

Copper Washers

| Part Number | Bolt Size | Diameter | Thickness |
| --- | --- | --- | --- |
| BQ3858 | 3/8” | 5/8” | 1.5mm |
| BQ1118 | 7/16” | 18mm | 1.5mm |
| BQ1320 | 1/2” | 20mm | 2mm |
| BQ1421 | 9/16” | 21mm | 2mm |
| BQ813 | 8mm | 13mm | 1.3mm |
| BQ1016 | 10mm | 16mm | 1.5mm |
| BQ1218 | 12mm | 18mm | 1.5mm |
| BQ1522 | 15mm | 22mm | 2mm |
| BQ1624 | 16mm | 24mm | 2mm |

BQ244

### Copper Seat Seal (Flare Gasket)

See page 13 for assortment kit BQ7007.

Banjo Bolts

Standard

| Part Number | Thread | Stud Length | Hex | Washer |
| --- | --- | --- | --- | --- |
| BQ8093 | 3/8-24 UNF | 25mm | 11mm | BQ3858 |
| BQ8089 | 7/16-20 UNF | 23.8mm | 1/2” | BQ1118 |
| BQ8096 | 7/16-20 UNF | 27mm | 11mm | BQ1118 |
| BQ8090 | 7/16-24 UNS | 26.5mm | 1/2” | BQ1118 |
| BQ8097 | 1/2-20 UNF | 31mm | 9/16” | BQ1320 |
| BQ8076 | M8 × 1 | 17mm | 10mm | BQ813 |
| BQ8077 | M8 × 1.25 | 16mm | 10mm | BQ813 |
| BQ8095 | M10 × 1 | 22mm | 11mm | BQ1016 |
| BQ8094 | M10 × 1 | 26mm | 11mm | BQ1016 |
| BQ8099 | M10 × 1.25 | 22mm | 11mm | BQ1016 |
| BQ8075 | M10 × 1.5 | 20mm | 11mm | BQ1016 |
| BQ8086 | M10 × 1.5 | 23mm | 11mm | BQ1016 |
| BQ8086SS | M10 × 1.5 | 23mm | 11mm | BQ1016 |
| BQ8088 | M10 × 1.5 | 27mm | 11mm | BQ1016 |
| BQ8087 | M11 × 1.25 | 23mm | 11mm | BQ1118 |
| BQ8098 | M12 × 1 | 21.3mm | 13mm | BQ1218 |
| BQ8078 | M12 × 1.25 | 25mm | 13mm | BQ1218 |
| BQ8092 | M12 × 1.5 | 25mm | 13mm | BQ1218 |

Stud Length

Profile

Double

| Part Number | Thread | Stud Length | Hex | Washer |
| --- | --- | --- | --- | --- |
| BQ8085 | 3/8-24 UNF | 34.5mm | 7/16” | BQ3858 |
| BQ8083 BQ8084 | M10 × 1 M10 × 1.25 | 34.5mm 34.5mm | 11mm 11mm | BQ1016 BQ1016 |

BQ8085 has a plain hex profile rather than round.

See page 13 for assortment kit BQ7001.

Banjo Adapters

| Part Number | Thread | Hole | Thickness | Width |
| --- | --- | --- | --- | --- |
| BQ60 BQ62 BQ63 | 3/8-24 UNF 7/16-24 UNS 1/2-20 UNF | 10.2mm 11.2mm 12.9mm | 1/2” 14mm 16mm | 17mm 18mm 20mm |
| BQ61 | M10 × 1 | 10.2mm | 13mm | 17mm |

Square

Width

Hole

Thickness

Inverted Seat

Round

BQ151 and BQ271

| Part Number | Thread | JIC | Hole | Thickness | Width | Overall Length |
| --- | --- | --- | --- | --- | --- | --- |
| BQ150 | 3/8-24 UNF | -3 | 10mm | 10mm | 17mm | 42mm |
| BQ151 | 3/8-24 UNF | -3 | 10mm | 10mm | 17mm | 19.6mm |
| BQ152 | 3/8-24 UNF | -3 | 10mm | 10mm | 17mm | 39.2mm |
| BQ155 | 7/16-20 UNF | -4 | 10mm | 10mm | 18mm | 42mm |
| BQ271 | 1/2-20 UNF | -5 | 14mm | 14mm | 24mm | 26mm |
| BQ392 | 1/2-20 UNF | -5 | 14mm | 14mm | 24mm | 44mm |
| BQ393 | 9/16-18 UNF | -6 | 16mm | 16mm | 28mm | 44mm |

are short.

Width

Thickness

Hole

Overall Length

BQ152

BQ150, BQ155, BQ392 and

BQ393 all have a long tube.

JIC Union

See page 5 for brazing/welding rods.

Weld-On Banjo Heads

| Part Number | Hole | Thickness | Width | Tube Size | Overall Length |
| --- | --- | --- | --- | --- | --- |
| BQ225 | 7/16” | 10mm | 19mm | 3/16” | 27.5mm |
| BQ223 | 10.2mm | 8mm | 17mm | 3/16” | 25.5mm |
| BQ224 | 10.2mm | 10mm | 18mm | 3/16” | 26mm |
| BQ178 | 12.2mm | 12mm | 22mm | 3/8” | 27mm |
| BQ179 | 14.2mm | 14mm | 24mm | 3/8” | 28mm |

Hole

Width	Thickness	Tube Size

Overall Length

See page 13 for assortment kit BQ7014.

Bleed Screws

| Part Number | Thread | Stud Length | Hex Size | Cap Size |
| --- | --- | --- | --- | --- |
| BQ4439 | 1/4-28 UNF | 15.3 mm | 7 mm | Small |
| BQ4438 | 5/16-24 UNF | 17.5 mm | 5/16” | Large |
| BQ4433 | 3/8-24 UNF | 20.8 mm | 3/8” | Large |
| BQ4445 | 7/16-20 UNF | 20.5 mm | 7/16” | Large |
| BQ4444 | 7/16-24 UNS | 24.5 mm | 7/16” | Large |
| BQ4436 | M7 × 1 | 12.5 mm | 8 mm | Small |
| BQ4441 | M7 × 1 | 21.5 mm | 7 mm | Small |
| BQ4443 | M8 × 1 | 17.5 mm | 8 mm | Large |
| BQ4442 | M8 × 1.25 | 22.1 mm | 8 mm | Large |
| BQ4435 | M10 × 1 | 19.5 mm | 10 mm | Large |
| BQ4434 | M10 × 1 | 24.1 mm | 10 mm | Large |
| BQ4446 | M10 × 1.25 | 20.4 mm | 10 mm | Large |
| BQ4440 | M10 × 1.5 | 17.9 mm | 10 mm | Large |

Stud Length

Bleeder caps

BQ315  Small

BQ316  Large

BQ181 Bleed Screw Repair Kit

See page 13 for assortment kit BQ7012.

Fitting Caps and Plugs

Rescue your brake component that has a stripped or broken bleed screw. Simply drill and tap to 1/8-27 NPT.

Caps

| Part Number | Thread | Hex Size |
| --- | --- | --- |
| BQ160 | 3/8-24 UNF | 1/2” |
| BQ161 | 7/16-24 UNS | 9/16” |
| BQ162 | 7/16-20 UNF | 9/16” |
| BQ163 | 1/2-20 UNF | 5/8” |
| BQ164 | M10 × 1 | 13 mm |
| BQ165 | M10 × 1.25 | 13 mm |
| BQ166 | M12 × 1 | 16 mm |
| BQ167 | M12 × 1.5 | 16 mm |

| Part Number | Thread | Hex Size |
| --- | --- | --- |
| BQ19 BQ26 | 3/8-24 UNF 7/16-24 UNS | 3/16” 1/4” |
| BQ18 | M10 × 1 | 5 mm |

45° inverted seat 8mm thread length

See page 17 for bleed screw removal solutions.

Plugs

BQ1020 Banjo Blocking-Off Tool

See page 17 for line-stopper kits.

Hex	Length

Assortment Kits

BQ7006 Imperial Tube Nuts

PN	Description	Qty

BQ9176	3/8-24 UNF  female	4

BQ57	7/16-20 UNF  female	4

BQ9177	1/2-20 UNF  female	4

BQ4276	3/8-20 BSF  male	4

BQ4285	3/8-24 UNF  male	4

BQ5555	3/8-24 UNF male	10

BQ4277	7/16-20 UNF  male	4

BQ6118	7/16-20 UNF  male	6

BQ2577	7/16-24 UNS  male	10

BQ5842	7/16-24 UNS  male	10

| BQ9184 | 1/2-20 UNF male | 4 | BQ125 | M10 × 1 — 3/8-24 UNF drill-point | 4 |
| --- | --- | --- | --- | --- | --- |
| BQ9187 | 1/2-20 UNF male | 4 | BQ49 | M10 × 1 — 3/8-24 UNF inverted | 4 |
| BQ9185 | 9/16-18 UNF male | 4 | BQ40 | M10 × 1 — M10 × 1 drill-point | 2 |
| BQ9186 | 9/16-18 UNF male | 4 | BQ41 | M10 × 1 — M10 × 1.25 inverted | 4 |
|  |  |  | BQ44 | M10 × 1 — M12 × 1 inverted | 4 |

BQ2578	1/2-20 UNF male	6

BQ7004 Male-Female Adapters

PN	Description	Qty

BQ48	1/8-27 NPT — 3/8-24 UNF  inverted	4

BQ156	3/8-24 UNF — 3/8-24 UNF  inverted	4

BQ45	3/8-24 UNF — 7/16-24 UNS  inverted	4

BQ50	3/8-24 UNF — M10 × 1  inverted	4

BQ47	7/16-20 UNF — 3/8-24 UNF  inverted	4

BQ46	7/16-24 UNS — 3/8-24 UNF  inverted	4

BQ120	1/2-20 UNF — 3/8-24 UNF  inverted	4

BQ122	1/2-20 UNF — 7/16-24 UNS  inverted	4

BQ124	9/16-18 UNF — 3/8-24 UNF  inverted	2

BQ123	9/16-18 UNF — 7/16-24 UNS  inverted	2

BQ121	9/16-20 UNS — 3/8-24 UNF  inverted	2

BQ7000 Copper Washers

PN	Description	Qty

BQ3858	3/8” ID	20

BQ1118	7/16” ID	20

BQ1320	1/2” ID	20

BQ1421	9/16” ID	10

BQ1016	10mm ID	20

BQ1218	12mm ID	20

BQ1522	15mm ID	10

BQ1624	16mm ID	10

| PN | Description | Qty |
| --- | --- | --- |
| BQ8093 | 3/8-24 UNF | 6 |
| BQ8096 | 7/16-20 UNF | 4 |
| BQ8089 | 7/16-20 UNF | 4 |
| BQ8090 | 7/16-24 UNS | 4 |

BQ7007 Banjo Bolts

BQ7005 Metric Tube Nuts

PN	Description	Qty

BQ9175	M10 × 1  female	4

BQ9174	M12 × 1  female	4

BQ9179	M12 × 1  female	4

BQ9178	M12 × 1.5  female	4

BQ4511	M10 × 1  male	10

BQ72	M10 × 1 DIN  male	4

BQ4512	M10 × 1.25  male	6

BQ9188	M10 × 1.5  male	4

BQ9191	M11 × 1.5  male	4

BQ4513	M12 × 1  male	6

| BQ9180 | M12 × 1 male | 4 |
| --- | --- | --- |
| BQ9192 | M12 × 1.25 male | 4 |
| BQ9193 | M12 × 1.5 male | 4 |
| BQ9182 | M12 × 1.5 male | 4 |
| BQ9183 | M14 × 1.5 male | 4 |

BQ7009 Female Unions

PN	Description	Qty

BQ115	3/8-24 UNF  drill-point	4

BQ110	3/8-24 UNF  inverted	4

| BQ114 | M10 × 1 drill-point | 4 | PN | Description | Qty |
| --- | --- | --- | --- | --- | --- |
| BQ117 | M10 × 1 inverted | 4 | BQ160 | 3/8-24 UNF cap | 2 |
| BQ116 | M12 × 1 inverted | 4 | BQ19 | 3/8-24 UNF plug | 2 |
|  |  |  | BQ161 | 7/16-24 UNS cap | 2 |

BQ111	7/16-24 UNS  inverted	4

BQ42	M10 × 1.25 — M10 × 1  inverted	4

BQ43	M12 × 1 — M10 × 1  inverted	4

BQ7010 Male-Male Adapters

PN	Description	Qty

BQ135	3/8-24 UNF — 3/8-24 UNF	4

BQ131	3/8-24 UNF — 7/16-20 UNF	4

BQ134	7/16-24 UNS — 7/16-20 UNF	4

BQ129	M10 × 1 — 3/8-24 UNF	4

BQ130	M10 × 1.5 — 3/8-24 UNF	4

BQ144	M12 × 1 — 3/8-24 UNF	4

BQ7003 Bump Tube Adapters

PN	Description	Qty

5MBT-5MJ	1/2-20 UNF — 1/2-20 UNF	4

7MBT-6MJ	11/16-18 UNS — 9/16-18 UNF	4

| 14MMBT-6MJ | M14 × 1.5 — 9/16-18 UNF | 4 |
| --- | --- | --- |
| 16MMBT-6FI | M16 × 1.5 — 5/8-18 UNF | 4 |
| 16MMBT-6MJ | M16 × 1.5 — 9/16-18 UNF | 4 |
| 18MMBT-6MJ | M18 × 1.5 — 9/16-18 UNF | 4 |

BQ7012 Brake Hose Plugs and Caps

BQ8097	1/2-20 UNF	4

BQ8095	M10 × 1	6

BQ8099	M10 × 1.25	4

BQ8086	M10 × 1.5	4

BQ8088	M10 × 1.5	4

BQ8087	M11 × 1.25	4

BQ8098	M12 × 1	4

BQ8092	M12 × 1.5	4

BQ7001 Banjo Unions & Adapters

| PN | Description | Qty |
| --- | --- | --- |
| BQ60 | 3/8-24 UNF, 10mm hole | 4 |
| BQ150 | 3/8-24 UNF, 10mm hole | 4 |
| BQ151 | 3/8-24 UNF, 10mm hole | 4 |
| BQ62 | 7/16-24 UNS, 7/16” hole | 4 |

BQ63	1/2-24 UNF, 1/2” hole	4

BQ61	M10 × 1, 10mm hole	4

BQ7014 Bleed Screws

| PN | Description | Qty |
| --- | --- | --- |
| BQ4439 | 1/4-28 UNF | 4 |
| BQ4438 | 5/16-24 UNF | 4 |
| BQ4433 | 3/8-24 UNF | 6 |
| BQ4444 | 7/16-24 UNS | 4 |

BQ4445	7/16-20 UNF	4

BQ7002 3-way Tube Tees

PN	Description	Qty

BQ26	7/16-24 UNS  plug	2

| BQ4441 | M7 × 1 | 4 |
| --- | --- | --- |
| BQ4443 | M8 × 1 | 4 |
| BQ4442 | M8 × 1.25 | 4 |
| BQ4435 | M10 × 1 short | 4 |
| BQ4434 | M10 × 1 long | 4 |
| BQ4446 | M10 × 1.25 | 4 |

BQ163	1/2-20 UNF  cap	2

BQ18	M10 × 1 plug	2

BQ93	1/8-27 NPT	2

BQ90A	3/8-24 UNF  drill-point	2

BQ56A	3/8-24 UNF  inverted	2

BQ92A	7/16-24 UNS  inverted	2

BQ91A	M10 × 1  inverted	2

BQ95A	M10 × 1 DIN	2

BQ164	M10 × 1  cap	2

BQ165	M10 × 1.25  cap	2

BQ166	M12 × 1  cap	2

BQ167	M12 × 1.5  cap	2

BQ1020        Banjo Blocking-Off Tool	1

BQ4440	M10 × 1.5	4

| BQ315 | Small Cap | 10 |
| --- | --- | --- |
| BQ316 | Large Cap | 10 |

Tube Workers — Straighteners

BQ1028 Professional Tube Straightener

The BQ1028 provides the average shop the ability to complete a professional tube installation by forming coiled tube into straight sections of any length, offering precise tube routing and reducing the number of unions required. Adjust the wheels to the tube size and pull the tube through the tool to straighten.

  - for 3/16” to 1/2” light-walled, annealed tube, including stainless steel

  - Non-marking ABS wheels

  - 5-point precision and fine-threaded adjustment

  - Vice mount or bolt straight to workbench

  - Smooth ball bearing action

  - Heavy-duty construction

BQ1028-75

##### Replacement wheel

BQ1030 Brake Tube Straightener

##### BQ1030-MB Mounting Bracket for Brake Tube Straightener

The BQ1030 was designed for the automotive DIY-er with a need to straighten coils of the most brake line sizes.

  - Straighten 3/16”–1/4” annealed tubing, including cunifer and stainless steel

  - U-shaped rollers straighten both tubing sizes — no need to change rollers

  - Hard polymer rollers leave a smooth finish on tubing

  - Smooth ball bearing action

  - Bench-mount ready

  - Optional vise-mount base (BQ1030-MB)

BQ405   Tube-Straightening Pliers

Remove imperfections or fix mistakes with the BQ405 pliers.

    - Straightens incorrect bends in 3/16” and 1/4” tube, including stainless steel

    - Makes tight-radius bends, up to 90°, for both 3/16” and 1/4” tube

    - The flat side of the mandrel provides a guide surface for cutting

Tube Workers — Benders

BQ402 Brake Tube Pliers

For use on 3/16” and 1/4” brake tube.

These pliers are perfect for working in constrained areas under a vehicle to

- align the tube and nut with the port during installation

- put a tight 90° bend behind a tube nut (such as a master cylinder drop)

One side of the tool end is flat to facilitate cutting.

BQ406 Professional Tube Bender

For use on 3/16” and 1/4” brake tube, this bender is easy to use and yields professional results. Bend on the vehicle or mount the tool in a vise for precision work or mass production.

- Low-friction rollers make for easy precision work

- Smooth roller channels provide a clean, scratch free finish

- Tight radius bends up to 180°

- Handles separate for quick and easy tube removal

##### Metal Tube Cutters

These tube cutters have a heavy-duty wheel for cutting stainless steel or other light-walled tubing, 3/16” to 5/8”.

BQ601 Mini Tube Cutter

Great for cutting in tight places.

##### BQ602 Professional Tube Cutter

Replacement blade is BQ602C.

Cutters

BQ2103 Hose Cutter

This heavy duty tool cuts hose clean and square with ease. The blade has two cutting points for double life.

#### BQ2101B

##### Replacement blade

Flaring Tools

See pages 2 and 3 for stainless steel and cunifer tubing.

BrakeQuip High-Speed Flaring Tools are the industry standard for accurate and consistent flares.

  - Smooth channel dies minimize tube scarring and deformation for a show quality finish.

  - The cam-and-lever design, combined with ball-detent turrets, provide fast and easy single, double, bubble, and DIN flares.

  - BrakeQuip flaring tools are designed to be used in a bench vise or with our table flange mount (BQ350-1).

  - Replacement components are available, all the way down to the individual punches on the turrets.

37° or 45°

Single

37° or 45°

Double

Bubble

DIN

#### Tool Body and Handle

Dies and Turrets

Part Number	BQ350A	BQ350	BQ351	BQ352

45° flares		3/16”, 1/4”, 5/16”, 3/8” 4.75mm, 6mm, 8mm, 10mm

3/16”, 1/4”, 5/16”, 3/8”, 1/2”

  - mm, 6mm, 8mm, 10mm

3/16”, 1/4”, 5/16”, 3/8”, 1/2”

none

37° flares	3/16”, 1/4”, 5/16”, 3/8”, 1/2”	3/16”, 1/4”, 5/16”, 3/8”, 1/2”

none

3/16”, 1/4”, 5/16”, 3/8”, 1/2”

DIN flares	3/16”, 1/4”	3/16”, 1/4”	3/16”	none

Already have the BQ351 and just need 37° dies and turrets? Order BQ352-BQ to get the dies and turrets pictured with the BQ352.

The BQ350A features . . .

a tube-setting tab to ensure proper placement in the dies

a die-cast steel body for added strength and an extra-long handle for more leverage

a die-clamp bolt for quick die swaps

BQ350-1 Mounting Flange

Works will all BrakeQuip flaring tools. Included with the BQ350A.

Other Tools

BQ2100 Thread Identification Kit

A kit of the highest quality, the BQ2100 has gauges for the vast majority of brake plumbing threads, both male and female. Gauges are color-coded for fast selection. The threads are hardened for long life, and each gauge is available separately as a replacement part.

The gauges can also be used to block-off fluid ports during service.

Imperial Sizes

3/8-20 BSF

3/8-24 UNF

7/16-20 UNF

7/16-24 UNS

1/2-20 UNF

9/16-18 UNF

9/16-20 UNS

Metric Sizes

M9 × 1

M10 × 1

M10 × 1.25

M10 × 1.5

M11 × 1.25

M12 × 1

M12 × 1.25

M12 × 1.5

M14 × 1.5

BQ2200 Bleeder Buster Kit

Remove and replace frozen bleeder screws like a pro. Here's how:

    - Insert filler wire into the bleed screw and cut flush with the end

    - Install the Bleed Screw Removal Tool into an air chisel and shock the bleed screw from the top

    - Unscrew the bleed screw with a conventional tool

The kit includes the bleed screw removal tool and an assortment of filler wire, bleed screws, and caps for all applications. The tool, screws, caps, and filler wire are all available individually as replacement parts.

#### BQ2200-1

##### Bleed Screw Removal Tool

BQ2200W

Filler Wire Replenishment Kit

See page 12 for a complete selection of bleed screws and caps.

BQ407 and BQ408 Line Stopper Kits

Prevent fluid leaks and contamination during service. Simply slide the appropriate stopper around the tube nut or banjo head and release. The spring-loaded plunger will seal around the fitting and prevent leaks.

Note: the 5/16” stopper in the BQ407 kit is also suitable for 3/16” and 1/4” lines.

BQ407 4-Piece Stopper Kit

| Stopper Size | BQ407 Qty | BQ408 Qty |  |
| --- | --- | --- | --- |
| 3/16”–1/4” | — | 2 |  |
| 5/16” | 1 | 2 |  |
| 3/8” | 1 | 1 |  |
| 1/2” | 1 | 1 |  |
| Banjo | 1 | 2 |  |
|  |  |  | BQ408 8-Piece Stopper Kit Comes in slide-lock clamshell case. |

Fuel	BrakeQuip

Fuel Fittings

Quick Connects

Thumb-release convenience. No special tools required. Works with both domestic and foreign vehicles.

| Straight Part Number | Female | Male |  | 90° Bend Part Number | Female | Male |
| --- | --- | --- | --- | --- | --- | --- |
| FL08-4-5 | 1/4” | 5/16” |  | FL09-5-5 | 5/16” | 5/16” |
| FL08-5-5 | 5/16” | 5/16” |  | FL09-5-6 | 5/16” | 3/8” |
| FL08-5-6 | 5/16” | 3/8” |  | FL09-6-5 | 3/8” | 5/16” |
| FL08-6-5 | 3/8” | 5/16” |  | FL09-6-6 | 3/8” | 3/8” |
| FL08-6-6 | 3/8” | 3/8” |  |  |  |  |

##### Quick Connector Tubes

Part Number	Description

FL05-4	1/4” quick connect to 8mm FuelFlex - straight

FL03-5	5/16” quick connect to 8mm FuelFlex - straight

FL03-6	3/8” quick connect to 10mm FuelFlex - straight

FL04-6	3/8” quick connect to 3/8” quick connect - straight

Part Number	Description

FL20-5	8mm FuelFlex to 8mm FuelFlex - 90°

FL20-6	10mm FuelFlex to 10mm FuelFlex - 90°

FuelFlex Connectors

FuelFlex Reducers

Part Number	Description

FL06-5-4	8mm FuelFlex to 1/4” FuelFlex

FL06-6-5	10mm FuelFlex to 8mm FuelFlex

FuelFlex to Rubber Hose

Part Number	Description                                   FL02-5		8mm FuelFlex to 5/16” rubber hose

FL02-6	10mm FuelFlex to 3/8” rubber hose

| Threaded Connectors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Straight Part Number | Tube End | End Form | Thread | Length |  |
|  | FL07-5 | 5/16” rubber hose | bump | M14 × 1.5 | 230mm |  |
|  | FL07-6 | 3/8” rubber hose | bump | M16 × 1.5 | 230mm |  |
|  | FL12-14mm | 8mm FuelFlex | double flare | M14 × 1.5 | 90mm |  |
|  | FL13-5 | 8mm FuelFlex | bump | M14 × 1.5 | 155mm |  |
|  | FL14-5 | 8mm FuelFlex | bump | M14 × 1.5 | 100mm |  |
|  | FL13-6 | 10mm FuelFlex | bump | M16 × 1.5 | 155mm |  |
|  | FL14-6 | 10mm FuelFlex | bump | M16 × 1.5 | 100mm |  |
|  | Bent Part Number | Tube End | End Form | Thread | Angle |  |
|  | FL11-14mm | 8mm FuelFlex | double flare | M14 × 1.5 | 30° |  |
|  | FL15 | 10mm FuelFlex | bump | M16 × 1.5 | 180° |  |

FuelFlex Tubing

Part Number	Size

T8-FF	8mm (5/16”)

T10-FF	10mm (3/8”)

Coil Length

25ft 25ft

Spring Mandrels

Part Number	Size

BQ318

BQ319

8mm (5/16”)

10mm (3/8”)

Note: In keeping with industry conventions for plastic tubing sizes, FuelFlex tubing is measured by outer diameter.

Permanently form fuel tubing without kinking.

FuelFlex Unions

Part Number

FL01-5

Male-Female Adapters

Size

8mm

Part Number

FL10-5S

Male

8mm FuelFlex

Female

M14 × 1.5

Seat

bump

FL01-6	10mm

Fuel Fittings

Innovative Solutions for Automotive Tubes and Hoses	Fuel

| FL10-6S | 10mm FuelFlex | M16 × 1.5 | bump |
| --- | --- | --- | --- |
| FL10-14mm | 8mm FuelFlex | M14 × 1.5 | inverted |

##### Compression Fittings

Part Number	Description

##### Bump Tube Nuts

Part Number	Size

| FL37 | 8mm FuelFlex — 8mm FuelFlex |  | BQ9148 | M16 × 1.5 – 3/8” |
| --- | --- | --- | --- | --- |
| FL36 | 8mm FuelFlex — 5/16” metal tube |  | BQ9149 | M14 × 1.5 – 5/16” |
| FL39 | 10mm FuelFlex — 10mm FuelFlex |  |  |  |
| FL38 | 10mm FuelFlex — 3/8” metal tube |  |  | 360° Tube Clamps |

Tube Support Clamps

FL34   10mm (3/8”)

FL35   8mm (5/16”)

FL26   10mm (3/8”)

FL25   8mm (5/16”)

Fuel Pressure Repair Kit (Check Valve)

FL21   8mm (5/16”)

FL22   10mm (3/8”)

BQ412 Tube Clamp Pliers

| Part Number | Qty | Part Number | Qty | Part Number | Qty | BQFL4500 Fuel Line Kit |
| --- | --- | --- | --- | --- | --- | --- |

| Tubing |  | FuelFlex Reducers |  | Compression Fittings |  |
| --- | --- | --- | --- | --- | --- |
| T8-FF | 25ft | FL06-6-5 | 2 | FL36 | 2 |
| T10-FF | 25ft | FuelFlex Adapters |  | FL37 | 2 |
| Quick Connects |  | FL02-5 | 2 | FL38 | 2 |
| FL08-4-5 | 2 | FL02-6 | 2 | FL39 | 2 |
| FL08-5-5 | 2 | Theaded Connectors |  | Bump Tube Nuts |  |
| FL08-5-6 | 2 | FL07-5 | 2 | BQ9148 | 4 |
| FL08-6-5 | 2 | FL07-6 | 2 | BQ9149 | 4 |
| FL08-6-6 | 2 | FL11-14mm | 2 | Screws and Clamps |  |
| FL09-5-5 | 2 | FL12-14mm | 2 | BQ811 | 20 |
| FL09-5-6 | 2 | FL13-5 | 2 | FL25 | 20 |
| FL09-6-5 | 2 | FL13-6 | 2 | FL26 | 20 |
| FL09-6-6 | 2 | FL14-5 | 2 | FL34 | 20 |
| Quick Connector | Tubes | FL14-6 | 2 | FL35 | 20 |
| FL03-5 | 2 | FL15 | 1 | Tools |  |
| FL03-6 | 2 | FuelFlex Unions |  | BQ318 | 1 |
| FL04-6 | 2 | FL01-5 | 2 | BQ319 | 1 |
| FL05-4 | 2 | FL01-6 | 2 | BQ-01 cutter | 1 |
| FuelFlex Connectors Male-Female Adapters BQ412 1 |  |  |  |  |  |
| FL20-5 | 2 | FL10-5 | 1 | Tube Protection |  |

FL20-6	2

FL10-6S	1

FL10-14mm	1

BQ313-SV	5ft

The BrakeQuip Professional Fuel Line Kit comes with dozens of components and fittings, from quick connects and adapters to pliers, clamps, and heat shields. With over 180 pieces and 50 feet of FuelFlex Tubing, this kit is a complete fuel line solution for almost any vehicle.

JIC Fittings

37°

JIC (Joint Industry Council) fittings are defined by SAE J514 and use a 37° sealing surface. They are commonly found on aftermarket adapters, bias valves, calipers, and gauges. JIC fittings should not be mixed with 45° flares, or vice-versa.

##### Note on JIC vs AN fittings: JIC fittings have the same dimensions as their AN (Army-Navy) counterparts. The difference is that

AN fittings are specified to extreme tolerances for aerospace applications.

Part Number	JIC	Description	Page

Banjo Adapters	11

Part Number	JIC	Description	Page

Male-Male Adapters (continued)	9

| BQ150 | -3 | Round banjo head, long |  | BQ130 | -3 | JIC to M10 × 1.5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BQ151 | -3 | Round banjo head |  | BQ131 | -4 | JIC to 3/8-24 UNF |  |
| BQ152 | -3 | Round banjo union (2 studs) |  | BQ134 | -4 | JIC to 7/16-24 UNS |  |
| BQ155 | -4 | Round banjo head, long |  | BQ145 | -4 | JIC to M10 × 1 |  |
| BQ271 | -5 | Round banjo head |  | BQ176 | -4 | JIC to M10 × 1, long |  |
| BQ392 | -5 | Round banjo head, long |  | BQ144 | -4 | JIC to M12 × 1 |  |
| BQ393 | -6 | Round banjo head, long |  | BQ149 | -4 | JIC to M12 × 1.25 |  |
| Bump Tube Adapters 5MBT-5MJ | -5 | JIC to 1/2-20 UNF | 9 | Male Tees BQ360 | -3 | Bolt mount male tube tee | 7 |
| 7MBT-6MJ | -6 | JIC to 11/16-18 UNS |  | BQ361 | -4 | Bolt mount male tube tee |  |
| 14MMBT-6MJ | -6 | JIC to M14 × 1.5 |  | Male Unions |  |  | 6 |
| 16MMBT-6MJ | -6 | JIC to M16 × 1.5 |  | BQ292 | -3 | Steel JIC union |  |
| 18MMBT-6MJ | -6 | JIC to M18 × 1.5 |  | BQ292SS | -3 | Stainless steel JIC union |  |
| Elbow BQ252 | -3 | JIC to 3/8-24 UNF, elbow | 9 | BQ293SS Weld-On Studs | -3 | Stainless steel JIC union, long | 5 |
| Male-Male Adapters |  |  | 9 | BQ153 | -3 | Steel stud for 3/16” tube |  |
| BQ135 | -3 | JIC to 3/8-24 UNF |  | BQ170 | -6 | Aluminum stud for 3/8” tube |  |
| BQ132 | -3 | JIC to 1/4-28 UNF |  | BQ172 | -8 | Aluminum stud for 1/2” tube |  |
| BQ168 | -3 | JIC to 1/2-20 UNF |  | BQ173 | -10 | Aluminum stud for 5/8” tube |  |
| BQ157 | -3 | JIC to 1/8-28 BSPP |  | BQ226 | -6 | Steel stud for 3/8” tube |  |
| BQ129 | -3 | JIC to M10 × 1 |  | BQ227 | -6 | Steel stud for 10mm tube |  |
| BQ128 | -3 | JIC to M10 × 1.25 |  | BQ228 | -6 | Steel stud for 1/2” tube |  |

Stainless Steel

| Part Number | Description | Page |  | Part Number | Description | Page |
| --- | --- | --- | --- | --- | --- | --- |
| Banjo Bolts BQ8086SS | M10 × 1.5 | 10 |  | Male Tube Nuts BQ5555SS | 3/8-24 UNF | 4 |
| Clips BQ3052SS | Universal Clip | 6 |  | BQ5842SS BQ2577SS | 7/16-24 UNS 7/16-24 UNS |  |
| Female Tees BQ56ASS | 3/8-24 UNF, bolt mount | 7 |  | BQ4511SS BQ4513SS | M10 × 1 M12 × 1 |  |
| BQ177SS | 3/8-24 UNF, special tube tee |  |  | Spring Guards |  | 3 |
| Female Unions BQ111SS | 7/16-24 UNS | 6 |  | BQ231 BQ232 | for 3/16” tube for 1/4” tube |  |
| Male Unions BQ292SS | 3/8-24 UNF (-3 JIC) | 6 |  | Tubing T187-20SS | 3/16” tube, 20ft coil | 3 |
| BQ293SS | 3/8-24 UNF (-3 JIC), long |  |  | T250-20SS | 1/4” tube, 20ft coil |  |
|  |  |  |  | T312-20SS | 5/16” tube, 20ft coil |  |
|  |  |  |  | T375-20SS | 3/8” tube, 20ft coil |  |

Index

| Part Number | Page | Part Number | Page | Part Number | Page | Part Number | Page |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5MBT-5MJ | 9 | BQ89 | 7 | BQ162 | 12 | BQ313-SV | 3 |
| 7MBT-6MJ | 9 | BQ90A | 7 | BQ163 | 12 | BQ315 | 12 |
| 14MMBT-6MJ | 9 | BQ91A | 7 | BQ164 | 12 | BQ316 | 12 |
| 16MMBT-6FI | 9 | BQ92A | 7 | BQ165 | 12 | BQ318 | 19 |
| 16MMBT-6MJ | 9 | BQ93A | 7 | BQ166 | 12 | BQ319 | 19 |
| 18MMBT-6MJ | 9 | BQ94A | 7 | BQ167 | 12 | BQ320 | 3 |
| BQ-01 | 19 | BQ95A | 7 | BQ168 | 9 | BQ321 | 3 |
| BQ13 | 7 | BQ110 | 6 | BQ170 | 5 | BQ322 | 3 |
| BQ18 | 12 | BQ111 | 6 | BQ171 | 5 | BQ350 | 16 |
| BQ19 | 12 | BQ111SS | 6 | BQ172 | 5 | BQ350-1 | 16 |
| BQ26 | 12 | BQ112 | 6 | BQ173 | 5 | BQ350A | 16 |
| BQ33 | 5 | BQ113 | 6 | BQ174 | 5 | BQ351 | 16 |
| BQ34 | 5 | BQ114 | 6 | BQ176 | 9 | BQ352 | 16 |
| BQ40 | 8 | BQ115 | 6 | BQ177SS | 7 | BQ352-BQ | 16 |
| BQ41 | 8 | BQ116 | 6 | BQ178 | 11 | BQ360 | 7 |
| BQ42 | 8 | BQ117 | 6 | BQ179 | 11 | BQ361 | 7 |
| BQ43 | 8 | BQ119 | 6 | BQ181 | 12 | BQ378 | 7 |
| BQ44 | 8 | BQ120 | 8 | BQ182 | 9 | BQ379 | 8 |
| BQ45 | 8 | BQ121 | 8 | BQ183 | 9 | BQ380 | 8 |
| BQ46 | 8 | BQ122 | 8 | BQ187 | 8 | BQ383 | 8 |
| BQ47 | 8 | BQ123 | 8 | BQ202 | 3 | BQ392 | 11 |
| BQ48 | 8 | BQ124 | 8 | BQ222 | 6 | BQ393 | 11 |
| BQ49 | 8 | BQ125 | 8 | BQ223 | 11 | BQ402 | 15 |
| BQ50 | 8 | BQ127 | 8 | BQ224 | 11 | BQ405 | 14 |
| BQ55 | 5 | BQ128 | 9 | BQ225 | 11 | BQ406 | 15 |
| BQ56A | 7 | BQ129 | 9 | BQ226 | 5 | BQ407 | 17 |
| BQ56ASS | 7 | BQ130 | 9 | BQ227 | 5 | BQ408 | 17 |
| BQ57 | 5 | BQ131 | 9 | BQ228 | 5 | BQ412 | 19 |
| BQ60 | 11 | BQ132 | 9 | BQ231 | 3 | BQ601 | 15 |
| BQ61 | 11 | BQ134 | 9 | BQ232 | 3 | BQ602 | 15 |
| BQ62 | 11 | BQ135 | 9 | BQ244 | 10 | BQ602C | 15 |
| BQ63 | 11 | BQ141 | 5 | BQ246 | 8 | BQ813 | 10 |
| BQ71 | 6 | BQ144 | 9 | BQ247 | 8 | BQ1016 | 10 |
| BQ72 | 4 | BQ145 | 9 | BQ248 | 8 | BQ1020 | 12 |
| BQ75 | 7 | BQ147 | 6 | BQ249 | 6 | BQ1028 | 14 |
| BQ77 | 7 | BQ148 | 6 | BQ250 | 6 | BQ1028-75 | 14 |
| BQ78 | 7 | BQ149 | 9 | BQ252 | 9 | BQ1030 | 14 |
| BQ79 | 7 | BQ150 | 11 | BQ260 | 8 | BQ1030-MB | 14 |
| BQ80 | 7 | BQ151 | 11 | BQ261 | 8 | BQ1118 | 10 |
| BQ81 | 7 | BQ152 | 11 | BQ262 | 8 | BQ1218 | 10 |
| BQ82 | 7 | BQ153 | 5 | BQ264 | 7 | BQ1320 | 10 |
| BQ83 | 7 | BQ155 | 11 | BQ265 | 7 | BQ1421 | 10 |
| BQ84 | 7 | BQ156 | 8 | BQ271 | 11 | BQ1522 | 10 |
| BQ85 | 7 | BQ157 | 9 | BQ292 | 6 | BQ1624 | 10 |
| BQ86 | 7 | BQ160 | 12 | BQ292SS | 6 | BQ2100 | 17 |
| BQ87 | 7 | BQ161 | 12 | BQ293SS | 6 | BQ2101B | 15 |

Index

| Part Number | Page | Part Number | Page | Part Number | Page | Part Number | Page |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BQ2103 | 15 | BQ7006 | 13 | BQ9186 | 4 | FL10-5S | 19 |
| BQ2200 | 17 | BQ7007 | 13 | BQ9187 | 4 | FL10-6S | 19 |
| BQ2200-1 | 17 | BQ7009 | 13 | BQ9188 | 4 | FL10-14mm | 19 |
| BQ2200W | 17 | BQ7010 | 13 | BQ9189 | 4 | FL11-14mm | 18 |
| BQ2577 | 4 | BQ7012 | 13 | BQ9190 | 4 | FL12-14mm | 18 |
| BQ2577SS | 4 | BQ7014 | 13 | BQ9191 | 4 | FL13-5 | 18 |
| BQ2578 | 4 | BQ7812 | 7 | BQ9192 | 4 | FL13-6 | 18 |
| BQ3052 | 6 | BQ8075 | 10 | BQ9193 | 4 | FL14-5 | 18 |
| BQ3053 | 6 | BQ8076 | 10 | BQ9194 | 4 | FL14-6 | 18 |
| BQ3052SS | 6 | BQ8077 | 10 | BQ9195 | 4 | FL15 | 18 |
| BQ3858 | 10 | BQ8078 | 10 | BQ9196 | 4 | FL20-5 | 18 |
| BQ4276 | 4 | BQ8083 | 10 | BQ9197 | 4 | FL20-6 | 18 |
| BQ4277 | 4 | BQ8084 | 10 | BQ9198 | 4 | FL21 | 19 |
| BQ4285 | 4 | BQ8085 | 10 | BQ9199 | 4 | FL22 | 19 |
| BQ4433 | 12 | BQ8086 | 10 | BQ9207 | 5 | FL25 | 19 |
| BQ4434 | 12 | BQ8086SS | 10 | BQ9208 | 5 | FL26 | 19 |
| BQ4435 | 12 | BQ8087 | 10 | BQ9209 | 5 | FL34 | 6, 19 |
| BQ4436 | 12 | BQ8088 | 10 | BQ9210 | 5 | FL35 | 6, 19 |
| BQ4438 | 12 | BQ8089 | 10 | BQ9300 | 8 | FL36 | 19 |
| BQ4439 | 12 | BQ8090 | 10 | BQ9301 | 8 | FL37 | 19 |
| BQ4440 | 12 | BQ8092 | 10 | BQ9304 | 8 | FL38 | 19 |
| BQ4441 | 12 | BQ8093 | 10 | BQ9305 | 8 | FL39 | 19 |
| BQ4442 | 12 | BQ8094 | 10 | BQB06 | 6 | T8-FF | 19 |
| BQ4443 | 12 | BQ8095 | 10 | BQB244 | 6 | T10-FF | 19 |
| BQ4444 | 12 | BQ8096 | 10 | BQFL4500 | 19 | T187-20SS | 3 |
| BQ4445 | 12 | BQ8097 | 10 | FL01-5 | 19 | T187-25CN | 2 |
| BQ4446 | 12 | BQ8098 | 10 | FL01-6 | 19 | T187-25SE | 2 |
| BQ4510 | 4 | BQ8099 | 10 | FL02-5 | 18 | T187-100CN | 2 |
| BQ4511 | 4 | BQ9148 | 4, 19 | FL02-6 | 18 | T187-100SE | 2 |
| BQ4511LT | 4 | BQ9149 | 4, 19 | FL03-5 | 18 | T250-20SS | 3 |
| BQ4511SS | 4 | BQ9156 | 5 | FL03-6 | 18 | T250-25CN | 2 |
| BQ4512 | 4 | BQ9157 | 4 | FL04-6 | 18 | T250-25SE | 2 |
| BQ4513 | 4 | BQ9166 | 4 | FL05-4 | 18 | T250-100CN | 2 |
| BQ4513LT | 4 | BQ9169 | 4 | FL06-5-4 | 18 | T250-100SE | 2 |
| BQ4513SS | 4 | BQ9174 | 5 | FL06-6-5 | 18 | T312-20SS | 3 |
| BQ5555 | 4 | BQ9175 | 5 | FL07-5 | 18 | T312-25CN | 2 |
| BQ5555SS | 4 | BQ9176 | 5 | FL07-6 | 18 | T312-25SE | 2 |
| BQ5842 | 4 | BQ9177 | 5 | FL08-4-5 | 18 | T375-20SS | 3 |
| BQ5842SS | 4 | BQ9178 | 5 | FL08-5-5 | 18 | T375-25CN | 2 |
| BQ6118 | 4 | BQ9179 | 5 | FL08-5-6 | 18 | T375-25SE | 2 |
| BQ7000 | 13 | BQ9180 | 4 | FL08-6-5 | 18 |  |  |
| BQ7001 | 13 | BQ9181 | 4 | FL08-6-6 | 18 |  |  |
| BQ7002 | 13 | BQ9182 | 4 | FL09-5-5 | 18 |  |  |
| BQ7003 | 13 | BQ9183 | 4 | FL09-5-6 | 18 |  |  |
| BQ7004 | 13 | BQ9184 | 4 | FL09-6-5 | 18 |  |  |
| BQ7005 | 13 | BQ9185 | 4 | FL09-6-6 | 18 |  |  |

Find a Dealer

# BrakeQuip Products are Available

from Authorized Dealers in more than 60 Countries

Find your local dealer at

brakequip.com/find-a-dealer

#### Standard Profile Brake Hoses Low-Profile Brake Hoses Power Steering Hoses

Metal Tube Hardware Tools

Replacement or Custom Brake, Clutch, and Power Steering Hoses

Replace your old hoses to improve system performance. Custom hoses can reduce the number of leak points, provide for proper hose routing, and offer a personalized touch.

Standard crimp on rubber hose

Low-profile crimp on braided hose

Standard crimp on braided hose

High-pressure power steering hose with a custom tube form

BrakeQuip hoses, tubes, and fittings are designed to meet or exceed all international safety and quality standards.

Every BrakeQuip brake hose is pressure tested to 3000 psi for quality that you can trust.

Complete your next build with the right tools. Whether double flaring our stainless steel or shaping our signature EziBend copper-nickel, BrakeQuip hand tools are fast and consistent and deliver clean, professional results.

BrakeQuip offers a comprehensive selection of fuel line and hardware components.

Every BrakeQuip hose assembly is DOT compliant and meets all international standards. For replacement or custom hoses, contact one of over 600 local BrakeQuip dealers.

Learn more at brakequip.com.

V

DOT

SINCE 1998

ULLY

B

O PL ANT

To find a local hose dealer, visit

brakequip.com/find-a-dealer