BVA H Y D R A U L I C S "

Dndlustriai Hydraulic Equipment

The BVA Advantage

BVA Hydraulics was launched in 2003 and is centrally located in the United States in Kansas City, Missouri. BVA is one of the Shinn Fu of America Companies including; Hein-Werner, Omega Lift, Pro-Lift, Blackhawk, Porto-Power, Bone Creeper, Omega Lighting and Save Phace. Shinn Fu was founded in Taiwan in 1971 and began doing business in the United States in 1978. Shinn Fu has over 2,000 Employees worldwide with its headquarters in Taipei, Taiwan.

Confidence Built on Quality

ISO 9001

BVA products are manufactured through an International Quality Assurance System, ISO 9001 which involves consistent observation of administration, management, manufacturing, and production control. Under this system, the related operations are monitored on all aspects from the purchase order to the assembly line and through to shipping the order.

Products comply with the following industry standards

ASME B30.1

The cylinders comply with load testing requirements and / or the safety factor for material yield strength while still functioning properly without oil leakage.

ANSI B40.1

BVA pressure gauges meet the criteria for grade A.

SAE 100

BVA hydraulic hoses comply with the Material Handling Institute’s specification of minimum life cycles

and minimum burst pressure rating.

BVA Limited Lifetime Warranty

BVA products are warranted to be free of defects in materials and workmanship. Any BVA product that does not conform to specification will be repaired or credited (at BVA’s option) at BVA’s expense for the life of the product. Contact BVA for disposition of this warranty.

This warranty does not cover ordinary wear and tear, abuse, misuse, alteration, or use of improper fluids or maintenance as determined by BVA and its authorized service centers. All electric motors, gas engines, batteries and other parts or materials sold with or part of BVA products but are separately warranted by another manufacturer, are covered by that manufacturer’s warranty

and not the BVA Lifetime Warranty. BVA shall not be liable for consequential or incidental damage or loss.

Contact Us

For technical support, call

1-888-332-6419. For questions about your product, contact your local BVA dealer.

BVA Website

To view our most up-to-date catalog or view our new product, visit our website: www.BVAhydraulics.com

Repair Parts

We offer a large range of product replacement parts, please contact your BVA dealer for more details.

1-888-332-6419

2	www.BVAhydraulics.com

Model Number Index

Cylinders

H

6

HUD

16

HL

26

HULN

32

HC

36

HP

41

HU

10

HG

18

HF

28

HLNF

33

HDC

38

HT

42

HD

12

HDG

22

HLN

30

HPD

43

Pages 6-43

HUC/HUDC 40

Accessories 34

Pumps

| P Single Acting 46 Hand Pumps | PAR Rotary Air 49 Pumps | PA Double Acting 50 Air Pumps | PG Gas Engine 60 Pumps |
| --- | --- | --- | --- |
| P Double Acting 46 Hand Pumps | PA All Position 50 Air Pumps | PE 1-Phase 52 Electric Pumps | PD Diesel Engine 60 Pumps |
| P57 Foot Pumps 48 | PA Single Acting 50 Air Pumps | PE 3-Phase 56 Electric Pumps | PU Universal Motor 63 Pumps |

Combo Kits

| H | General Purpose Cylinder Combinations | 64 | HL | Low Profile Cylinder Combinations | 65 |
| --- | --- | --- | --- | --- | --- |
| HF | Flat Body Cylinder Combinations | 64 | HLN | Lock Nut Cylinder Combinations | 65 |
| HC | Hollow Hole Cylinder Combinations | 65 | HU | Aluminum Cylinder Combinations | 65 |

Pages 44-63

Pages 64-65

Control Valves & Accessories

Flow Control Valves

66

Gauge Adapters

69

Solenoid Valves

70

Wireless Controller

71

Pressure Switch

72

Directional Control Valves

74

Manifolds	68

Hydraulic Oil

70

Hydraulic Couplers

71

External Pressure Adj

71

Hydraulic Hoses

72

Fittings

75

69

70

72

Pump Mounted 73

Valves

Manifold Blocks

Foot Pedals 71

Coupler Dust Cover

Pressure Gauges

Pages 66-75

| Movable Head 77 Shop Presses | Shop Press 80 Guards | Oil Field Valve 82 Seat Pullers | Hydraulic Gear Pullers 83 |
| --- | --- | --- | --- |
| Fixed Head 78 Shop Presses | Flange Spreaders 81 | Aluminum Cylinder 82 w/ Base/Needle Valve | Mechanical Gear 83 Pullers |
| Shop Press 80 Accessories | Duck Bill 81 Spreaders | Hydraulic Bench Vise 82 | Nut Splitters 83 |

Presses/Tools

Pages 76-83

| Maintenance Kits/Rail Cylinders | 85 | Manual Bottle Jacks | 88 | Mini Jacks | 89 |
| --- | --- | --- | --- | --- | --- |
| Lifting Bags | 86 | Air/Manual Bottle Jacks | 88 | OMNI All-Position Jacks | 89 |
| Lifting Bag Accessories | 87 | Toe Jacks | 88 | Orbital Jacks | 89 |

Maintenance Kits/Bottle Jacks

Pages 84-89

| BVA Cross Reference Chart | 90 | Hydraulic Formula Reference | 94 |
| --- | --- | --- | --- |
| Safety Instructions | 92 |  |  |
| Maintaining Your Equipment | 94 |  |  |

Cross-Reference & Engineering

Pages 90-94

© Copyright 2016 SFA Companies

All rights reserved. Any copying or other use of the material in this catalog (text, illustrations, drawings, photos) without

prior express written consent is prohibited. Information in this catalog is subject to change without prior notice.	3

T

06

30

DC

H

Cylinder Matrix Chart	1

2	3	4	5

Cylinder Type

Cylinder Options

Cylinder Capacity

Cylinder Stroke

Suffix

Code

| H Hydraulic Cylinder | 02 | 2, 2.5 Tons | 75 | 75 Tons | 00 | 0.63" |
| --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 4 Tons | 100 | 100 Tons | 01 | 1" |
| 2 - Cylinder Options | 05 | 5 Tons | 150 | 150 Tons | 02 | 2" |
|  | 09 | 9 Tons | 160 | 160 Tons | 04 | 4" |
| C S/A - Hollow Hole | 10 | 10 Tons | 200 | 200 Tons | 05 | 5" |
| D D/A - General Purpose | 12 | 12 Tons | 250 | 250 Tons | 06 | 6" |
| DC D/A - Hollow Hole | 15 | 15 Tons | 300 | 300 Tons | 08 | 8" |
| DG D/A - High Tonnage | 16 | 16 Tons | 400 | 400 Tons | 10 | 10" |
| F S/A - Flat Body | 20 | 20 Tons | 500 | 500 Tons | 12 | 12" |
| G S/A - Load Return High Tonnage | 25 | 25 Tons | 600 | 600 Tons | 13 | 13" |
| L S/A - Low Profile | 30 | 30 Tons | 700 | 700 Tons | 14 | 14" |
| LN S/A - Lock Nut | 50 | 50 Tons | 800 | 800 Tons | 18 | 18" |
| LNF S/A - Lock Nut Low Profile | 55 | 55 Tons | 1,000 | 1,000 Tons | 20 | 20" |
| P S/A - Pull | 60 | 60 Tons | 1,500 | 1,500 Tons | 24 | 24" |
| PD Precision Double Acting | 75 | 75 Tons |  |  | 32 | 32" |
| T S/A - Threaded |  |  |  |  |  |  |

1 - Cylinder Type	3 - Cylinder Capacity	4 - Cylinder Stroke

U	S/A - Aluminum

UC S/A - Aluminum Hollow Hole UDC D/A - Aluminum Hollow Hole ULN S/A - Aluminum Lock Nut

5 - Suffix Code

T	BVA Factory Designation

US	US Made Cylinder

XT	No Internal Threads, HC Series only

B	Flat Body Cylinder Kit

Cylinder Accessories

See the assortment

of accessories designed

Safety Practices

Good industry practice recommends not exceeding

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

to work with our products.

pg34

80% of maximum rated capacities of all our products.

methods of operation. Always be prepared.

pg 92

Heavy duty, heat treated, grooved saddle provides better surface traction during operation

Rod wiper seals out dirt and contamination

Industrial hard chrome plated rod prevents scratching and corrosion

One piece steel rod stopper for extra strength and prevents over travel

Steel stop ring for rod over travel protection

Collar fixture threads

Cylinders are burnish rolled smoother finish than honing, which reduces scoring and prolongs seal life

Polyurethane cup seal provides optimum performance

Rectangular return spring design eliminates spring breakage and provides quicker retraction

1-888-332-6419

4	www.BVAhydraulics.com

One piece high grade alloy bearing for side load protection and long life

High flow ball coupler provides more flow than conventional couplers

5

Cylinders

| Cylinder | Cylinder Type | Max Capacity Range | Max Stroke Range | Page Number / |
| --- | --- | --- | --- | --- |
| Series |  | (tons) | (in) | Reference Image |
| H | Single Acting Cylinders | 2-100 | 0.63-14.25 | 6-9 |
| HU | Aluminum Single Acting Cylinders | 20-150 | 1.97-10.00 | 10-11 |
| HD | Double Acting Cylinders | 10-500 | 5.90-24.00 | 12-15 |
| HUD | Aluminum Double Acting Cylinders | 30-150 | 1.97-10.00 | 16-17 |
| HG | High Tonnage Single Acting Cylinders | 100-1500 | 1.97-11.81 | 18-21 |
| HDG | High Tonnage Double Acting Cylinders | 200-1500 | 6.00-24.00 | 22-25 |
| HL | Low Profile Cylinders | 10-100 | 1.5-2.44 | 26-27 |
| HF | Flat Body Cylinders | 5-150 | 0.24-0.63 | 28-29 |
| HLN | Lock Nut Cylinders | 55-300 | 1.97-11.81 | 30-31 |
| HULN | Aluminum Lock Nut Cylinders | 30-150 | 1.97-11.81 | 32 |
| HLNF | Low Profile Lock Nut Cylinders | 60-250 | 1.77-1.97 | 33 |
| - | Cylinder Accessories | - | - | 34-35 |
| HC | Hollow Hole Cylinders | 12-100 | 0.31-6.11 | 36-37 |
| HDC | Double Acting Hollow Hole Cylinders | 30-150 | 1.50-10.13 | 38-39 |
| HUC | Aluminum Hollow Hole Cylinders | 30-60 | 3.00-5.91 | 40 |
| HUDC | Aluminum Double Acting Hollow Hole Cylinders | 30-150 | 5.91-9.84 | 40 |
| HP | Pull Cylinders | 2.5-10 | 5.00-5.95 | 41 |
| HT | Threaded Cylinders | 4-20 | 5.07-9.96 | 42 |
| HPD | Precision Double Acting Cylinders | 9-16 | 1.77-3.14 | 43 |
| Pump Section Page 44 |  |  |  |  |

### Single Acting Cylinders

H Series - General Purpose, Spring Return

H1514

Capacity:

2 - 100 Tons*

Stroke:

0.63 - 14.25 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

1.63 - 32.99 Inches

H10010

* Larger Tonnages

For tonnages that exceed 100, please refer to our HG Section

pg

18

H7513

H1501

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

HU Series – Aluminum Cylinders

Aluminum cylinders make a great lightweight alternative being up to 60% lighter

Single Acting Cylinder H1506 in the collapsed position with CB15 cylinder base for safety.

Single Acting Cylinder H5513 in the collapsed

position ready to lift a load

- Collar threads and base

mounting holes for fixturing

- Hard chrome plated piston rod protects against corrosion

and damage

- Polyurethane U-Cup seals provide a sure seal and long life

- Solid one piece alloy steel body

- One piece aluminum alloy bearing protects against cylinder wall damage

- CH38F high flow coupler with dust cap is standard on most cylinders**

than comparable steel models.

1-888-332-6419

pg10

###### 6	www.BVAhydraulics.com

H Series

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Cylinder Accessories

See the assortment

of accessories designed to work with our products.

pg

34

Combo Kits

Most cylinders are available in easy to order kits: Cylinder, Gauge, Couplers, Hose

and Pump.

pg64

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

Optional Tilt-Saddles

Saddles tilt to a 5% maximum to adjust to the load. Match up these saddles with the following cylinders:

SDT05    →

SDT10    →

H05 Cylinders*

H10 Cylinders

SDT15

→

H15 Cylinders

SDT25    →	H25 Cylinders

SDT55 → H55, H75, H100 Cylinders

* Except the H0500 cylinder

pg 35

7

Cylinders

Quick Selection Guide

| Cylinder Capacity (ton) | Stroke (in) | Model Number | Collapsed Height (in) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 2 | 3.07 | H0203* | 5.39 | 2.28 | 2.0 |
| 5 | 0.63 | H0500** | 1.63 | 0.69 | 1.9 |
|  | 1.00 | H0501 | 4.58 | 1.00 | 3.1 |
|  | 3.07 | H0503 | 6.65 | 3.07 | 3.2 |
|  | 5.08 | H0505 | 8.70 | 5.08 | 3.9 |
|  | 7.09 | H0507 | 10.71 | 7.09 | 4.7 |
|  | 9.00 | H0509 | 12.75 | 9.03 | 5.5 |
| 10 | 0.98 | H1001 | 3.62 | 2.19 | 3.9 |
|  | 2.01 | H1002 | 4.84 | 4.50 | 5.0 |
|  | 4.02 | H1004 | 6.85 | 9.00 | 7.7 |
|  | 5.94 | H1006 | 9.84 | 13.30 | 9.5 |
|  | 7.95 | H1008 | 11.85 | 17.81 | 11.3 |
|  | 9.96 | H1010 | 13.86 | 22.31 | 13.1 |
|  | 11.97 | H1012 | 15.87 | 26.81 | 15.0 |
|  | 14.02 | H1014 | 17.72 | 31.40 | 16.7 |
| 15 | 0.98 | H1501 | 4.88 | 3.07 | 6.9 |
|  | 2.01 | H1502 | 5.87 | 6.31 | 8.5 |
|  | 3.98 | H1504 | 7.87 | 12.49 | 11.1 |
|  | 5.98 | H1506 | 10.63 | 18.77 | 14.8 |
|  | 7.99 | H1508 | 12.64 | 25.08 | 17.3 |
|  | 10.00 | H1510 | 14.65 | 31.40 | 20.0 |
|  | 12.01 | H1512 | 16.65 | 37.71 | 22.5 |
|  | 14.02 | H1514 | 18.66 | 44.02 | 25.2 |
| 25 | 1.02 | H2501 | 5.63 | 5.25 | 12.1 |
|  | 2.01 | H2502 | 6.61 | 10.35 | 14.0 |
|  | 4.02 | H2504 | 8.62 | 20.70 | 18.0 |
|  | 6.10 | H2506 | 10.71 | 31.41 | 22.2 |
|  | 8.11 | H2508 | 12.72 | 41.77 | 26.3 |
|  | 10.12 | H2510 | 14.72 | 52.12 | 30.3 |
|  | 12.20 | H2512 | 16.77 | 62.83 | 34.3 |
|  | 14.25 | H2514 | 18.74 | 73.39 | 38.3 |
| 30 | 8.27 | H3008 | 15.24 | 53.67 | 40.7 |
| 55 | 2.01 | H5502 | 6.93 | 21.97 | 33.0 |
|  | 4.06 | H5504 | 8.94 | 44.57 | 41.0 |
|  | 6.26 | H5506† | 11.14 | 68.73 | 57.5 |
|  | 10.25 | H5510† | 15.16 | 112.55 | 67.4 |
|  | 13.27 | H5513† | 18.15 | 145.70 | 87.0 |
| 75 | 6.14 | H7506† | 11.22 | 98.85 | 74.0 |
|  | 13.11 | H7513† | 19.37 | 211.07 | 122.0 |
| 100 | 2.01 | H10002A† | 8.62 | 41.34 | 66.0 |
|  | 6.61 | H10006† | 14.06 | 136.03 | 127.0 |
|  | 10.24 | H10010† | 17.68 | 210.74 | 153.0 |

See pages 8-9 for detailed specifications.

* H0203 load return.

** H0500 comes with CRZ14F coupler.

† Cylinder is equipped with carrying handle.

### Single Acting Cylinders

| Cylinder Capacity (ton) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure at Capacity (psi) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 3.07 | H0203* | 0.76 | 5,263 | 2.28 | 2.0 |
| 5 | 0.63 | H0500** | 1.10 | 9,091 | 0.69 | 1.9 |
|  | 1.00 | H0501 | 1.00 | 10,000 | 1.00 | 3.1 |
|  | 3.07 | H0503 | 1.00 | 10,000 | 3.07 | 3.2 |
|  | 5.08 | H0505 | 1.00 | 10,000 | 5.08 | 3.9 |
|  | 7.09 | H0507 | 1.00 | 10,000 | 7.09 | 4.7 |
|  | 9.00 | H0509 | 1.00 | 10,000 | 9.03 | 5.5 |
| 10 | 0.98 | H1001 | 2.24 | 8,928 | 2.19 | 3.9 |
|  | 2.01 | H1002 | 2.24 | 8,928 | 4.50 | 5.0 |
|  | 4.02 | H1004 | 2.24 | 8,928 | 9.00 | 7.7 |
|  | 5.94 | H1006 | 2.24 | 8,928 | 13.30 | 9.5 |
|  | 7.95 | H1008 | 2.24 | 8,928 | 17.81 | 11.3 |
|  | 9.96 | H1010 | 2.24 | 8,928 | 22.31 | 13.1 |
|  | 11.97 | H1012 | 2.24 | 8,928 | 26.81 | 15.0 |
|  | 14.02 | H1014 | 2.24 | 8,928 | 31.40 | 16.7 |
| 15 | 0.98 | H1501 | 3.14 | 9,554 | 3.07 | 6.9 |
|  | 2.01 | H1502 | 3.14 | 9,554 | 6.31 | 8.5 |
|  | 3.98 | H1504 | 3.14 | 9,554 | 12.49 | 11.1 |
|  | 5.98 | H1506 | 3.14 | 9,554 | 18.77 | 14.8 |
|  | 7.99 | H1508 | 3.14 | 9,554 | 25.08 | 17.3 |
|  | 10.00 | H1510 | 3.14 | 9,554 | 31.40 | 20.0 |
|  | 12.01 | H1512 | 3.14 | 9,554 | 37.71 | 22.5 |
|  | 14.02 | H1514 | 3.14 | 9,554 | 44.02 | 25.2 |
| 25 | 1.02 | H2501 | 5.15 | 9,708 | 5.25 | 12.1 |
|  | 2.01 | H2502 | 5.15 | 9,708 | 10.35 | 14.0 |
|  | 4.02 | H2504 | 5.15 | 9,708 | 20.70 | 18.0 |
|  | 6.10 | H2506 | 5.15 | 9,708 | 31.41 | 22.2 |
|  | 8.11 | H2508 | 5.15 | 9,708 | 41.77 | 26.3 |
|  | 10.12 | H2510 | 5.15 | 9,708 | 52.12 | 30.3 |
|  | 12.20 | H2512 | 5.15 | 9,708 | 62.83 | 34.3 |
|  | 14.25 | H2514 | 5.15 | 9,708 | 73.39 | 38.3 |
| 30 | 8.27 | H3008 | 6.49 | 9,245 | 53.67 | 40.7 |
| 55 | 2.01 | H5502 | 10.98 | 10,018 | 21.97 | 33.0 |
|  | 4.06 | H5504 | 10.98 | 10,018 | 44.57 | 41.0 |
|  | 6.26 | H5506† | 10.98 | 10,018 | 68.73 | 57.5 |
|  | 10.25 | H5510† | 10.98 | 10,018 | 112.55 | 67.4 |
|  | 13.27 | H5513† | 10.98 | 10,018 | 145.70 | 87.0 |
| 75 | 6.14 | H7506† | 16.10 | 9,316 | 98.85 | 74.0 |
|  | 13.11 | H7513† | 16.10 | 9,316 | 211.07 | 122.0 |
| 100 | 2.01 | H10002A† | 20.58 | 9,718 | 41.34 | 66.0 |
|  | 6.61 | H10006† | 20.58 | 9,718 | 136.03 | 127.0 |
|  | 10.24 | H10010† | 20.58 | 9,718 | 210.74 | 153.0 |

H Series - Spring Return

G

3/8"-18NPTF

C	E

A

B

** H0500 Only

Y

Y

D1

X D2

X

D		 F

K

J

E

L

H	B

A

C

3/8"-18NPTF

Z

G

1-888-332-6419

8	www.BVAhydraulics.com

- H0203 load return.

** H0203 comes with CRZ14F coupler.

† Cylinder is equipped with carrying handle.

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore C (in) | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) | Base to Inlet Port G (in) | Collar Thread H (in) |  | Collar Thread Length L (in) | Rod Internal Thread J (in) | Rod Thread Length K (in) | Base Mounting Hole (in) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Bolt Circle X | Thread Y | Thread Depth Z |
| H0203* | 5.39 | 8.46 | 1.00 | 1.31 | 0.87 | 1.00 | 0.46 | 1¼”-16 |  | 0.54 | - | - | - | - | - |
| H0500** | 1.63 | 2.26 | 1.18 | 2.37x1.59‡ | 0.98 | - | 0.75 | - |  | - | - | - | 1.10 | - | - |
| H0501 | 4.58 | 5.58 | 1.13 | 1.50 | 1.00 | 0.98 | 0.79 | 1½”-16 |  | 1.10 | ¾”-16UNF | 0.63 | 1.00 | ¼”-20UNC | 0.55 |
| H0503 | 6.65 | 9.72 | 1.13 | 1.50 | 1.00 | 0.98 | 0.79 | 1½”-16 |  | 1.10 | ¾”-16UNF | 0.63 | 1.00 | ¼”-20UNC | 0.55 |
| H0505 | 8.70 | 13.78 | 1.13 | 1.50 | 1.00 | 0.98 | 0.79 | 1½”-16 |  | 1.10 | ¾”-16UNF | 0.63 | 1.00 | ¼”-20UNC | 0.55 |
| H0507 | 10.71 | 17.80 | 1.13 | 1.50 | 1.00 | 0.98 | 0.79 | 1½”-16 |  | 1.10 | ¾”-16UNF | 0.63 | 1.00 | ¼”-20UNC | 0.55 |
| H0509 | 12.75 | 21.75 | 1.12 | 1.50 | 1.00 | 0.98 | 0.79 | 1½”-16 |  | 1.10 | ¾”-16UNF | 0.63 | 1.00 | ¼”-20UNC | 0.55 |
| H1001 | 3.62 | 4.60 | 1.69 | 2.26 | 1.49 | - | 0.77 | 2¼”-14 |  | 1.18 | - | 0.24 | 1.56 | ”-18UNC | 0.55 |
| H1002 | 4.84 | 6.85 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1004 | 6.85 | 10.87 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1006 | 9.84 | 15.78 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1008 | 11.85 | 19.80 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1010 | 13.86 | 23.82 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1012 | 15.87 | 27.84 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1014 | 17.72 | 31.74 | 1.69 | 2.26 | 1.49 | 1.38 | 0.77 | 2¼”-14 |  | 1.18 | 1”-8UNC | 0.79 | 1.56 | ”-18UNC | 0.55 |
| H1501 | 4.88 | 5.86 | 2.00 | 2.75 | 1.63 | 1.50 | 0.77 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1502 | 5.87 | 7.88 | 2.00 | 2.75 | 1.63 | 1.50 | 0.77 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1504 | 7.87 | 11.85 | 2.00 | 2.75 | 1.63 | 1.50 | 0.77 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1506 | 10.63 | 16.61 | 2.00 | 2.75 | 1.63 | 1.50 | 0.98 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1508 | 12.64 | 20.63 | 2.00 | 2.75 | 1.63 | 1.50 | 0.98 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1510 | 14.65 | 24.65 | 2.00 | 2.75 | 1.63 | 1.50 | 0.98 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1512 | 16.65 | 28.66 | 2.00 | 2.75 | 1.63 | 1.50 | 0.98 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H1514 | 18.66 | 32.68 | 2.00 | 2.75 | 1.63 | 1.50 | 0.98 | 2¾”-16 |  | 1.18 | 1”-8UNC | 0.98 | 1.88 | ⅜”-16UNC | 0.51 |
| H2501 | 5.63 | 6.65 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2502 | 6.61 | 8.62 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2504 | 8.62 | 12.64 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2506 | 10.71 | 16.81 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2508 | 12.72 | 20.83 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2510 | 14.72 | 24.84 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2512 | 16.77 | 28.97 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H2514 | 18.74 | 32.99 | 2.56 | 3.35 | 2.24 | 1.97 | 0.98 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.31 | ½”-13UNC | 0.75 |
| H3008 | 15.24 | 23.51 | 2.87 | 4.00 | 2.24 | 1.97 | 1.97 | 3 | ”-12 | 1.93 | 1½”-16 | 1.18 | 2.20 | - | - |
| H5502 | 6.93 | 8.94 | 3.74 | 5.00 | 3.15 | 2.76 | 1.22 | 5”-12 |  | 2.17 | - | - | 3.75 | ½”-13UNC | 0.75 |
| H5504 | 8.94 | 13.00 | 3.74 | 5.00 | 3.15 | 2.76 | 1.22 | 5”-12 |  | 2.17 | - | - | 3.75 | ½”-13UNC | 0.75 |
| H5506† | 11.14 | 17.40 | 3.74 | 5.00 | 3.15 | 2.76 | 1.22 | 5”-12 |  | 2.17 | - | - | 3.75 | ½”-13UNC | 0.75 |
| H5510† | 15.16 | 25.41 | 3.74 | 5.00 | 3.15 | 2.76 | 1.22 | 5”-12 |  | 2.17 | - | - | 3.75 | ½”-13UNC | 0.75 |
| H5513† | 18.15 | 31.42 | 3.74 | 5.00 | 3.15 | 2.76 | 1.22 | 5”-12 |  | 2.17 | - | - | 3.75 | ½”-13UNC | 0.75 |
| H7506† | 11.22 | 18.36 | 4.53 | 5.79 | 3.74 | 2.76 | 1.18 | 5¾”-12 |  | 1.77 | - | - | - | - | - |
| H7513† | 19.37 | 32.48 | 4.53 | 5.79 | 3.74 | 2.76 | 1.18 | 5¾”-12 |  | 1.77 | - | - | 3.94 | ”-10UNC | 0.85 |
| H10002A† | 8.62 | 10.63 | 5.12 | 6.25 | 4.13 | 2.76 | 1.50 | 6¼”-12 |  | 2.17 | - | - | - | - | - |
| H10006† | 14.06 | 20.67 | 5.12 | 6.25 | 4.13 | 2.76 | 1.57 | 6¼”-12 |  | 1.77 | - | - | 5.50 | ¾”-10UNC | 1.00 |
| H10010† | 17.68 | 27.92 | 5.12 | 6.25 | 4.13 | 2.76 | 1.57 | 6¼”-12 |  | 1.77 | - | - | 5.50 | ¾”-10UNC | 1.00 |

9

Cylinders

* H0203 load return.

** H0500 comes with CRZ14F coupler.

† Cylinder is equipped with carrying handle.

‡ Length x Width

### Aluminum Single Acting Cylinders

HU Series - General Purpose, Spring Return, Lightweight

HU5006

Capacity:

20 - 150 Tons

Stroke:

1.97 - 10.00 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

6.85 - 15.73 Inches

HU7506

HU2006

HU7504

Aluminum vs. Steel Cylinders

Aluminum cylinders provide a great alternative to the traditional steel cylinder in many applications. Being up to 60% lighter in weight, Aluminum cylinders are highly portable and reduce user fatigue and strain. However, due to the finite properties of Aluminum vs. Steel, these cylinders should NOT be used in high cycle applications. The BVA Aluminum cylinders are designed for a maximum safety rating of 5,000 cycles. Under normal lifting applications this should provide a long service life.

HU5006T

H5506

Aluminum Cylinders

Steel Cylinders

•

•

•

•

60% lighter in weight than steel

Highly portable and reduces user fatigue Not used for high cycle applications Under normal conditions, the maximum rated safety cycle life is 5,000 cycles

Base of the cylinder contains a steel plate*

•

•

•

•

Much heavier than aluminum cylinders Low portability with heavier cylinders which creates user fatigue and strain

Will out-perform aluminum cylinders in higher cycle applications

- Can exceed aluminum cylinders maximum safety cycle life of 5,000 cycles.

*The base of the cylinders contain a steel plate with mounting holes that are designed to protect the cylinder

from damage and should NOT be used in fixturing applications and ONLY be used to attach larger base

mounting plates for lifting stability. They are NOT designed to withstand the full ratings of the cylinders.

Lightweight aluminum alloy construction

  - Up to 60% lighter than comparable tonnage steel cylinders

  - Convenient carry handle on 50, 75 and 100 ton models

  - Hard anodized aluminum rod provides extra protection against corrosion and wear

  - Integral steel protective plate

  - Maximum working pressure: 10,000 psi / 700 bar

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

1-888-332-6419

10	www.BVAhydraulics.com

HU Series

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge. A variety of graduation’s and types to suit any need.

pg69

Related Product: Hoses

We offer a variety of hoses, fittings, lengths and materials.

pg

72

11

F

B	A

3/8”-18NPTF

G

C

D

Cylinders

E

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in²) | Internal Pressure at Capacity (psi) | Oil Capacity (in³) | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) | Base to Inlet Port G (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 1.97 | HU2002T | 4.83 | 10,000 | 9.51 | 6.85 | 8.82 | 2.48 | 3.35 | 2.00 | 1.56 | 1.08 | 6.6 |
|  | 3.94 | HU2004T | 4.83 | 10,000 | 19.02 | 8.82 | 12.76 | 2.48 | 3.35 | 2.00 | 1.56 | 1.08 | 8.1 |
|  | 5.91 | HU2006T | 4.83 | 10,000 | 28.53 | 10.79 | 16.69 | 2.48 | 3.35 | 2.00 | 1.56 | 1.08 | 9.6 |
| 30 | 1.97 | HU3002T | 6.85 | 10,000 | 13.48 | 7.13 | 9.09 | 2.95 | 3.94 | 2.36 | 1.56 | 1.32 | 10.4 |
|  | 3.94 | HU3004T | 6.85 | 10,000 | 26.96 | 9.09 | 13.03 | 2.95 | 3.94 | 2.36 | 1.56 | 1.32 | 12.0 |
|  | 5.91 | HU3006T | 6.85 | 10,000 | 40.44 | 11.06 | 16.97 | 2.95 | 3.94 | 2.36 | 1.56 | 1.32 | 13.5 |
| 50 | 1.97 | HU5002T | 10.99 | 10,000 | 21.63 | 7.32 | 9.29 | 3.74 | 5.12 | 3.15 | 2.76 | 1.20 | 16.8 |
|  | 3.94 | HU5004T | 10.99 | 10,000 | 43.25 | 9.29 | 13.23 | 3.74 | 5.12 | 3.15 | 2.76 | 1.20 | 22.6 |
|  | 5.91 | HU5006T | 10.99 | 10,000 | 64.88 | 11.26 | 17.17 | 3.74 | 5.12 | 3.15 | 2.76 | 1.20 | 25.5 |
|  | 7.87 | HU5008T | 10.99 | 10,000 | 86.51 | 13.23 | 21.10 | 3.74 | 5.12 | 3.15 | 2.76 | 1.20 | 27.1 |
|  | 9.84 | HU5010T | 10.99 | 10,000 | 108.13 | 15.20 | 25.04 | 3.74 | 5.12 | 3.15 | 2.76 | 1.20 | 31.3 |
| 75 | 4.0 | HU7504 | 15.00 | 10,000 | 60.10 | 8.73 | 12.73 | 4.37 | 6.50 | 3.88 | 3.00 | 1.50 | 29.0 |
|  | 6.0 | HU7506 | 15.00 | 10,000 | 90.20 | 10.73 | 16.73 | 4.37 | 6.50 | 3.88 | 3.00 | 1.50 | 35.0 |
| 100 | 2.0 | HU10002 | 20.70 | 9,662 | 41.40 | 6.50 | 8.50 | 5.10 | 8.00 | 4.30 | 3.50 | 1.50 | 36.0 |
|  | 4.0 | HU10004 | 20.70 | 9,662 | 82.80 | 8.50 | 12.50 | 5.10 | 8.00 | 4.30 | 3.50 | 1.50 | 43.0 |
|  | 6.0 | HU10006 | 20.70 | 9,662 | 124.40 | 10.50 | 16.50 | 5.10 | 8.00 | 4.30 | 3.50 | 1.50 | 51.0 |
|  | 10.0 | HU10010 | 20.70 | 9,662 | 207.00 | 14.50 | 24.50 | 5.10 | 8.00 | 4.30 | 3.50 | 1.50 | 68.0 |
| 150 | 2.00 | HU15002 | 30.70 | 9662 | 61.40 | 7.73 | 9.73 | 6.26 | 10.00 | 5.00 | 4.50 | 1.50 | 55.0 |
|  | 4.00 | HU15004 | 30.70 | 9662 | 122.70 | 9.73 | 13.73 | 6.26 | 10.00 | 5.00 | 4.50 | 1.50 | 65.0 |
|  | 6.00 | HU15006 | 30.70 | 9662 | 184.10 | 11.73 | 17.73 | 6.26 | 10.00 | 5.00 | 4.50 | 1.50 | 81.0 |
|  | 10.00 | HU15010 | 30.70 | 9662 | 306.80 | 15.73 | 25.73 | 6.26 | 10.00 | 5.00 | 4.50 | 1.50 | 110.0 |

### Double Acting Cylinders

HD Series - General Purpose

HD20013	HD3008

Capacity:

10 - 500 Tons

Stroke:

5.90 - 24.00 Inches

Pressure:

10,000 psi

Min. - Max. Height:

10.56 - 60.13 Inches

- Rod wiper seals out dirt and contamination

- High strength steel construction from gun drilled solid bar

- High flow 3/8″ NPTF female couplers

- One piece high grade alloy bearing for side load protec- tion and long life

- Ideal for pushing and pulling

- Maximum working pressure: 10,000 psi / 700 bar

- Threaded cylinders, removable saddle, thicker cylinder walls

- Retract under hydraulic power for faster operation

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Clevis and Tangs

Install a clevis or a tang to your double acting

cylinder for a pulling application.

pg34

HD cylinders can be used in pushing and pulling applications. In this photo, we see a HD2506 HD cylinder set up for

a pull application using a Rod Clevis cylinder accessory

1-888-332-6419

12	www.BVAhydraulics.com

HD Series

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Cylinder Accessories

See the assortment

of accessories designed to work with our products.

pg

34

Combo Kits

Most cylinders are available in easy to order kits: Cylinder, Gauge, Couplers, Hose

and Pump.

pg64

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

HUD Series – Aluminum Cylinders

Aluminum cylinders make a great lightweight alternative being up to 60% lighter than comparable

steel models.

pg

16

13

Cylinders

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

60.13”

10.56”

With a variety of tonnage and sizes, you can apply our cylinders to a large variety of applications. Custom tonnages and stroke lengths are available upon request.

HD20024	HD2506

#### Quick Selection Guide

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Collapsed Height (in) | Oil Capacity |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Push (in3) | Pull (in3) |  |
| 10 | 10.04 | HD1010 | 16.12 | 22.41 | 7.47 | 26.7 |
|  | 11.97 | HD1012 | 18.03 | 26.71 | 8.90 | 30.0 |
| 25 | 5.90 | HD2506 | 10.56 | 30.35 | 15.81 | 24.8 |
| 30 | 8.15 | HD3008 | 15.31 | 52.93 | 26.02 | 43.0 |
|  | 14.49 | HD3014 | 21.65 | 94.09 | 46.26 | 60.6 |
| 55 | 6.18 | HD5506‡ | 13.03 | 67.93 | 19.74 | 70.5 |
|  | 13.19 | HD5513‡ | 20.04 | 144.94 | 42.11 | 104.0 |
|  | 20.12 | HD5520‡ | 28.86 | 221.09 | 64.24 | 146.2 |
| 75 | 6.10 | HD7506‡ | 13.70 | 98.10 | 31.21 | 93.7 |
|  | 13.11 | HD7513‡ | 20.71 | 210.93 | 67.06 | 136.8 |
| 100 | 6.57 | HD10006‡ | 14.06 | 135.23 | 62.98 | 138.6 |
|  | 10.13 | HD10010‡ | 17.67 | 208.30 | 97.10 | 169.2 |
|  | 13.11 | HD10013‡ | 20.63 | 269.66 | 125.58 | 193.6 |
|  | 18.15 | HD10018‡ | 27.05 | 373.31 | 173.86 | 248.3 |
| 200 | 6.00 | HD20006 | 16.94 | 265.28 | 135.00 | 325.0 |
|  | 13.00 | HD20013 | 23.94 | 574.78 | 293.00 | 440.0 |
|  | 24.02 | HD20024 | 36.13 | 1061.00 | 528.00 | 616.0 |
| 300 | 6.00 | HD30006 | 19.13 | 425.56 | 228.00 | 441.0 |
| 500 | 6.00 | HD50006 | 22.75 | 678.92 | 378.00 | 953.0 |

See pages (pg.14-15) for detailed specifications.

### Double Acting Cylinders

HD Series - General Purpose

Cylinder Accessories

See the assortment

of accessories designed

to work with our products. pg34

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Combo Kits

Most cylinders are available in easy to order kits: Cylinder,

HD20013

HD7513

Gauge, Couplers, Hose and Pump.

pg64

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg92

HD10018

HD3008

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Maximum Cylinder Capacity |  | Cylinder Effective Area |  | Oil Capacity |  | Weight (lbs) | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Push (ton) | Pull (ton) | Push (in2) | Pull (in2) | Push (in3) | Pull (in3) |  |  |  |  |
| 10 | 10.04 | HD1010 | 11.40 | 3.80 | 2.24 | 0.74 | 22.41 | 7.47 | 26.7 | 16.12 | 26.19 | 1.69 |
|  | 11.97 | HD1012 | 11.40 | 3.80 | 2.24 | 0.74 | 26.71 | 8.90 | 30.0 | 18.03 | 30.00 | 1.69 |
| 25 | 5.90 | HD2506 | 25.70 | 13.39 | 5.14 | 2.68 | 30.35 | 15.81 | 24.8 | 10.56 | 16.46 | 2.56 |
| 30 | 8.15 | HD3008 | 32.90 | 16.20 | 6.49 | 3.19 | 52.93 | 26.02 | 43.0 | 15.31 | 23.46 | 2.87 |
|  | 14.49 | HD3014 | 32.90 | 16.20 | 6.49 | 3.19 | 94.09 | 46.26 | 60.6 | 21.65 | 36.14 | 2.87 |
| 55 | 6.18 | HD5506* | 55.70 | 16.20 | 10.99 | 3.19 | 67.93 | 19.74 | 70.5 | 13.03 | 19.21 | 3.74 |
|  | 13.19 | HD5513* | 55.70 | 16.20 | 10.99 | 3.19 | 144.94 | 42.11 | 104.0 | 20.04 | 33.23 | 3.74 |
|  | 20.12 | HD5520* | 55.70 | 16.20 | 10.99 | 3.19 | 221.09 | 64.24 | 146.2 | 28.86 | 49.98 | 3.74 |
| 75 | 6.10 | HD7506* | 81.70 | 25.90 | 16.09 | 5.12 | 98.10 | 31.21 | 93.7 | 13.70 | 19.80 | 4.53 |
|  | 13.11 | HD7513* | 81.70 | 25.90 | 16.09 | 5.12 | 210.93 | 67.06 | 136.8 | 20.71 | 33.82 | 4.53 |
| 100 | 6.57 | HD10006* | 104.40 | 48.60 | 20.57 | 9.58 | 135.23 | 62.98 | 138.6 | 14.06 | 20.63 | 5.12 |
|  | 10.13 | HD10010* | 104.40 | 48.60 | 20.57 | 9.58 | 208.30 | 97.10 | 169.2 | 17.67 | 27.80 | 5.12 |
|  | 13.11 | HD10013* | 104.40 | 48.60 | 20.57 | 9.58 | 269.66 | 125.58 | 193.6 | 20.63 | 33.74 | 5.12 |
|  | 18.15 | HD10018* | 104.40 | 48.60 | 20.57 | 9.58 | 373.31 | 173.86 | 248.3 | 27.05 | 45.20 | 5.12 |
| 200 | 6.57 | HD20006 | 221.00 | 112.50 | 44.21 | 22.50 | 265.28 | 135.00 | 325.0 | 16.94 | 22.94 | 7.50 |
|  | 13.11 | HD20013 | 221.00 | 112.50 | 44.21 | 22.50 | 574.78 | 293.00 | 440.0 | 23.94 | 36.94 | 7.50 |
|  | 24.00 | HD20024 | 221.00 | 112.50 | 44.21 | 22.50 | 1061.00 | 528.00 | 616.0 | 36.13 | 60.13 | 7.50 |
| 300 | 6.57 | HD30006 | 354.60 | 190.00 | 70.93 | 38.00 | 425.56 | 228.00 | 441.0 | 19.13 | 25.13 | 9.50 |
| 500 | 6.57 | HD50006 | 565.70 | 315.00 | 113.15 | 63.00 | 678.92 | 378.00 | 953.0 | 22.75 | 28.75 | 12.00 |

1-888-332-6419

14	www.BVAhydraulics.com

HD Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

15

Cylinders

Y

X

H		 	G

3/8"-18NPTF

F	E	C	D

J

K

M	L	 	Z

A	 B

| Model Number | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) | Base to Adv. Port G (in) | Top to Return Port H (in) | Rod Internal Thread J (in) | Rod Thread Length K (in) | Collar Thread L (in) | Collar Thread Length M (in) | Base Mounting Hole |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Bolt Circle X (in) | Thread Y (in) | Thread Depth Z (in) |
| HD1010 | 2.87 | 1.38 | 1.38 | 1.42 | 2.09 | 1”-8 | 0.98 | 2¼”-14 | 1.02 | 1.30 | ”-18 | 0.60 |
| HD1012 | 2.87 | 1.37 | 1.38 | 1.42 | 2.09 | 1”-8 | 0.98 | 2¼”-14 | 1.02 | 1.30 | ”-18 | 0.60 |
| HD2506 | 3.62 | 1.77 | 1.50 | 0.98 | 1.93 | 1’-8 | 1.18 | 3”-12 | 0.98 | 2.30 | ½”-13 | 0.70 |
| HD3008 | 3.98 | 2.13 | 1.97 | 1.65 | 2.91 | 1½”-16 | 0.98 | 3”-12 | 1.93 | 2.20 | ½”-13 | 0.70 |
| HD3014 | 3.98 | 2.13 | 1.97 | 1.65 | 2.91 | 1½”-16 | 0.98 | 3”-12 | 1.93 | 2.20 | ½”-13 | 0.70 |
| HD5506* | 5.00 | 3.14 | 2.76 | 1.14 | 2.87 | 1”-12 | 0.98 | 5”-12 | 1.73 | 3.70 | ½”-13 | 0.70 |
| HD5513* | 5.00 | 3.14 | 2.76 | 1.14 | 2.87 | 1”-12 | 0.98 | 5”-12 | 1.73 | 3.70 | ½”-13 | 0.70 |
| HD5520* | 5.00 | 3.14 | 2.76 | 1.14 | 2.87 | 1”-12 | 0.98 | 5”-12 | 1.73 | 3.70 | ½”-13 | 0.70 |
| HD7506* | 5.75 | 3.75 | 2.76 | 1.18 | 2.80 | 1”-12 | 1.50 | 5¾”-12 | 1.50 | 4.00 | ¾”-10 | 0.90 |
| HD7513* | 5.75 | 3.75 | 2.76 | 1.18 | 2.80 | 1”-12 | 1.50 | 5¾”-12 | 1.50 | 4.00 | ¾”-10 | 0.90 |
| HD10006* | 6.97 | 3.75 | 2.95 | 1.22 | 2.95 | 1¾”-12 | 1.38 | 6⅞”-12 | 1.97 | 5.47 | ¾”-10 | 0.98 |
| HD10010* | 6.97 | 3.74 | 2.95 | 1.22 | 2.95 | 1¾”-12 | 1.50 | 6⅞”-12 | 2.01 | 5.50 | ¾”-10 | 0.98 |
| HD10013* | 6.97 | 3.75 | 2.95 | 1.22 | 2.95 | 1¾”-12 | 1.38 | 6⅞”-12 | 1.97 | 5.47 | ¾”-10 | 0.98 |
| HD10018* | 6.97 | 3.75 | 2.95 | 1.50 | 2.95 | 1¾”-12 | 1.38 | 6⅞”-12 | 1.97 | 5.47 | ¾”-10 | 0.98 |
| HD20006 | 9.75 | 5.25 | 5.25 | 2.25 | 3.81 | - | - | - | - | 5.00 | 1-8 | 1.00 |
| HD20013 | 9.75 | 5.25 | 5.25 | 2.25 | 3.81 | 2½”-12 | 2.50 | 9¾”-12 | 2.13 | 5.00 | 1-8 | 1.00 |
| HD20024 | 9.75 | 5.25 | 5.25 | 3.38 | 4.00 | 2½”-12 | 2.50 | 9¾”-12 | 2.13 | 5.00 | 1-8 | 1.00 |
| HD30006 | 12.25 | 6.50 | 6.50 | 3.50 | 4.50 | 2½”-12 | 3.25 | 12¼”-12 | 2.31 | 6.25 | 1¼-7 | 1.75 |
| HD50006 | 15.63 | 8.00 | 8.00 | 4.75 | 6.00 | 3¼”-12 | 4.25 | 15”-8 | 3.13 | 8.00 | 1¾-5 | 2.12 |

* 55-100 ton cylinders are equipped with carrying handles.

### Aluminum Double Acting Cylinders

HUD Series - General Purpose, Lightweight

HUD7513

HUD15006

Capacity:

30 - 150 Tons

Stroke:

2 - 13 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

8.30 - 33.80 Inches

Aluminum vs. Steel Cylinders

Aluminum cylinders provide a great alternative to the traditional steel cylinder in multiple applications. Being up to 60% lighter in weight, Aluminum cylinders are highly portable and reduce user fatigue and strain. However, due to the finite properties of Aluminum vs. Steel, these cylinders should NOT be used in high cycle applications. The BVA Aluminum cylinders are designed for a maximum safety rating of 5,000 cycles. Under normal lifting applications this should provide a long service life.

HUD7513

HD7513

Aluminum Cylinders	Steel Cylinders

- 60% lighter in weight than steel	•	Much heavier than aluminum cylinders

- Highly portable and reduces user fatigue	•	Low portability with heavier cylinders which

- Not used for high cycle applications	creates user fatigue and strain

- Under normal conditions, the maximum	•	Will out-perform aluminum cylinders in rated safety cycle life is 5,000 cycles		higher cycle applications

- Base of the cylinder contains a steel plate*	•	Can exceed aluminums maximum safety

cycle life of 5,000 cycles.

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Maximum Cylinder Capacity (ton) |  | Cylinder Effective Area (in2) |  | Oil Capacity (in3) |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Push | Pull | Push | Pull | Push | Pull |  |
| 30 | 6.00 | HUD3006 | 30.00 | 12.57 | 6.50 | 2.50 | 39.00 | 15.00 | 18.0 |
|  | 8.00 | HUD3008 | 30.00 | 12.57 | 6.50 | 2.50 | 51.90 | 18.00 | 26.0 |
|  | 13.00 | HUD3013 | 30.00 | 12.57 | 6.00 | 2.50 | 90.90 | 32.41 | 35.0 |
| 50 | 6.00 | HUD5006 | 50.00 | 13.50 | 10.30 | 2.70 | 61.80 | 16.00 | 28.0 |
|  | 8.00 | HUD5008 | 50.00 | 13.50 | 10.30 | 2.70 | 88.30 | 22.00 | 36.0 |
|  | 13.00 | HUD5013 | 50.00 | 13.58 | 10.30 | 2.70 | 143.50 | 35.80 | 48.0 |
| 75 | 6.00 | HUD7506 | 75.00 | 27.00 | 15.00 | 5.41 | 90.20 | 19.40 | 41.0 |
|  | 8.00 | HUD7508 | 75.00 | 27.00 | 15.00 | 5.41 | 121.00 | 43.30 | 50.0 |
|  | 13.00 | HUD7513 | 75.00 | 27.00 | 15.00 | 5.41 | 195.40 | 70.40 | 62.0 |
| 100 | 2.00 | HUD10002 | 100.00 | 23.75 | 20.60 | 4.75 | 41.20 | 9.40 | 42.0 |
|  | 6.00 | HUD10006 | 100.00 | 23.75 | 20.60 | 4.75 | 123.70 | 28.30 | 57.0 |
|  | 13.00 | HUD10013 | 100.00 | 23.75 | 20.60 | 4.75 | 268.10 | 61.40 | 90.0 |
| 150 | 6.00 | HUD15006 | 100.00 | 34.60 | 30.70 | 6.90 | 184.10 | 41.40 | 87.0 |
|  | 13.00 | HUD15013 | 100.00 | 34.60 | 30.70 | 6.90 | 378.80 | 90.10 | 122.0 |

1-888-332-6419

16	www.BVAhydraulics.com

- The base of the cylinders contain a Steel plate with mounting holes and is designed to protect the cylinder from damage and should NOT be used in fixturing applications and ONLY be used to attach a larger base mounting plate for lifting stability. They are NOT designed to withstand the full rating of the cylinder.

F

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

Cylinders

HUD Series

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation.

D	Always be prepared.

G

A

H

pg92

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

E	C

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

  - Lightweight aluminum alloy construction

  - Up to 60% lighter than comparable tonnage steel cylinders

  - Convenient carry handle on 50, 75, 100 and 150 ton models

  - Hard anodized aluminum rod provides extra protection

against corrosion and wear

  - Steel protective plate on cylinder base

  - Maximum working pressure: 10,000 psi / 700 bar

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Diameter D (in) | Rod Dia. E (in) | Saddle Diameter F (in) | Base To Advance Port G (in) | Top To Retract Port H (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUD3006 | 11.80 | 17.80 | 2.88 | 4.50 | 2.88 | 2.00 | 1.50 | 2.00 |
| HUD3008 | 13.80 | 21.80 | 2.88 | 4.50 | 2.88 | 2.00 | 1.50 | 2.00 |
| HUD3013 | 19.30 | 32.30 | 2.88 | 4.50 | 2.88 | 2.00 | 1.50 | 2.00 |
| HUD5006 | 11.80 | 17.80 | 3.63 | 5.50 | 3.13 | 2.50 | 1.50 | 2.50 |
| HUD5008 | 13.80 | 21.80 | 3.63 | 5.50 | 3.13 | 2.50 | 1.50 | 2.50 |
| HUD5013 | 19.30 | 32.30 | 3.63 | 5.50 | 3.13 | 2.50 | 1.50 | 2.50 |
| HUD7506 | 12.30 | 18.30 | 4.38 | 6.50 | 3.50 | 3.00 | 1.50 | 2.50 |
| HUD7508 | 14.30 | 20.30 | 4.38 | 6.50 | 3.50 | 3.00 | 1.50 | 2.50 |
| HUD7513 | 19.30 | 32.30 | 4.38 | 6.50 | 3.50 | 3.00 | 1.50 | 2.50 |
| HUD10002 | 8.30 | 10.30 | 5.13 | 8.00 | 4.50 | 3.50 | 1.50 | 2.50 |
| HUD10006 | 12.30 | 18.30 | 5.13 | 8.00 | 4.50 | 3.50 | 1.50 | 2.50 |
| HUD10013 | 19.30 | 32.30 | 5.13 | 8.00 | 4.50 | 3.50 | 1.50 | 2.50 |
| HUD15006 | 14.30 | 20.30 | 6.26 | 10.00 | 5.50 | 4.50 | 2.00 | 3.00 |
| HUD15013 | 20.80 | 33.80 | 6.26 | 10.00 | 5.50 | 4.50 | 2.00 | 3.00 |

NOTE: The base of the aluminum cylinders contain a Steel plate with mounting holes and is designed to protect the

cylinder from damage and should NOT be used in fixturing applications and ONLY be used to attach larger

base mounting plates for lifting stability. The threads are NOT designed to withstand the full rating of the cylinder.	17

### High Tonnage Single Acting Cylinders

HG series - Heavy Lifting, Load Return

Capacity:

100-1,500 Tons

Stroke:

1.97 - 11.81 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

7.17 - 43.86 Inches

HG10004

HG15006

HG20002

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

- Cylinders are burnish rolled creating a smoother surface than honing.

- Rod wiper seals out dirt and contamination

- Base mounting holes on all models

and materials.

pg 72

- High strength steel construction from gun drilled solid bar pro- viding extra combine strength for high tonnage applications

- High flow 3/8” NPTF coupler

Flow Control Valves

Control the speed of your

cylinder by adding a flow

  - Polyurethane cup seal provides optimum performance

  - Industrial hard chrome plated rod helps prevent rust and corrosion

control valve.

pg 66

- Maximum working pressure: 10,000 psi / 700 bar

- Custom sizes are available upon request

- Load Return

Gas/Diesel Engine Pumps	Electric Motor Pumps

These HG cylinders pair up well with a Gas or Diesel Engine pumps.

We offer a variety of different flow rates, motor sizes and reservoir sizes.

Contact your local BVA dealer for more details.

These HG cylinders pair up well with Electric Motor pumps.

We offer a variety of different flow rates and motor sizes.

Contact your local BVA dealer for more details.

PG70M3N05

  - Will provide 10,000 psi. hydraulic force at remote locations

  - Industrial grade 5.5 or 10.2 hp Honda gas engine or 7.3 hp HATZ diesel engine.

  - Ideal for applications where electricity

isn’t easily accessible.

  - Two speed pump designed for rapid cylinder advance

  - Gas pump features a sturdy protective roll cage for use in the toughest environments

  - Electric start standard on PD80 and optional on PG90

    - Meets OSHA regulations for

sound (80dBA)

    - Two speed pump designed for rapid cylinder advance

    - Internal filter screens contamination

to prevent pump damage

    - All units are wired as 120 VAC 60 Hz; Optional wiring includes 50 Hz motors and 240VAC 60Hz

    - 100% Duty Cycle TEFC motors

    - All hand pendants are wired at 24VAC for safety

    - Full load starting capability

PE50W3N03A

1-888-332-6419

18	www.BVAhydraulics.com

HG Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg 69

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

Optional Tilt-Saddles

Saddles tilt to a 5% maximum to adjust to the load. View our cylinder accessories page to figure out what saddle best

fits these cylinders.	pg 35

19

Cylinders

Quick Reference Guide

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Collapsed Height (in) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 100 | 1.97 | HG10002 | 7.17 | 40.50 | 61.0 |
|  | 3.94 | HG10004 | 9.13 | 81.00 | 74.0 |
|  | 5.91 | HG10006 | 11.10 | 121.50 | 87.0 |
|  | 7.87 | HG10008 | 13.07 | 161.99 | 100.0 |
|  | 9.84 | HG10010 | 15.04 | 202.49 | 114.0 |
|  | 11.81 | HG10012 | 17.01 | 242.99 | 127.0 |
| 150 | 1.97 | HG15002 | 7.72 | 59.89 | 102.0 |
|  | 3.94 | HG15004 | 9.69 | 119.78 | 122.0 |
|  | 5.91 | HG15006 | 11.65 | 181.75 | 142.0 |
|  | 7.87 | HG15008 | 13.62 | 239.57 | 162.0 |
|  | 9.84 | HG15010 | 15.59 | 299.46 | 181.0 |
|  | 11.81 | HG15012 | 17.56 | 359.35 | 202.0 |
| 200 | 1.97 | HG20002 | 8.50 | 80.68 | 121.0 |
|  | 5.91 | HG20006 | 12.44 | 243.40 | 201.0 |
|  | 7.87 | HG20008 | 14.41 | 322.73 | 242.0 |
|  | 11.81 | HG20012 | 18.35 | 486.79 | 322.0 |
| 300 | 1.97 | HG30002 | 12.28 | 139.19 | 406.0 |
|  | 5.91 | HG30006 | 16.22 | 417.56 | 511.0 |
|  | 11.81 | HG30012 | 22.13 | 835.11 | 668.0 |
| 400 | 1.97 | HG40002 | 14.74 | 170.84 | 595.0 |
|  | 5.91 | HG40006 | 18.68 | 512.51 | 728.0 |
|  | 12.01 | HG40012 | 24.61 | 1042.10 | 928.0 |
| 500 | 1.97 | HG50002 | 16.50 | 222.92 | 884.0 |
|  | 5.91 | HG50006 | 20.43 | 668.77 | 1058.0 |
|  | 11.81 | HG50012 | 26.34 | 1338.00 | 1321.0 |
| 600 | 1.97 | HG60002 | 16.89 | 260.97 | 1045.0 |
|  | 5.91 | HG60006 | 20.83 | 782.90 | 1246.0 |
|  | 9.84 | HG60010 | 19.88 | 1408.93 | 948.0 |
|  | 11.81 | HG60012 | 26.73 | 1566.00 | 1545.0 |
| 800 | 1.97 | HG80002 | 18.66 | 358.91 | 1634.0 |
|  | 5.91 | HG80006 | 22.60 | 1077.00 | 1914.0 |
|  | 11.81 | HG80012 | 28.50 | 2153.00 | 2332.0 |
| 1000 | 1.97 | HG100002 | 22.20 | 447.23 | 2341.0 |
|  | 5.91 | HG100006 | 26.14 | 1342.00 | 2674.0 |
|  | 11.81 | HG100012 | 32.05 | 2683.00 | 3172.0 |
| 1100-1500 | For these higher tonnages, contact your local BVA dealer for pricing and details |  |  |  |  |

See next pages (pg.20-21) to view more detail stats on High Tonnage Single Acting Cylinders.

### High Tonnage Single Acting Cylinders

HG Series - Heavy Lifting, Load Return

E F

B

3/8"-18NPTF

A

C	HG100

HG150	HG200

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Maximum Cylinder Capacity (ton) | Cylinder Effective Area (in2) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | 1.97 | HG10002 | 102.86 | 20.57 | 40.50 | 61.0 |
|  | 3.94 | HG10004 | 102.86 | 20.57 | 81.00 | 74.0 |
|  | 5.91 | HG10006 | 102.86 | 20.57 | 121.50 | 87.0 |
|  | 7.87 | HG10008 | 102.86 | 20.57 | 161.99 | 100.0 |
|  | 9.84 | HG10010 | 102.86 | 20.57 | 202.49 | 114.0 |
|  | 11.81 | HG10012 | 102.86 | 20.57 | 242.99 | 127.0 |
| 150 | 1.97 | HG15002 | 152.11 | 30.43 | 59.89 | 102.0 |
|  | 3.94 | HG15004 | 152.11 | 30.43 | 119.78 | 122.0 |
|  | 5.91 | HG15006 | 153.90 | 30.78 | 181.75 | 142.0 |
|  | 7.87 | HG15008 | 152.11 | 30.43 | 239.57 | 162.0 |
|  | 9.84 | HG15010 | 152.11 | 30.43 | 299.46 | 181.0 |
|  | 11.81 | HG15012 | 152.11 | 30.43 | 359.35 | 202.0 |
| 200 | 1.97 | HG20002 | 204.92 | 40.99 | 80.68 | 121.0 |
|  | 5.91 | HG20006 | 206.10 | 41.22 | 243.40 | 201.0 |
|  | 7.87 | HG20008 | 204.92 | 40.99 | 322.73 | 242.0 |
|  | 11.81 | HG20012 | 206.10 | 41.22 | 486.79 | 322.0 |
| 300 | 1.97 | HG30002 | 353.60 | 70.71 | 139.19 | 406.0 |
|  | 5.91 | HG30006 | 353.60 | 70.71 | 417.56 | 511.0 |
|  | 11.81 | HG30012 | 353.60 | 70.71 | 835.11 | 668.0 |
| 400 | 1.97 | HG40002 | 433.90 | 86.78 | 170.84 | 595.0 |
|  | 5.91 | HG40006 | 433.90 | 86.78 | 512.51 | 728.0 |
|  | 12.01 | HG40012 | 433.90 | 86.78 | 1042.10 | 928.0 |
| 500 | 1.97 | HG50002 | 566.30 | 113.25 | 222.93 | 884.0 |
|  | 5.91 | HG50006 | 566.30 | 113.25 | 668.78 | 1058.0 |
|  | 11.81 | HG50012 | 566.30 | 113.25 | 1337.56 | 1321.0 |
| 600 | 1.97 | HG60002 | 662.90 | 132.57 | 260.97 | 1045.0 |
|  | 5.91 | HG60006 | 662.90 | 132.57 | 782.90 | 1246.0 |
|  | 9.84 | HG60010 | 670.90 | 134.18 | 1408.93 | 948.0 |
|  | 11.81 | HG60012 | 662.90 | 132.57 | 1566.00 | 1545.0 |
| 800 | 1.97 | HG80002 | 911.60 | 182.32 | 358.91 | 1634.0 |
|  | 5.91 | HG80006 | 911.60 | 182.32 | 1077.00 | 1914.0 |
|  | 11.81 | HG80012 | 911.60 | 182.32 | 2153.00 | 2332.0 |
| 1000 | 1.97 | HG100002 | 1136.00 | 227.19 | 447.23 | 2341.0 |
|  | 5.91 | HG100006 | 1136.00 | 227.19 | 1342.00 | 2674.0 |
|  | 11.81 | HG100012 | 1136.00 | 227.19 | 2683.00 | 3172.0 |

G

Z

Y

X

D

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg 92

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Optional Tilt-Saddles

Saddles tilt to a 5% maximum to adjust to the load. View our cylinder accessories page to figure out what saddle best

fits these cylinders.

pg 35

1-888-332-6419

20	www.BVAhydraulics.com

HG Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

21

Cylinders

HG300 - HG1000

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) | Base To Adv. Port G (in) | Base Mounting Holes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Bolt Circle (in) X | Thread Size (mm) | Thread Depth (in) Z |
| HG10002 | 7.17 | 9.14 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG10004 | 9.13 | 13.07 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG10006 | 11.10 | 17.01 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG10008 | 13.07 | 20.94 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG10010 | 15.04 | 24.88 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG10012 | 17.01 | 28.82 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 3.74 | M12 | 0.87 |
| HG15002 | 7.72 | 9.69 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG15004 | 9.69 | 13.62 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG15006 | 11.65 | 17.56 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG15008 | 13.62 | 21.50 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG15010 | 15.59 | 25.43 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG15012 | 17.56 | 29.37 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 5.12 | M12 | 0.87 |
| HG20002 | 8.50 | 10.47 | 7.22 | 9.25 | 5.31 | 4.45 | 2.62 | 6.50 | M12 | 0.80 |
| HG20006 | 12.44 | 18.35 | 7.24 | 9.25 | 5.31 | 4.45 | 2.62 | 6.50 | M12 | 0.80 |
| HG20008 | 14.41 | 22.28 | 7.22 | 9.25 | 5.31 | 4.45 | 2.62 | 6.50 | M12 | 0.80 |
| HG20012 | 18.35 | 30.16 | 7.24 | 9.25 | 5.31 | 4.45 | 2.62 | 6.50 | M12 | 0.80 |
| HG30002 | 12.28 | 14.25 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 7.09 | M16 | 1.42 |
| HG30006 | 16.22 | 22.13 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 7.09 | M16 | 1.42 |
| HG30012 | 22.13 | 33.94 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 7.09 | M16 | 1.42 |
| HG40002 | 14.74 | 16.71 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 8.07 | M16 | 1.42 |
| HG40006 | 18.68 | 24.59 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 8.07 | M16 | 1.42 |
| HG40012 | 24.61 | 36.61 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 8.07 | M16 | 1.42 |
| HG50002 | 16.50 | 18.46 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 9.84 | M24 | 1.50 |
| HG50006 | 20.43 | 26.34 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 9.84 | M24 | 1.50 |
| HG50012 | 26.34 | 38.15 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 9.84 | M24 | 1.50 |
| HG60002 | 16.89 | 18.86 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 10.83 | M24 | 1.50 |
| HG60006 | 20.83 | 26.73 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 10.83 | M24 | 1.50 |
| HG60010 | 18.00 | 28.50 | 13.07 | 17.05 | 9.00 | 7.87 | 4.49 | 10.83 | M24 | 1.50 |
| HG60012 | 26.73 | 38.54 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 10.83 | M24 | 1.50 |
| HG80002 | 18.66 | 20.63 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 12.99 | M24 | 1.50 |
| HG80006 | 22.60 | 28.50 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 12.99 | M24 | 1.50 |
| HG80012 | 28.50 | 40.31 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 12.99 | M24 | 1.50 |
| HG100002 | 22.20 | 24.17 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 14.76 | M24 | 1.50 |
| HG100006 | 26.14 | 32.05 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 14.76 | M24 | 1.50 |
| HG100012 | 32.05 | 43.86 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 14.76 | M24 | 1.50 |

### High Tonnage Double Acting Cylinders

HDG Series - Heavy Lifting

HDG10002

HDG30006

HDG10008

HDG20008

HDG15006

Capacity:

100-1,500 Tons

Stroke:

1.97 - 11.81 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

7.17 - 43.80 Inches

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg 92

- Cylinders are burnish rolled creating a smoother

finish than honing

- Industrial hard chrome plated rod prevents

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

scratching and corrosion

- Base mounting holes standard on all models

- Rod wiper seals out dirt and contamination

and materials.

pg 72

- High strength steel construction from gun drilled solid bar

- High flow 3/8″ NPTF couplers

Flow Control Valves

Control the speed of your

cylinder by adding a flow

  - Maximum working pressure: 10,000 psi / 700 bar

  - Custom sizes are available upon request

control valve.

pg 66

Gas/Diesel Engine Pumps	Electric Motor Pumps

These HDG cylinders pair up well with a Gas or Diesel Engine pumps. We offer a variety of different flow rates, motor sizes and reservoir sizes on our gas/diesel engine pumps. Contact your local BVA dealer for more details.

These HDG cylinders pair up well with Electric Motor pumps. We offer a

variety of different flow rates and motor sizes on our electric motor pumps.

Contact your local BVA dealer for more details.

PG70M4N05

    - Gas engine pump will provide 10,000 psi. hydraulic force at remote locations

    - Industrial grade 5.5 or 10.2 hp Honda gas engine or 7.3 hp HATZ diesel engine.

    - Ideal for applications where electricity

isn’t easily accessible.

    - Two speed pump designed for rapid cylinder advance

    - Gas pump features a sturdy protective roll cage for use in the toughest environments

    - Optional electric start

      - Meets OSHA regulations for

sound (80dBA)

      - Two speed pump designed for rapid cylinder advance

      - Internal filter screens contamination

to prevent pump damage

      - All units are wired as 120 VAC 60 Hz; Optional wiring includes 50 Hz motors and 240VAC 60Hz

      - 100% Duty Cycle TEFC motors

      - All hand pendants are wired at 24VAC for safety

      - Full load starting capability

PE50W4N03A

1-888-332-6419

22	www.BVAhydraulics.com

HDG Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg 69

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

Safety Check: CVP3-K

Pilot operated check valve kit. Used to hold the load in the event of a pressure loss in the system.

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

Optional Tilt-Saddles

Saddles tilt to a 5% maximum to adjust to the load. View our cylinder accessories page to figure out what saddle best

fits these cylinders.	pg 35

23

Cylinders

Quick Reference Guide

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Collapsed Height (in) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 100 | 1.97 | HDG10002 | 7.17 | 40.50 | 61.0 |
|  | 3.94 | HDG10004 | 9.13 | 81.00 | 74.0 |
|  | 5.91 | HDG10006 | 11.10 | 121.50 | 87.0 |
|  | 7.87 | HDG10008 | 13.07 | 161.99 | 100.0 |
|  | 9.84 | HDG10010 | 15.04 | 202.49 | 114.0 |
|  | 11.81 | HDG10012 | 17.01 | 242.99 | 127.0 |
| 150 | 1.97 | HDG15002 | 7.72 | 59.89 | 102.0 |
|  | 3.94 | HDG15004 | 9.69 | 119.78 | 122.0 |
|  | 5.91 | HDG15006 | 11.65 | 181.75 | 142.0 |
|  | 7.87 | HDG15008 | 13.62 | 239.57 | 162.0 |
|  | 9.84 | HDG15010 | 15.59 | 299.46 | 181.0 |
|  | 11.81 | HDG15012 | 17.56 | 359.35 | 202.0 |
| 200 | 1.97 | HDG20002 | 8.50 | 80.68 | 121.0 |
|  | 5.91 | HDG20006 | 12.44 | 243.40 | 201.0 |
|  | 7.87 | HDG20008 | 14.41 | 322.73 | 242.0 |
|  | 11.81 | HDG20012 | 18.35 | 486.79 | 322.0 |
| 300 | 1.97 | HDG30002 | 12.28 | 139.19 | 406.0 |
|  | 5.91 | HDG30006 | 16.22 | 417.56 | 511.0 |
|  | 11.81 | HDG30012 | 22.13 | 835.11 | 668.0 |
| 400 | 1.97 | HDG40002 | 14.74 | 170.84 | 595.0 |
|  | 5.91 | HDG40006 | 18.68 | 512.51 | 728.0 |
|  | 12.01 | HDG40012 | 24.61 | 1042.10 | 928.0 |
| 500 | 1.97 | HDG50002 | 16.50 | 222.92 | 884.0 |
|  | 5.91 | HDG50006 | 20.43 | 668.77 | 1058.0 |
|  | 11.81 | HDG50012 | 26.34 | 1337.56 | 1321.0 |
| 600 | 1.97 | HDG60002 | 16.89 | 260.97 | 1045.0 |
|  | 5.91 | HDG60006 | 20.83 | 782.90 | 1246.0 |
|  | 9.84 | HDG60010 | 19.88 | 1408.93 | 948.0 |
|  | 11.81 | HDG60012 | 26.73 | 1566.00 | 1545.0 |
| 800 | 1.97 | HDG80002 | 18.66 | 358.91 | 1634.0 |
|  | 5.91 | HDG80006 | 22.60 | 1077.00 | 1914.0 |
|  | 11.81 | HDG80012 | 28.50 | 2153.00 | 2332.0 |
| 1000 | 1.97 | HDG100002 | 22.20 | 447.23 | 2341.0 |
|  | 5.91 | HDG100006 | 26.14 | 1342.00 | 2674.0 |
|  | 11.81 | HDG100012 | 32.05 | 2683.00 | 3172.0 |
| 1100-1500 | For these higher tonnages, contact your local BVA dealer for pricing and details |  |  |  |  |

See next pages (pg.20-21) to view more detail stats on High Tonnage Single Acting Cylinders.

### High Tonnage Double Acting Cylinders

HDG Series - Heavy Lifting

E F

H

B

3/8"-18NPTF

G

A

C	HDG100

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Maximum Cylinder Capacity (ton) |  | Cylinder Effective Area (in2) |  | Oil Capacity (in3) |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Push | Pull | Push | Pull | Push | Pull |  |
| 100 | 1.97 | HDG10002 | 102.86 | 47.93 | 20.57 | 9.59 | 40.50 | 18.87 | 61.0 |
|  | 3.94 | HDG10004 | 102.86 | 47.93 | 20.57 | 9.59 | 81.00 | 37.74 | 74.0 |
|  | 5.91 | HDG10006 | 102.86 | 47.93 | 20.57 | 9.59 | 121.50 | 56.61 | 87.0 |
|  | 7.87 | HDG10008 | 102.86 | 47.93 | 20.57 | 9.59 | 161.99 | 75.49 | 100.0 |
|  | 9.84 | HDG10010 | 102.86 | 47.93 | 20.57 | 9.59 | 202.49 | 94.36 | 114.0 |
|  | 11.81 | HDG10012 | 102.86 | 47.93 | 20.57 | 9.59 | 242.99 | 113.23 | 127.0 |
| 150 | 1.97 | HDG15002 | 152.15 | 78.50 | 30.43 | 15.70 | 59.89 | 30.90 | 102.0 |
|  | 3.94 | HDG15004 | 152.15 | 78.50 | 30.43 | 15.70 | 119.78 | 61.79 | 122.0 |
|  | 5.91 | HDG15006 | 152.15 | 78.50 | 30.43 | 15.70 | 181.75 | 88.32 | 142.0 |
|  | 7.87 | HDG15008 | 152.15 | 78.50 | 30.43 | 15.70 | 239.57 | 123.59 | 162.0 |
|  | 9.84 | HDG15010 | 152.15 | 78.50 | 30.43 | 15.70 | 299.46 | 154.48 | 181.0 |
|  | 11.81 | HDG15012 | 152.15 | 78.50 | 30.43 | 15.70 | 359.35 | 185.38 | 202.0 |
| 200 | 1.97 | HDG20002 | 204.95 | 94.00 | 40.99 | 18.80 | 80.68 | 37.01 | 121.0 |
|  | 5.91 | HDG20006 | 204.95 | 94.00 | 40.99 | 18.80 | 243.40 | 116.23 | 201.0 |
|  | 7.87 | HDG20008 | 204.95 | 94.00 | 40.99 | 18.80 | 322.73 | 148.03 | 242.0 |
|  | 11.81 | HDG20012 | 204.95 | 94.00 | 40.99 | 18.80 | 486.79 | 232.46 | 322.0 |
| 300 | 1.97 | HDG30002 | 353.60 | 117.00 | 70.71 | 23.46 | 139.19 | 46.18 | 406.0 |
|  | 5.91 | HDG30006 | 353.60 | 117.00 | 70.71 | 23.46 | 417.56 | 138.55 | 511.0 |
|  | 11.81 | HDG30012 | 353.60 | 117.00 | 70.71 | 23.46 | 835.11 | 277.10 | 668.0 |
| 400 | 1.97 | HDG40002 | 433.90 | 152.55 | 86.78 | 30.51 | 170.84 | 59.03 | 595.0 |
|  | 5.91 | HDG40006 | 433.90 | 152.55 | 86.78 | 30.51 | 512.51 | 177.09 | 728.0 |
|  | 12.01 | HDG40012 | 433.90 | 152.55 | 86.78 | 30.51 | 1042.10 | 366.38 | 928.0 |
| 500 | 1.97 | HDG50002 | 566.30 | 192.00 | 113.25 | 38.37 | 222.92 | 75.54 | 884.0 |
|  | 5.91 | HDG50006 | 566.30 | 192.00 | 113.25 | 38.37 | 668.77 | 226.61 | 1058.0 |
|  | 11.81 | HDG50012 | 566.30 | 192.00 | 113.25 | 38.37 | 1337.56 | 453.23 | 1321.0 |
| 600 | 1.97 | HDG60002 | 662.90 | 229.00 | 132.57 | 45.79 | 260.97 | 90.13 | 1045.0 |
|  | 5.91 | HDG60006 | 662.90 | 229.00 | 132.57 | 45.79 | 782.90 | 270.39 | 1246.0 |
|  | 9.84 | HDG60010 | 670.90 | 352.85 | 134.18 | 70.57 | 1408.93 | 740.94 | 948.0 |
|  | 11.81 | HDG60012 | 662.90 | 229.00 | 132.57 | 45.79 | 1566.00 | 540.79 | 1545.0 |
| 800 | 1.97 | HDG80002 | 911.60 | 300.00 | 182.32 | 59.99 | 358.91 | 118.09 | 1634.0 |
|  | 5.91 | HDG80006 | 911.60 | 300.00 | 182.32 | 59.99 | 1077.00 | 354.28 | 1914.0 |
|  | 11.81 | HDG80012 | 911.60 | 300.00 | 182.32 | 59.99 | 2153.00 | 708.57 | 2332.0 |
| 1000 | 1.97 | HDG100002 | 1136.00 | 420.00 | 227.19 | 83.97 | 447.23 | 165.29 | 2341.0 |
|  | 5.91 | HDG100006 | 1136.00 | 420.00 | 227.19 | 83.97 | 1342.00 | 495.87 | 2674.0 |
|  | 11.81 | HDG100012 | 1136.00 | 420.00 | 227.19 | 83.97 | 2683.00 | 991.75 | 3172.0 |

Z

Y

X D

HDG150	HDG200

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg 92

Related Product: CVP3-K

Pilot operated check valve kit. Used to hold the load in the event of a pressure loss in the system.

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

Optional Tilt-Saddles

Saddles tilt to a 5% maximum to adjust to the load. View our cylinder accessories page to figure out what saddle best

fits these cylinders.

pg 35

1-888-332-6419

24	www.BVAhydraulics.com

HDG Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

25

Cylinders

HDG300 - HDG1000

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) | Base To Adv. Port G (in) | Top To Retract Port H (in) | Base Mounting Holes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Bolt Circle X (in) | Thread Size (mm) | Thread Depth Z (in) |
| HDG10002 | 7.17 | 9.14 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG10004 | 9.13 | 13.07 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG10006 | 11.10 | 17.01 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG10008 | 13.07 | 20.94 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG10010 | 15.04 | 24.88 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG10012 | 17.01 | 28.82 | 5.12 | 3.50 | 3.74 | 2.95 | 2.13 | 1.89 | 3.74 | M12 | 0.87 |
| HDG15002 | 7.72 | 9.69 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG15004 | 9.69 | 13.62 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG15006 | 11.65 | 17.56 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG15008 | 13.62 | 21.50 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG15010 | 15.59 | 25.43 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG15012 | 17.56 | 29.37 | 6.22 | 8.07 | 4.33 | 3.70 | 2.40 | 2.22 | 5.12 | M12 | 0.87 |
| HDG20002 | 8.50 | 10.47 | 7.22 | 9.25 | 5.31 | 4.45 | 2.62 | 2.22 | 6.50 | M12 | 0.80 |
| HDG20006 | 12.44 | 18.35 | 7.24 | 9.25 | 5.31 | 4.45 | 2.62 | 2.22 | 6.50 | M12 | 0.80 |
| HDG20008 | 14.41 | 22.28 | 7.22 | 9.25 | 5.31 | 4.45 | 2.62 | 2.22 | 6.50 | M12 | 0.80 |
| HDG20012 | 18.35 | 30.16 | 7.24 | 9.25 | 5.31 | 4.45 | 2.62 | 2.22 | 6.50 | M12 | 0.80 |
| HDG30002 | 12.28 | 14.25 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 2.95 | 7.09 | M16 | 1.42 |
| HDG30006 | 16.22 | 22.13 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 2.95 | 7.09 | M16 | 1.42 |
| HDG30012 | 22.13 | 33.94 | 9.49 | 12.20 | 7.76 | 6.97 | 3.98 | 2.95 | 7.09 | M16 | 1.42 |
| HDG40002 | 14.74 | 16.71 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 4.13 | 8.07 | M16 | 1.42 |
| HDG40006 | 18.68 | 24.59 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 4.13 | 8.07 | M16 | 1.42 |
| HDG40012 | 24.59 | 36.40 | 10.51 | 13.78 | 8.50 | 7.72 | 4.49 | 4.13 | 8.07 | M16 | 1.42 |
| HDG50002 | 16.50 | 18.46 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 5.31 | 9.84 | M24 | 1.50 |
| HDG50006 | 20.43 | 26.34 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 5.31 | 9.84 | M24 | 1.50 |
| HDG50012 | 26.34 | 38.15 | 12.01 | 15.75 | 9.76 | 8.98 | 4.49 | 5.31 | 9.84 | M24 | 1.50 |
| HDG60002 | 16.89 | 18.86 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 5.31 | 10.83 | M24 | 1.50 |
| HDG60006 | 20.83 | 26.73 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 5.31 | 10.83 | M24 | 1.50 |
| HDG60010 | 18.00 | 28.50 | 13.07 | 17.05 | 9.00 | 7.87 | 4.49 | 5.31 | 10.83 | M24 | 1.50 |
| HDG60012 | 26.73 | 38.54 | 12.99 | 16.93 | 10.51 | 9.72 | 4.49 | 5.31 | 10.83 | M24 | 1.50 |
| HDG80002 | 18.66 | 20.63 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 5.31 | 12.99 | M24 | 1.50 |
| HDG80006 | 22.60 | 28.50 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 5.31 | 12.99 | M24 | 1.50 |
| HDG80012 | 28.50 | 40.31 | 15.24 | 19.88 | 12.48 | 11.69 | 5.87 | 5.31 | 12.99 | M24 | 1.50 |
| HDG100002 | 22.20 | 24.17 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 6.69 | 14.76 | M24 | 1.50 |
| HDG100006 | 26.14 | 32.05 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 6.69 | 14.76 | M24 | 1.50 |
| HDG100012 | 32.05 | 43.86 | 17.01 | 22.05 | 13.50 | 12.72 | 6.85 | 6.69 | 14.76 | M24 | 1.50 |

### Low Profile Cylinders

HL Series - Low Clearance Lifting

HL Series

Compact design for use in confined spaces

- Rod wiper helps seal out dirt and contamination

- Specially designed return spring provides quick retraction and extends the life of the spring

- Hard chrome plated rod helps prevent scratching and corrosion

- Rod end is grooved to reduce load slippage

- Includes high flow coupler and dust cap

- HL1001, HL2002, and HL3002 coupler is tilted 5° for added clearance

4.63”

HL6003KK ONLY*

3/8"-16 UNC

E	Z

Y

X

B

A

5°

C	 3/8"-18NPTF	 	D

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure at Capacity (psi) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 1.50 | HL1001 | 2.35 | 8,511 | 3.53 | 5.7 |
| 20 | 1.75 | HL2002 | 4.37 | 9,153 | 7.65 | 11.0 |
| 30 | 2.44 | HL3002 | 6.49 | 9,245 | 15.84 | 15.0 |
| 50 | 2.36 | HL5002 | 9.87 | 10,132 | 23.29 | 21.8 |
| 60 | 2.50 | HL6003KK* | 12.57 | 10,183 | 31.40 | 28.0 |
| 100 | 2.24 | HL10002 | 19.64 | 10,183 | 44.07 | 45.4 |

Low profile cylinders are great for confined spaces.

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Diameter C (in) | Outside Diameter D (in) | Rod Diameter E (in) | Rod Mounting |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Bolt Circle X (in) | Thread Y (mm) | Thread Depth Z (in) |
| HL1001 | 3.48 | 4.98 | 1.73 | 2.75 | 1.50 | 1.03 | M4 | 0.32 |
| HL2002 | 3.90 | 5.65 | 2.36 | 3.63 | 2.00 | 1.56 | M4 | 0.32 |
| HL3002 | 4.61 | 7.05 | 2.87 | 4.00 | 2.50 | 1.56 | M5 | 0.32 |
| HL5002 | 4.80 | 7.17 | 3.54 | 4.88 | 2.76 | 1.56 | M5 | 0.32 |
| HL6003KK* | 5.63 | 8.13 | 4.00 | 5.26 | 3.15 | - | - | - |
| HL10002 | 5.55 | 7.80 | 5.00 | 6.50 | 3.54 | 2.19 | M8 | 0.10 |

1-888-332-6419

26	www.BVAhydraulics.com

- HL6003KK is equipped with 4x 3/8”-16UMC mounting holes

J72101

### Low Profile Cylinders Combo Kit

Low Clearance Lifting

  - Compact design for use in confined spaces

  - Ideal for pushing applications

  - Rod wiper helps seal out dirt and contamination

Cylinders

Return spring ensures a quick retraction and ex- tends the life of the spring

  - Industrial hard chrome plated rod helps prevent scratching and corrosion

  - Includes stackable 1″, 2″, and 3″ aluminum adapt- ers that conveniently snap into the base of the rod

  - Includes high flow CH38F coupler and dust cap

HL Series

- Convenient storage case

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

and materials.

pg 72

3"

Adapter

1"

Adapter

2"

Adapter

1.5"

Saddle

J72101

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

| Cylinder Capacity (ton) | Model Number | Stroke (in) | Cylinder Effective Area (in2) | Collapsed Height (in) | Extended Height w/o adapter (in) | Internal Pressure at Capacity (psi) | Cylinder Housing Diameter (in) | Rod Diameter (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | J72101 | 1.50 | 2.35 | 3.48 | 4.98 | 8,511 | 2.75 | 1.50 | 10 .0 |

Bolt-On Tilt Saddles

SDTL50

SDTL100

SDTL10

Bolt-On tilt saddles are designed specifically for use with the HL Series Low Profile Cylinders and Low Profile Cylinder Kit. These saddles provide a max. 5° tilt for uneven loads.

| Model Number | Height (in) | Description |
| --- | --- | --- |
| SDTL10 | 0.83 | J72101, HL1001 Cylinder Kit |
| SDTL50 | 1.14 | HL5002 Cylinder |
| SDTL100 | 1.39 | HL10002 Cylinder |

## 27

### Flat Body Cylinders

HF15006

HF0503

HF10006

HF Series

E

3/8” - 18NPTF

B

A

G

Extremely Low Lifting

  - Extremely low collapsed height for applications with minimal clearance at the lift point

  - Maximum working pressure 10,000 psi / 700 bar

  - HF0503 and HF0503B come with a 3/8″ regular flow coupler

X

L

M

D1

Y

D2

C

HF5006 in the collapsed position ready to lift a load with a PA1500 foot pump.

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure at Capacity (psi) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0.24 | HF0503* | 1.10 | 9,090 | 0.26 | 1.3 |
| 10 | 0.43 | HF1005 | 2.36 | 8,470 | 1.01 | 3.3 |
| 20 | 0.43 | HF2005 | 4.38 | 9,130 | 1.88 | 5.9 |
| 30 | 0.51 | HF3005 | 6.49 | 9,240 | 3.31 | 9.7 |
| 50 | 0.63 | HF5006 | 9.86 | 10,140 | 6.21 | 14.9 |
| 75 | 0.63 | HF7506 | 16.10 | 9,320 | 10.14 | 25.4 |
| 100 | 0.63 | HF10006 | 19.64 | 10,180 | 12.37 | 31.5 |
| 150 | 0.63 | HF15006 | 30.67 | 9,780 | 19.32 | 53.4 |

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D1 x D2 D (in) | Rod Dia. E (in) | Base to Port G (in) | Rod to Base L (in) | Rod to Mtg. Hole M (in) | Hole Pitch X (in) | Hole Dia. Y (in) | Counter Bore Dia. (in) | Counter Bore Depth (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HF0503* | 1.26 | 1.50 | 1.18 | 2.4x1.6 | 0.98 | 0.69 | 0.80 | 0.87 | 1.12 | 0.20 | 0.31 | 0.17 |
| HF1005 | 1.69 | 2.12 | 1.73 | 3.3x2.2 | 1.50 | 0.77 | 1.12 | 1.32 | 1.44 | 0.28 | 0.42 | 0.31 |
| HF2005 | 2.01 | 2.44 | 2.36 | 4.0x3.0 | 2.00 | 0.75 | 1.54 | 1.56 | 1.94 | 0.40 | 0.59 | 0.39 |
| HF3005 | 2.31 | 2.82 | 2.87 | 4.6x3.8 | 2.50 | 0.79 | 1.88 | 1.75 | 2.06 | 0.40 | 0.63 | 0.44 |
| HF5006 | 2.64 | 3.27 | 3.54 | 4.5x4.5 | 2.76 | 0.79 | 2.25 | 2.13 | 2.62 | 0.43 | 0.75 | 0.50 |
| HF7506 | 3.13 | 3.76 | 4.53 | 6.5x5.5 | 3.54 | 0.77 | 2.75 | 2.63 | 3.00 | 0.51 | 0.83 | 0.56 |
| HF10006 | 3.37 | 4.00 | 5.00 | 7.0x6.0 | 3.54 | 0.79 | 3.00 | 2.94 | 3.00 | 0.51 | 0.83 | 0.56 |
| HF15006 | 3.94 | 4.57 | 6.25 | 8.5x7.5 | 4.65 | 0.94 | 3.75 | 3.25 | 4.62 | 0.51 | 0.83 | 0.56 |

1-888-332-6419

28	www.BVAhydraulics.com

*HF0503 comes equipped with the CR38F coupler instead of CH38F

### Flat Body Cylinder Kits

•

•

•

•

HF Series - Kits

Various height stackable magnetic adapters eliminate dangerous cribbing

Cylinders

Convenient carrying case Magnetic adapters are stackable

U.S. Patent No.7,237,762 B2

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

HF Series

Magnet Shim

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified)

F B

G

E

D

C

Magnet Shim

A

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Model | Oil Capacity (in3) | Collapsed Height (in) | Weight (lbs) | Adapter Dimensions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | A (in) | B (in) | C (in) | D (in) | E (in) | F (in) | G (in) |
| 5 | 0.24 | HF0503B** | HF0503 | 0.26 | 1.26 | 5.5 | 0.17 | 1.61 | 2.56 | 2.32 | 0.48 | 1.10 | 0.24 |
|  |  |  |  |  |  |  | 0.42 | 1.61 | 2.56 | 2.32 | 0.48 | 1.10 | 0.24 |
|  |  |  |  |  |  |  | 0.92 | 1.61 | 2.56 | 2.32 | 0.48 | 1.10 | 0.24 |
|  |  |  |  |  |  |  | 1.17 | 1.61 | 2.56 | 2.32 | 0.48 | 1.10 | 0.24 |
| 10 |  |  |  |  |  |  | 0.39 | 2.20 | 3.54 | 3.23 | 0.81 | 1.46 | 0.28 |
|  | 0.43 | HF1005B | HF1005 | 1.01 | 1.69 | 9.2 | 0.79 | 2.20 | 3.54 | 3.23 | - | - | - |
|  |  |  |  |  |  |  | 1.57 | 2.20 | 3.54 | 3.23 | - | - | - |
| 20 | 0.43 | HF2005B | HF2005 | 1.88 | 2.01 | 15.0 | 0.39 | 2.99 | 4.25 | 3.94 | 1.08 | 1.93 | 0.39 |
|  |  |  |  |  |  |  | 0.79 | 2.99 | 4.25 | 3.94 | - | - | - |
|  |  |  |  |  |  |  | 1.57 | 2.99 | 4.25 | 3.94 | - | - | - |
| 30 |  |  |  |  |  |  | 0.39 | 3.75 | 4.76 | 4.45 | 1.32 | 2.06 | 0.39 |
|  | 0.51 | HF3005B | HF3005 | 3.31 | 2.31 | 21.0 | 0.79 | 3.75 | 4.76 | 4.45 | - | - | - |
|  |  |  |  |  |  |  | 1.57 | 3.75 | 4.76 | 4.45 | - | - | - |
| 50 | 0.63 | HF5006B | HF5006 | 6.21 | 2.64 | 32.3 | 0.39 | 4.49 | 5.89 | 5.57 | 1.61 | 2.62 | 0.43 |
|  |  |  |  |  |  |  | 0.79 | 4.49 | 5.89 | 5.57 | - | - | - |
|  |  |  |  |  |  |  | 1.57 | 4.49 | 5.89 | 5.57 | - | - | - |

**HF0503B comes equipped with the coupler CR38F

## 29

### Lock Nut Cylinders

HLN Series - Heavy Lifting w/ Mechanical Lock Nut

HLN15006

HLN5504

Capacity:

55-300 Tons

Stroke:

1.97 - 11.81 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

  - - 33.27 Inches

HLN10012

    - Single acting cylinder with mechanical safety lock nut

    - Designed to support loads without hydraulic pressure

    - Overflow port prevents cylinders

from over travel

    - Load return

    - Solid steel cylinder body

    - Electroless nickel plated rod to resist corrosion and rust

    - Maximum working pressure: 10,000 psi / 700 bar

Optional Tilt Saddles

HLN10002

| SDT55 | → | HLN55, HLN100 Cylinders |
| --- | --- | --- |
| SDT200 | → | HLN150, HLN200 Cylinders |

Saddles tilt to a 5° max to adjust to the load.

pg35

(42) HLN5508 lock nut cylinders lifting the Jordan Bridge in Chesapeake, VA.

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 55 | 1.97 | HLN5502 | 10.99 | 21.65 | 34.0 |
|  | 3.94 | HLN5504 | 10.99 | 43.27 | 45.0 |
|  | 5.91 | HLN5506 | 10.99 | 64.90 | 55.0 |
|  | 7.87 | HLN5508 | 10.99 | 86.54 | 66.0 |
|  | 9.84 | HLN5510 | 10.99 | 108.17 | 77.0 |
|  | 11.81 | HLN5512 | 10.99 | 129.80 | 87.0 |
| 100 | 1.97 | HLN10002 | 20.57 | 40.49 | 67.0 |
|  | 3.94 | HLN10004 | 20.57 | 80.98 | 86.0 |
|  | 5.91 | HLN10006 | 20.57 | 121.48 | 104.0 |
|  | 7.87 | HLN10008 | 20.57 | 161.97 | 123.0 |
|  | 9.84 | HLN10010 | 20.57 | 202.46 | 141.0 |
|  | 11.81 | HLN10012 | 20.57 | 242.95 | 160.0 |
| 150 | 1.97 | HLN15002 | 31.16 | 61.39 | 116.0 |
|  | 3.94 | HLN15004 | 31.16 | 122.79 | 145.0 |
|  | 5.91 | HLN15006 | 31.16 | 184.18 | 174.0 |
|  | 7.87 | HLN15008 | 31.16 | 245.27 | 202.0 |
|  | 9.84 | HLN15010 | 31.16 | 306.66 | 231.0 |
|  | 11.81 | HLN15012 | 31.16 | 368.05 | 259.0 |
| 200 | 1.97 | HLN20002 | 41.00 | 80.70 | 181.0 |
|  | 3.94 | HLN20004 | 41.00 | 161.38 | 218.0 |
|  | 5.91 | HLN20006 | 41.00 | 242.00 | 256.0 |
|  | 7.87 | HLN20008 | 41.00 | 323.00 | 293.0 |
|  | 9.84 | HLN20010 | 41.00 | 403.50 | 331.0 |
|  | 11.81 | HLN20012 | 41.00 | 484.15 | 368.0 |
| 250 | 1.97 | HLN25002 | 56.27 | 110.77 | 253.0 |
|  | 5.91 | HLN25006 | 56.27 | 332.32 | 356.0 |
|  | 11.81 | HLN25012 | 56.27 | 664.64 | 510.0 |
| 300 | 1.97 | HLN30002 | 70.41 | 138.61 | 379.0 |
|  | 5.91 | HLN30006 | 70.41 | 415.83 | 510.0 |
|  | 11.81 | HLN30012 | 70.41 | 831.66 | 706.0 |

1-888-332-6419

30	www.BVAhydraulics.com

HLN Series

Safety Color Band

BVA Lock nut cylinders are equipped with a yellow safety band. Do not extend the ram beyond this point as it may result in a loss of hydraulic oil and damage to the cylinder and/or load.

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

31

I

H	3/8"-18NPTF

G1

Cylinders

F	C	D

G2

A

B

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Diameter C (in) | Outside Diameter D (in) | Saddle Diameter F (in) | Depth of Rod Hole G1 (in) | Saddle Protrusion From Rod G2 (in) | Lock Nut Length H (in) | Base to Inlet Port I (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HLN5502 | 6.50 | 8.40 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN5504 | 8.40 | 12.40 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN5506 | 10.40 | 16.30 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN5508 | 12.40 | 20.20 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN5510 | 14.30 | 24.20 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN5512 | 16.30 | 28.10 | 3.70 | 4.90 | 2.80 | 0.51 | 0.08 | 1.40 | 1.10 |
| HLN10002 | 7.40 | 9.30 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN10004 | 9.30 | 13.30 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN10006 | 11.30 | 17.20 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN10008 | 13.30 | 21.10 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN10010 | 15.20 | 25.10 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN10012 | 17.20 | 29.00 | 5.10 | 6.50 | 2.80 | 0.51 | 0.08 | 1.80 | 1.10 |
| HLN15002 | 8.23 | 10.20 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN15004 | 10.20 | 14.13 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN15006 | 12.17 | 18.07 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN15008 | 14.13 | 22.01 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN15010 | 16.10 | 25.94 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN15012 | 18.07 | 29.88 | 6.30 | 8.07 | 5.12 | 0.98 | 0.08 | 1.73 | 1.54 |
| HLN20002 | 9.57 | 11.50 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN20004 | 11.54 | 15.47 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN20006 | 13.50 | 19.50 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN20008 | 15.50 | 23.40 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN20010 | 17.40 | 27.30 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN20012 | 19.41 | 31.22 | 7.22 | 9.25 | 5.12 | 0.98 | 0.08 | 1.97 | 1.97 |
| HLN25002 | 9.80 | 11.77 | 8.46 | 10.83 | 5.91 | 0.98 | 0.08 | 2.20 | 1.97 |
| HLN25006 | 13.74 | 19.65 | 8.46 | 10.83 | 5.91 | 0.98 | 0.08 | 2.20 | 1.97 |
| HLN25012 | 19.65 | 31.46 | 8.46 | 10.83 | 5.91 | 0.98 | 0.08 | 2.20 | 1.97 |
| HLN30002 | 11.61 | 13.58 | 9.47 | 12.20 | 5.47 | 0.98 | 0.20 | 2.36 | 2.32 |
| HLN30006 | 15.55 | 21.46 | 9.47 | 12.20 | 5.47 | 0.98 | 0.20 | 2.36 | 2.32 |
| HLN30012 | 21.46 | 33.27 | 9.47 | 12.20 | 5.47 | 0.98 | 0.20 | 2.36 | 2.32 |

### Lock Nut Aluminum Cylinders

HULN Series - Heavy Lifting w/ Mechanical Lock Nut

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our product.

G2

F

G1

G

H

HULN Series

B

A

3/8"-18NPTF

I

C D

- Single acting cylinder with mechanical safety lock nut

- Designed to support loads without hydraulic pressure

- Overflow port prevents cylinders from over travel

- Load return

- Lightweight aluminum cylinder body

- Steel saddle

- Hard anodized rod to resist corrosion and rust

- Maximum working pressure: 10,000 psi / 700 bar

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Oil Capacity (in3) | Weight (lbs) | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | O.D. D (in) | Saddle Dia. F (in) | Saddle Protrusion from Rod G (in) | Lock Nut Length H (in) | Base To Adv Port I (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 2.00 | HULN3002 | 6.50 | 13.00 | 15.0 | 7.75 | 9.75 | 2.88 | 4.50 | 2.00 | 0.10 | 1.25 | 1.50 |
|  | 4.00 | HULN3004 | 6.50 | 26.00 | 18.0 | 9.75 | 13.75 | 2.88 | 4.50 | 2.00 | 0.10 | 1.25 | 1.50 |
|  | 6.00 | HULN3006 | 6.50 | 39.00 | 21.0 | 11.75 | 17.75 | 2.88 | 4.50 | 2.00 | 0.10 | 1.25 | 1.50 |
|  | 8.00 | HULN3008 | 6.50 | 52.00 | 24.0 | 13.75 | 21.75 | 2.88 | 4.50 | 2.00 | 0.10 | 1.25 | 1.50 |
|  | 10.00 | HULN3010 | 6.50 | 65.00 | 27.0 | 15.75 | 25.75 | 2.88 | 4.50 | 2.00 | 0.10 | 1.25 | 1.50 |
| 50 | 2.00 | HULN5002 | 11.04 | 21.65 | 21.0 | 8.13 | 10.13 | 3.75 | 5.50 | 2.50 | 0.10 | 1.50 | 1.50 |
|  | 4.00 | HULN5004 | 11.04 | 43.30 | 26.0 | 10.13 | 14.13 | 3.75 | 5.50 | 2.50 | 0.10 | 1.50 | 1.50 |
|  | 6.00 | HULN5006 | 11.04 | 64.92 | 29.0 | 12.13 | 18.13 | 3.75 | 5.50 | 2.50 | 0.10 | 1.50 | 1.50 |
|  | 8.00 | HULN5008 | 11.04 | 86.56 | 34.0 | 14.13 | 22.13 | 3.75 | 5.50 | 2.50 | 0.10 | 1.50 | 1.50 |
|  | 10.00 | HULN5010 | 11.04 | 108.20 | 38.0 | 16.13 | 26.13 | 3.75 | 5.50 | 2.50 | 0.10 | 1.50 | 1.50 |
| 100 | 2.00 | HULN10002 | 20.68 | 40.51 | 43.0 | 8.38 | 10.38 | 5.13 | 8.00 | 3.50 | 0.10 | 1.75 | 1.50 |
|  | 4.00 | HULN10004 | 20.68 | 81.00 | 51.0 | 10.38 | 14.38 | 5.13 | 8.00 | 3.50 | 0.10 | 1.75 | 1.50 |
|  | 6.00 | HULN10006 | 20.68 | 121.50 | 60.0 | 12.38 | 18.38 | 5.13 | 8.00 | 3.50 | 0.10 | 1.75 | 1.50 |
|  | 8.00 | HULN10008 | 20.68 | 162.00 | 69.0 | 14.38 | 22.38 | 5.13 | 8.00 | 3.50 | 0.10 | 1.75 | 1.50 |
|  | 10.00 | HULN10010 | 20.68 | 202.50 | 77.0 | 16.38 | 26.38 | 5.13 | 8.00 | 3.50 | 0.10 | 1.75 | 1.50 |
| 150 | 2.00 | HULN15002 | 30.68 | 60.60 | 66.0 | 8.38 | 10.38 | 6.25 | 10.00 | 4.50 | 0.10 | 1.75 | 1.50 |
|  | 4.00 | HULN15004 | 30.68 | 121.20 | 79.0 | 10.38 | 14.38 | 6.25 | 10.00 | 4.50 | 0.10 | 1.75 | 1.50 |
|  | 6.00 | HULN15006 | 30.68 | 181.80 | 92.0 | 12.38 | 18.38 | 6.25 | 10.00 | 4.50 | 0.10 | 1.75 | 1.50 |
|  | 8.00 | HULN15008 | 30.68 | 242.40 | 106.0 | 14.38 | 22.38 | 6.25 | 10.00 | 4.50 | 0.10 | 1.75 | 1.50 |
|  | 10.00 | HULN15010 | 30.68 | 303.00 | 123.0 | 16.38 | 26.38 | 6.25 | 10.00 | 4.50 | 0.10 | 1.75 | 1.50 |

1-888-332-6419

32	www.BVAhydraulics.com

Low Profile Lock Nut Cylinders

Low Clearance Lifting w/ Mechanical Lock Nut

- Single acting cylinder with mechanical safety lock nut

- Designed to support loads without hydraulic pressure

- Low profile for confined spaces

Cylinders

5° tilting saddle is standard

- Maximum working pressure: 10,000 psi/700 bar

- Equipped with CH38F high flow coupler

- Load Return

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation.	92

Always be prepared.	pg

HLNF Series

E F

Safety Color Band

BVA Lock nut cylinders are equipped with a yellow safety band. Do not extend the ram beyond this point as it may result in a loss of hydraulic oil and damage to the cylinder and/or load.

5°

G	Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and

H	types to suit any need.

B	pg69

A

I

C	 D

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Oil Capacity (in3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 60 | 1.97 | HLNF6002 | 13.42 | 26.42 | 31.3 |
| 100 | 1.97 | HLNF10002 | 22.47 | 44.23 | 54.1 |
| 160 | 1.77 | HLNF16002 | 35.51 | 62.91 | 91.0 |
| 200 | 1.77 | HLNF20002 | 43.95 | 77.86 | 118.6 |
| 250 | 1.77 | HLNF25002 | 56.27 | 99.70 | 154.6 |

J

K

| Model Number | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) | Rod Outside Thread E | Saddle Dia. F (in) | Saddle Protrusion From Rod G (in) | Lock Nut Length H (in) | Base to Inlet Port I (in) | Coupler to Eylet J (in) | Eyelet to Eyelet K (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HLNF6002 | 4.92 | 6.89 | 4.13 | 5.51 | TW105-4N | 3.23 | 0.24 | 1.10 | 0.75 | 9.78 | 7.39 |
| HLNF10002 | 5.39 | 7.36 | 5.35 | 6.89 | TW135-4N | 4.33 | 0.31 | 1.22 | 0.83 | 10.95 | 8.37 |
| HLNF16002 | 5.83 | 7.60 | 6.72 | 8.66 | TW170-4N | 5.83 | 0.35 | 1.57 | 1.06 | 12.46 | 9.62 |
| HLNF20002 | 6.10 | 7.87 | 7.48 | 9.66 | TW190-4N | 6.30 | 0.39 | 1.69 | 1.18 | 13.30 | 10.31 |
| HLNF25002 | 6.26 | 8.03 | 8.46 | 10.83 | TW215-4N | 7.28 | 0.43 | 1.73 | 1.26 | 14.31 | 11.15 |

## 33

##### Cylinder Lifting Handles

LH25T34

Designed for ease of carrying cylinders

- Provides better control for positioning cylinder

- Easy installation

| Model Number | For Cylinders |
| --- | --- |
| LH25T34 | H2504 through H2514 |
| LH30T40 | H3008, HD3008, HD3014, HC2002T, HC2006T |
| LH30T45 | HC3002T, HC3006T, HD3007, HDC3010 |
| LH55T49 | HLN5504 through HLF5512 |

Clevis and Tangs

Rod

CEP Series

Base

CEB Series

B

H

D

C

G

B

F

F

D

H

C

HD cylinders can be used in pushing and pulling applications. In this photo, we see a HD2506 HD cylinder set up for a pull application using a rod clevis cylinder accessory

A	A

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

| Application | Cylinder Series | Model Number | Dimensions (in) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C | D | F | G | H |
| Rod | H05* | CEP05 | ¾″-16N | 1.13 | 2.25 | 0.63 | 0.56 | - | 0.63 |
|  | H10**, H15, HD10 | CEP15 | 1″-8UN | 1.69 | 3.31 | 0.88 | 1.00 | - | 1.00 |
|  | H25 | CEP25 | 1½″-16UNC | 2.25 | 3.81 | 1.25 | 1.50 | - | 1.25 |
| Base | H05* | CEB05 | 1.52 | 1.75 | 2.13 | 0.63 | 1.13 | 1.00 | 0.63 |
|  | H10 | CEB10 | 2.28 | 2.50 | 2.87 | 0.88 | 0.32 | 1.56 | 1.00 |
|  | H15 | CEB15 | 2.77 | 3.00 | 2.87 | 0.88 | 0.38 | 1.88 | 1.00 |
|  | H25 | CEB25 | 3.40 | 3.75 | 3.37 | 1.25 | 0.56 | 2.31 | 1.25 |

1-888-332-6419

34	www.BVAhydraulics.com

*Except H0500

**Except H1001

D

C

Cylinder Bases

C

A

B

| For use with Cylinder Series | Model Number | Dimensions (in) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| H05* | CB05 | 0.32 | 4.65 | 7.87 | 1.54 |
| H10** | CB10 | 0.39 | 4.72 | 9.06 | 2.28 |
| H15 | CB15 | 0.47 | 4.80 | 10.00 | 2.76 |
| H25 | CB25 | 0.63 | 4.96 | 11.02 | 3.41 |

Safety Instructions

Cylinder Accessories

Visit our DO’S and DON’TS

section to review the best

##### Cylinder Saddles

Grooved, Tilt, and Threaded

SD Series	SDT Series

SDTG Series

methods of operation. Always be prepared.

HTD10

pg 92

B

D

C

A

B

A

A

B	5˚	B

D	D	D

C	 C

A

| Saddle Type | Model Number | Capacity (ton) | For Cylinder(s) | Dimensions (in) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | A | B | C | D |
| Tilt | SDT05 | 5 | H05* | 0.98 | 1.22 | 0.67 | 1.20 |
|  | SDT10 | 10 | H10** | 1.38 | 1.50 | 0.86 | 1.50 |
|  | SDT15 | 15 | H15 | 1.50 | 1.57 | 0.86 | 1.58 |
|  | SDT25 | 25 | H25 | 1.97 | 1.60 | 1.14 | 1.59 |
|  | SDT55 | 55-100 | H55, HD55, H75, HD75, H100, HLN55 and HLN100 | 2.80 | 1.29 | 0.50 | 1.30 |
|  | SDTG100 | 100 | HG100 and HDG100 | 2.95 | 2.64 | 1.71 | 0.79 |
|  | SDTG150 | 150 | HG150 and HDG150 | 3.70 | 2.71 | 1.77 | 0.77 |
|  | SDTG200 | 200 | HG200 and HDG200 | 4.45 | 3.25 | 1.95 | 1.10 |
|  | SDT200 | 150-200 | HLN150 and HLN200 | 5.12 | 1.77 | 1.02 | 0.58 |
| Grooved | SD05 | 5 | H05* | 0.98 | 0.81 | - | 0.24 |
|  | SD10 | 10 | H10** | 1.38 | 1.06 | - | 0.24 |
|  | SD15 | 15 | H15 | 1.50 | 1.18 | - | 0.28 |
|  | SD25 | 25 | H25 | 1.97 | 1.14 | - | 0.28 |
| Threaded | HTD10 | 10 | HT1006 and HT1010 H1002-H1014, HD1010-HD1012 | 1-1/4"- 11.5NPTF | 0.95 | 1"- 8UNC | 1.77 |

*Except H0500

**Except H1001

## 35

### Hollow Hole Cylinders

HC Series

HC1201T

HC6006T

Capacity:

12-100 Tons

Stroke:

0.31 - 6.11 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

  - - 19.13 Inches

Here is a HC6006T in a pulling application.

HC3002T

    - Ideal for pulling and pushing applications

    - Heavy duty compression spring

    - Dual wipers help prevent contamination

    - Collar thread protector

    - High flow coupler with dust cap

    - XT cylinders do not come with internal rod threads

    - 60 and 100 Ton models are equipped with carrying handles

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure at Capacity (psi) | Oil Capacity (in3) | Weight (lbs) | Collapsed Height A (in) | Extended Height B (in) | Bore Dia. C (in) | O.D. D (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 0.31 | HC1201T* | 2.91 | 8,236 | 0.89 | 3.3 | 2.36 | 2.67 | 1.92 | 2.76 |
|  | 1.61 | HC1202T* | 2.91 | 8,236 | 4.70 | 6.2 | 4.74 | 6.36 | 1.92 | 2.76 |
|  | 1.61 | HC1202XT* | 2.91 | 8,236 | 4.70 | 6.2 | 4.74 | 6.36 | 1.92 | 2.76 |
|  | 2.99 | HC1203XT | 2.91 | 8,236 | 8.72 | 8.6 | 7.24 | 10.24 | 1.92 | 2.76 |
| 20 | 2.01 | HC2002T | 4.90 | 8,167 | 9.83 | 15.9 | 6.38 | 8.39 | 2.50 | 3.92 |
|  | 6.06 | HC2006T | 4.90 | 8,167 | 29.70 | 29.5 | 12.05 | 18.11 | 2.50 | 3.92 |
| 30 | 2.52 | HC3002T | 6.32 | 9,488 | 15.93 | 22.9 | 7.05 | 9.57 | 2.84 | 4.50 |
|  | 6.11 | HC3006T | 6.32 | 9,488 | 38.67 | 43.0 | 13.02 | 19.13 | 2.84 | 4.50 |
| 60 | 3.00 | HC6003T** | 13.05 | 9,195 | 39.10 | 65.7 | 9.77 | 12.77 | 4.08 | 6.25 |
|  | 6.00 | HC6006T** | 13.05 | 9,195 | 78.25 | 81.1 | 12.76 | 18.76 | 4.08 | 6.25 |
| 100 | 3.00 | HC10003T** | 22.13 | 9,036 | 66.31 | 113.3 | 10.00 | 13.00 | 5.31 | 8.39 |

1-888-332-6419

36	www.BVAhydraulics.com

- Cylinder comes with a CRZ14F regular flow coupler

** Cylinder is equipped with carrying handles

F

J

K

Cylinders

HC Series

H

Safety Practices

L	Good industry practice

B	recommends not exceeding

80% of maximum rated capacities of all our products.

A

3/8”-18NPTF

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Z	G

Safety Instructions

E	Visit our DO’S and DON’TS

section to review the best

C	methods of operation.

Always be prepared.

D

pg92

Combo Kits

Y	Most cylinders are available

in easy to order kits: cylinder,

gauge, couplers, hose and pump.

pg64

X E

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

| Model Number | Rod |  |  |  | Collar Thread |  | Base to Inlet Port G (in) | Base Mounting Hole |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I.D. E (in) | O.D. F (in) | Internal Thread J (in) | Internal Thread Length K (in) | Diameter H (in) | Length L (in) |  | Bolt Circle X (in) | Thread Y (in) | Thread Depth Z (in) |
| HC1201T* | 0.77 | 1.38 | ¾″-16UN | 0.63 | 2¾″-16 | 1.81 | 0.57 | 2.00 | 5/16″-18UNC | 0.35 |
| HC1202T* | 0.77 | 1.38 | ¾″-16UN | 0.63 | 2¾″-16 | 1.81 | 0.59 | 2.00 | 5/16″-18UNC | 0.35 |
| HC1202XT* | 0.77 | 1.38 | - | - | 2¾″-16 | 1.81 | 0.59 | 2.00 | 5/16″-18UNC | 0.35 |
| HC1203XT | 0.77 | 1.38 | - | - | 2¾″-16 | 1.81 | 0.75 | 2.00 | 5/16″-18UNC | 0.35 |
| HC2002T | 1.06 | 2.15 | 1-9/16″-16UN | 0.75 | 3⅞″-12 | 1.50 | 0.75 | 3.25 | ⅜″-16UNC | 0.37 |
| HC2006T | 1.06 | 2.15 | 1-9/16″-16UN | 0.75 | 3⅞″-12 | 1.50 | 0.75 | 3.25 | ⅜″-16UNC | 0.37 |
| HC3002T | 1.31 | 2.48 | 1-13/16″-16UN | 0.87 | 4½″-12 | 1.66 | 0.91 | 3.62 | ⅜″-16UNC | 0.55 |
| HC3006T | 1.31 | 2.48 | 1-13/16″-16UN | 0.87 | 4½″-12 | 1.66 | 0.91 | 3.62 | ⅜″-16UNC | 0.55 |
| HC6003T** | 2.13 | 3.66 | 2¾″-16UN | 0.75 | 6¼″-12 | 1.91 | 1.06 | 5.12 | ½″-13UNC | 0.55 |
| HC6006T** | 2.13 | 3.66 | 2¾″-16UN | 0.75 | 6¼″-12 | 1.91 | 1.06 | 5.12 | ½″-13UNC | 0.55 |
| HC10003T** | 3.11 | 4.92 | 4″-16UN | 0.98 | 8⅜″-12 | 2.38 | 1.06 | 7.01 | ⅝″-11UNC | 0.75 |

* Cylinder comes with a CRZ14F regular flow coupler

** Cylinder is equipped with carrying handles

## 37

### Double Acting Hollow Hole Cylinders

HDC Series

HDC3010

HDC10001

Capacity:

HDC6006

30 - 150 Tons

Stroke:

1.50 - 10.13 Inches

Max. Operating Pressure:

10,000 psi

Min. - Max. Height:

6.50 - 28.27 Inches

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

  - Ideal for pushing and pulling applications.

  - Dual wipers help prevent contamination

  - CH38F - 3/8” high flow female coupler

pg69

- Hard chrome rod prevents rust and corrosion

- Hardened steel seat

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

and materials.

pg 72

Combo Kits

Most cylinders are available in easy to order kits: Cylinder,

Gauge, Couplers, Hose and Pump.

pg64

| Cylinder Capacity |  | Stroke (in) | Model Number | Oil Capacity |  | Collapsed Height (in) | Extended Height (in) | Center Hole I.D. (in) | Rod O.D. (in) | Max Pressure (psi) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Push (ton) | Pull (ton) |  |  | Push (in3) | Pull (in3) |  |  |  |  |  |  |
| 30 | 20 | 7.00 | HDC3007 | 44.31 | 27.74 | 13.02 | 20.02 | 1.31 | 2.48 | 10.000 | 43.0 |
|  |  | 10.13 | HDC3010 | 64.15 | 40.16 | 17.00 | 27.13 | 1.31 | 2.48 | 10.000 | 56.7 |
| 60 | 44 | 3.50 | HDC6003 | 45.75 | 30.49 | 9.77 | 13.28 | 2.13 | 3.62 | 10.000 | 65.7 |
|  |  | 6.50 | HDC6006 | 84.81 | 56.53 | 12.76 | 19.26 | 2.13 | 3.62 | 10.000 | 81.1 |
|  |  | 10.12 | HDC6010 | 132.10 | 88.04 | 17.25 | 27.37 | 2.13 | 3.62 | 10.000 | 112.4 |
| 100 | 71 | 1.50 | HDC10001 | 33.23 | 21.18 | 6.50 | 8.00 | 3.11 | 4.92 | 10.000 | 73.7 |
|  |  | 3.00 | HDC10003 | 66.47 | 42.37 | 10.00 | 13.00 | 3.11 | 4.92 | 10.000 | 113.3 |
|  |  | 6.00 | HDC10006 | 132.39 | 84.73 | 13.49 | 19.49 | 3.11 | 4.92 | 10.000 | 153.2 |
|  |  | 10.13 | HDC10010 | 224.52 | 143.11 | 18.14 | 28.27 | 3.11 | 4.92 | 10.000 | 198.9 |
| 150 | 82.8 | 8.00 | HDC15008 | 259.19 | 132.44 | 13.75 | 21.75 | 3.94 | 5.91 | 10.000 | 239.2 |

1-888-332-6419

38	www.BVAhydraulics.com

E

Cylinders

Y	 HDC Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated

X	capacities of all our products.

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

L

G

K

3/8"-18NPTF

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

J    H	F

Safety Instructions

C

Visit our DO’S and DON’TS

D	section to review the best

methods of operation. Always be prepared.

pg 92

I

Z

A

| Model Number | Min. Height A (in) | Max. Height B (in) | Dia. of Center Tube C (in) | O.D. D (in) | I.D. E (in) | O.D. F (in) | Base to Inlet Port G (in) | Top to Return Port L (in) | Internal Thread H (in) | Internal Thread Length I (in) | Collar Thread |  | Base Mounting Hole |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Dia. J (in) | Length K (in) | Bolt Circle X (in) | Thread Y (in) | Thread Depth Z (in) |
| HDC3007 | 13.02 | 20.02 | 3.35 | 4.50 | 1.31 | 2.48 | 0.91 | 2.38 | 1 "-16UN | 0.94 | 4½"-12 | 1.66 | 3.62 | ⅜"-16UNC | 0.55 |
| HDC3010 | 17.00 | 27.13 | 3.35 | 4.50 | 1.31 | 2.48 | 0.91 | 2.38 | 1 "-16UN | 0.94 | 4½"-12 | 1.66 | 3.62 | ⅜"-16UNC | 0.55 |
| HDC6003 | 9.77 | 13.28 | 4.92 | 6.25 | 2.13 | 3.62 | 1.06 | 2.64 | 2¾"-16UN | 0.83 | 6¼"-12 | 1.91 | 5.12 | ½" 13UNC | 0.55 |
| HDC6006 | 12.76 | 19.26 | 4.92 | 6.25 | 2.13 | 3.62 | 1.06 | 2.64 | 2¾"-16UN | 0.83 | 6¼"-12 | 1.91 | 5.12 | ½" 13UNC | 0.55 |
| HDC6010 | 17.25 | 27.37 | 4.92 | 6.25 | 2.13 | 3.62 | 1.06 | 2.64 | 2¾"-16UN | 0.83 | 6¼"-12 | 1.91 | 5.12 | ½" 13UNC | 0.55 |
| HDC10001 | 6.50 | 8.00 | 6.50 | 8.39 | 3.11 | 4.92 | 1.06 | 1.75 | 4"- 16UN | 1.06 | - | - | 7.01 | ⅝" 11UNC | 0.75 |
| HDC10003 | 10.00 | 13.00 | 6.50 | 8.39 | 3.11 | 4.92 | 1.06 | 3.39 | 4"- 16UN | 1.06 | 8⅜"-12 | 2.38 | 7.01 | ⅝" 11UNC | 0.75 |
| HDC10006 | 13.49 | 19.49 | 6.50 | 8.39 | 3.11 | 4.92 | 1.06 | 3.39 | 4"- 16UN | 1.06 | 8⅜"-12 | 2.38 | 7.01 | ⅝" 11UNC | 0.75 |
| HDC10010 | 18.14 | 28.27 | 6.50 | 8.39 | 3.11 | 4.92 | 1.06 | 3.39 | 4"- 16UN | 1.06 | 8⅜"-12 | 2.38 | 7.01 | ⅝" 11UNC | 0.75 |
| HDC15008 | 13.75 | 21.75 | 7.48 | 9.75 | 3.94 | 5.91 | 1.50 | 2.38 | - | - | - | - | - | - | - |

## 39

### Hollow Hole Aluminum Cylinders

HUC Series - Lightweight, Spring Return

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and

types to suit any need.

pg69

HUC Series

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Oil Capacity (in3) | Weight (lbs) | Collapsed Height (in) | Extended Height (in) | Center Hole Diameter (in) | Outside Diameter (in) | Base To Advance Port (in) | Handle or Eyelets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 3.00 | HUC3003 | 7.21 | 22.00 | 22.0 | 8.68 | 11.68 | 1.31 | 5.50 | 1.00 | H |
|  | 6.00 | HUC3006 | 7.21 | 44.00 | 30.0 | 11.68 | 17.68 | 1.31 | 5.50 | 1.00 | H |
| 60 | 3.00 | HUC6003 | 12.3 | 37.00 | 42.0 | 10.25 | 13.25 | 2.13 | 7.50 | 1.00 | H |
|  | 6.00 | HUC6006 | 12.3 | 74.00 | 55.0 | 13.25 | 19.25 | 2.13 | 7.50 | 1.00 | H |

Double Acting Aluminum Hollow Hole Cylinders

HUDC Series - Lightweight

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg92

Relief Valve

All double acting cylinders are equipped with a

pressure relief valve located next to the retract port.

These pressure relief valves help prevent cylinder over pressurization on the return port side of the cylinder.

HUDC Series

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Maximum Cylinder Capacity (ton) |  | Cylinder Effective Area (in2) |  | Oil Capacity (in3) |  | Weight (lbs) | Collapsed Height (in) | Extended Height (in) | Cylinder Bore Dia. (in) | Outside Dia. (in) | Base To Adv. Port (in) | Top To Retract Port (in) | Handles or Eyelets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Push | Pull | Push | Pull | Push | Pull |  |  |  |  |  |  |  |  |
| 30 | 6.00 | HUDC3006 | 30.00 | 10.12 | 6.35 | 2.02 | 38.07 | 12.15 | 30.0 | 11.50 | 17.50 | 1.50 | 5.50 | 2.00 | 2.00 | H |
|  | 10.00 | HUDC3010 | 30.00 | 10.12 | 6.35 | 2.02 | 63.45 | 20.25 | 40.0 | 15.50 | 25.50 | 1.50 | 5.50 | 2.00 | 2.00 | H |
| 50 | 6.00 | HUDC5006 | 50.00 | 17.18 | 10.30 | 3.43 | 61.85 | 20.61 | 50.0 | 11.50 | 17.50 | 2.13 | 7.50 | 2.00 | 2.00 | H |
|  | 10.00 | HUDC5010 | 50.00 | 17.18 | 10.30 | 3.43 | 103.08 | 34.36 | 65.0 | 15.50 | 25.50 | 2.13 | 7.50 | 2.00 | 2.00 | H |
| 100 | 6.00 | HUDC10006 | 100.00 | 37.55 | 21.59 | 7.51 | 129.59 | 45.06 | 125.0 | 14.50 | 20.50 | 3.50 | 11.00 | 2.50 | 2.00 | E |
|  | 10.00 | HUDC10010 | 100.00 | 37.55 | 21.59 | 7.51 | 215.98 | 75.10 | 142.0 | 18.50 | 28.50 | 3.50 | 11.00 | 2.50 | 2.50 | E |
| 150 | 6.00 | HUDC15006 | 150.00 | 55.00 | 30.00 | 11.00 | 183.73 | 65.97 | 158.0 | 14.50 | 20.50 | 3.50 | 12.00 | 2.50 | 2.00 | E |
|  | 10.00 | HUDC15010 | 150.00 | 55.00 | 30.00 | 11.00 | 306.30 | 109.95 | 180.0 | 18.50 | 28.50 | 3.50 | 12.00 | 2.50 | 2.50 | E |

1-888-332-6419

40	www.BVAhydraulics.com

Pull Cylinders

HP Series - High-Tonnage Pulling

- High strength steel construction

- One piece high grade alloy bearing for side load protection and longer life

Cylinders

Polyurethane cup seal provides optimum performance

- External rod threads and Internal base threads

- Used for pulling and tensioning

- Maximum working pressure: 10,000 psi / 700 bar

- Spring extend return

HP Series

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

26.81

20.87

and materials.

pg 72

3/8"-18NPTF

2.78

CP10

M30X2	M30X2

G

3.35

2.17

The CP10* comes was a male/female set. This clevis is designed to aid in pulling applications with all the HP series cylinders.

Z	3/8”-18NPTF

6.30”

3.74”

M30x2

2.56”

D    A	C	E	F H

1.38”

2.95”

L		P

A

B

1.18”

* CP10 only works with HP1006

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure At Capacity (psi) | Oil Capacity (in3) | Collapsed Height A (in) | Extended Height B (in) | Outside Dia. D (in) | Cylinder Bore Dia. C (in) | Top to Inlet Port G (in) | Collar |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Thread H | Thread Length L (in) |
| 2.5 | 5.00 | HP0205 | 0.56 | 7,905 | 2.78 | 10.55 | 15.55 | 1.89 | 1.13 | 1.65 | 1½˝-16UN | 0.98 |
| 5 | 5.55 | HP0505 | 1.14 | 8,772 | 6.34 | 11.93 | 17.48 | 2.25 | 1.69 | 1.50 | 2¼˝-14UN | 1.00 |
| 10 | 5.95 | HP1006* | 2.44 | 7,663 | 14.48 | 11.75 | 17.70 | 3.35 | 2.17 | 1.50 | M85x2 | 1.00 |

| Model Number | Adapter |  | Rod |  |  | Base Mounting Hole |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Thread F (in) | Length P (in) | Dia. E (in) | Thread Length K (in) | Thread Q (mm) | Thread Y | Depth Z (in) |  |
| HP0205 | ¾˝-14NPTF | 1.13 | 0.75 | 0.76 | M16 x 1.5 | ¾˝-14NPTF | 0.72 | 6.4 |
| HP0505 | 1¼˝-11.5NPTF | 1.25 | 1.18 | 0.93 | M28 x 1.5 | 1¼˝-11.5NPTF | 0.95 | 9.3 |
| HP1006* | - | - | 1.26 | 1.02 | M30 x 2 | M30x2 | 0.98 | 20.9 |

* CP10 only works with HP1006

## 41

HT Series

Threaded Cylinders

HT Series

- Ideal for frame pulling machines

- Threaded rod and base

- Single acting cylinders with spring return

- Removable threaded Rod adapter used to attach accessories to rod end and base

- Maximum working pressure: 10,000 psi / 700 bar

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Safety Instructions

Visit our DO’S and DON’TS

3/8" - 18NPTF

L

V

E

K

S

A

B

section to review the best

methods of operation. Always be prepared.

pg 92	G

Combo Kits

Most cylinders are available in easy to order kits: cylinder,

gauge, couplers, hose and pump.

pg64

D

High-Flow Couplers	T High-Flow Coupler: CH38F is

included on all models (except where specified).

| Cylinder Capacity (tons) | Stroke (in) | Model Number | Cylinder Effective Area (in2) | Internal Pressure at Capacity (psi) | Oil Capacity (in3) | Collapsed Height A (in) | Extended Height B (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 5.07 | HT0405* | 1.00 | 10,120 | 5.00 | 10.31 | 15.39 | 5.0 |
| 10 | 5.95 | HT1006 | 2.24 | 8,928 | 13.33 | 11.46 | 17.41 | 10.6 |
|  | 9.96 | HT1010 | 2.24 | 8,928 | 22.30 | 15.47 | 25.43 | 14.1 |
| 20 | 5.16 | HT2005 | 5.14 | 10,741 | 25.70 | 11.77 | 16.93 | 22.0 |

| Model Number | Outside Dia. D (in) | Rod Dia. E (in) | Base to Inlet Port G (in) | Collar K | Collar Thread Length L (in) | Saddle Height S (in) | Base Threads T (in) | Rod Threads V (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HT0405* | 1.50 | 1.33 | 1.33 | 1.5”-16UNF | 0.94 | 1.00 | ¾” - 14 NPSM | 0.75” - 14 NPTF |
| HT1006 | 2.26 | 1.49 | 1.67 | 2.25”-14UN | 1.13 | 0.94 | 1.25” -11.5 NPSM | 1.25” -11.5 NPSM |
| HT1010 | 2.26 | 1.49 | 1.67 | 2.25”-14UN | 1.13 | 0.94 | 1.25” -11.5 NPSM | 1.25” -11.5 NPSM |
| HT2005 | 3.35 | 2.24 | 1.85 | 3.31”-12UN | 1.98 | 1.40 | 2” -11.5 NPTF | 2” -11.5 NPSM |

1-888-332-6419

42	www.BVAhydraulics.com

- Cylinder comes with a coupler: CRZ14F instead of CH38F

### Precision Double Acting Cylinders

HPD Series - High-Cycle Fixtured Application

B

A

Cylinders

M	L	I

Q	P

H              G

E	C	D

F

HPD Series

*Note: Precision double acting cylinders do not come with couplers.

  - Provides hydraulic power on both extend and retract

  - Designed for high cycle production applications

  - High strength steel construction from gun drilled solid bar

  - Rod wiper seals keep dirt and contamination out

N

K		3/8"-18NPTF	 	J

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

  - Clevis eye attachments are available

  - Maximum working pressure: 10,000 psi / 700 bar

and materials.

pg 72

| Cylinder Capacity (tons) | Model Number | Push Effective Area (in2) | Pull Effective Area (in2) | Collapsed Height A (in) | Extended Height B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | HPD0906 | 1.77 | 1.01 | 13.78 | 19.88 | 1.50 | 2.52 |
|  | HPD0910 | 1.77 | 1.01 | 17.80 | 27.91 | 1.50 | 2.52 |
| 16 | HPD1606 | 3.14 | 1.56 | 15.31 | 21.57 | 2.00 | 2.99 |
|  | HPD1610 | 3.14 | 1.56 | 19.29 | 29.53 | 2.00 | 2.99 |

| Model Number | Rod |  |  |  | Base to Adv. Port J (in) | Top to Return Port K (in) | Collar |  |  |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dia. E (in) | Protrusion F (in) | Thread G (in) | Thread Length H (in) |  |  | Length L (in) | Thread M (in) | Thread Length N (in) | Int. Base Thread P (in) | Int. Base Thread Length Q (in) |  |
| HPD0906 | 0.98 | 0.98 | ¾″-16 | 0.75 | 1.50 | 2.20 | 1.50 | 2″-12 | 0.60 | 1-11/16″-18 | 1.37 | 14.6 |
| HPD0910 | 0.98 | 0.98 | ¾″-16 | 0.75 | 1.50 | 2.20 | 1.50 | 2″-12 | 0.60 | 1-11/16″-18 | 1.37 | 18.5 |
| HPD1606 | 1.42 | 1.16 | 1⅛″-12 | 0.98 | 2.12 | 2.87 | 2.12 | 2⅝″-16 | 0.98 | 2-3/16″-16 | 1.61 | 22.0 |
| HPD1606 | 1.42 | 1.16 | 1⅛″-12 | 0.98 | 2.12 | 2.87 | 2.12 | 2⅝″-16 | 0.98 | 2-3/16″-16 | 1.61 | 28.2 |

A	D	CED Series

B	By attaching this clevis eye to a HPD cylinder, you gain the ability to pull applica- tions. For use with the 9 or 16 ton HPD cylinders only.

| Dimensions (in) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model Number | A | B | C | D | E | F | G |
| CED09 | 1.00 | 0.90 | 1-11/16”-16N | 0.75 | 1.31 | 0.94 | ¾”-16N |
| CED16 | 1.25 | 1.10 | 2-3/16”-16N | 1.00 | 1.88 | 1.19 | 1⅛”12N |

E

F

C	G

## 43

Hand Pump Matching Chart

Hand Pumps

- Match up the stroke length to the capacity of your cylinder to

find the selected hand pump to go with your cylinder

| Stroke (in) | Capacity (tons) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 25 | 30 | 55 | 75 | 100 |
| < 1.0 | 0.69 | - | - | - | - | - | - | - |
| 1.0 | 1.00 | 2.24 | 3.14 | 5.14 | 6.49 | 10.99 | 16.10 | 20.57 |
| 2.0 | 2.00 | 4.48 | 6.28 | 10.28 | 12.98 | 21.98 | 32.20 | 41.14 |
| 3.0 | 3.00 | 6.72 | 9.42 | 15.42 | 19.47 | 32.97 | 48.30 | 61.71 |
| 4.0 | 4.00 | 8.96 | 12.56 | 20.56 | 25.96 | 43.96 | 64.40 | 82.28 |
| 5.0 | 5.00 | 11.20 | 15.70 | 25.70 | 32.45 | 54.95 | 80.50 | 102.85 |
| 6.0 | 6.00 | 13.44 | 18.84 | 30.84 | 38.94 | 65.94 | 96.60 | 123.42 |
| 7.0 | 7.00 | 15.68 | 21.98 | 35.98 | 45.43 | 76.93 | 112.70 | 143.99 |
| 8.0 | 8.00 | 17.92 | 25.12 | 41.12 | 51.92 | 87.92 | 128.80 | 164.56 |
| 9.0 | 9.00 | 20.16 | 28.26 | 46.26 | 58.41 | 98.91 | 144.90 | 185.13 |
| 10.0 | 10.00 | 22.40 | 31.40 | 51.40 | 64.90 | 109.90 | 161.00 | 205.70 |
| 12.0 | 12.00 | 26.88 | 37.68 | 61.68 | 77.80 | 131.88 | 193.20 | 246.84 |
| 13.0 | 13.00 | 29.12 | 40.82 | 66.82 | 84.37 | 142.87 | 209.30 | 267.41 |
| 14.0 | 14.00 | 31.36 | 43.96 | 71.96 | 90.86 | 153.86 | 225.40 | 287.98 |
| Suggested* Hand Pump Models | P350 P601S | P1000 P1201S P1201 |  | P2001 P2301 P2301M |  | P4301 |  | P8701 P8701M |

*The suggested pump model is based on oil capacity requirements of the cylinder.

Matching Chart

Air, Electric, and Gas Engine Pumps

- Match up the stroke length and capacity of the cylinder to find the

suggested pump to work with your application

| Stroke (in) | Capacity (tons) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 25 | 30 | 55 | 75 | 100 |
| < 1.0 | 0.69 | - | - | - | - | - | - | - |
| 1.0 | 1.00 | 2.24 | 3.14 | 5.14 | 6.49 | 10.99 | 16.10 | 20.57 |
| 2.0 | 2.00 | 4.48 | 6.28 | 10.28 | 12.98 | 21.98 | 32.20 | 41.14 |
| 3.0 | 3.00 | 6.72 | 9.42 | 15.42 | 19.47 | 32.97 | 48.30 | 61.71 |
| 4.0 | 4.00 | 8.96 | 12.56 | 20.56 | 25.96 | 43.96 | 64.40 | 82.28 |
| 5.0 | 5.00 | 11.20 | 15.70 | 25.70 | 32.45 | 54.95 | 80.50 | 102.85 |
| 6.0 | 6.00 | 13.44 | 18.84 | 30.84 | 38.94 | 65.94 | 96.60 | 123.42 |
| 7.0 | 7.00 | 15.68 | 21.98 | 35.98 | 45.43 | 76.93 | 112.70 | 143.99 |
| 8.0 | 8.00 | 17.92 | 25.12 | 41.12 | 51.92 | 87.92 | 128.80 | 164.56 |
| 9.0 | 9.00 | 20.16 | 28.26 | 46.26 | 58.41 | 98.91 | 144.90 | 185.13 |
| 10.0 | 10.00 | 22.40 | 31.40 | 51.40 | 64.90 | 109.90 | 161.00 | 205.70 |
| 12.0 | 12.00 | 26.88 | 37.68 | 61.68 | 77.80 | 131.88 | 193.20 | 246.84 |
| 13.0 | 13.00 | 29.12 | 40.82 | 66.82 | 84.37 | 142.87 | 209.30 | 267.41 |
| 14.0 | 14.00 | 31.36 | 43.96 | 71.96 | 90.86 | 153.86 | 225.40 | 287.98 |
| Suggested Air/Electric/Gas Pump Models | PA600 | PA1500 PA1500L PA1500M PA2000 |  |  | PA3801 PA3801M PE30M3N01A |  | PE40M3N02A PE50M3N03A PG70M3N05 |  |

1-888-332-6419

44	www.BVAhydraulics.com

45

m

Pumps

| Power | Pump Type | Page Number / |  |
| --- | --- | --- | --- |
| Source |  | Reference I | age |
| Manual | Single Speed Metal Hand Pumps (P-Series) |  | 46-47 |
|  | Two Speed Metal Hand Pumps (P-Series) |  |  |
|  | Two Speed Aluminum Hand Pumps (P-Series) |  |  |
|  | Double Acting Hand Pumps (P-Series) |  |  |
|  | Low Pressure Hand Pump (P-Series) |  |  |
|  | Two Speed Aluminum Multi Direction Hand Pump (P-Series) |  |  |
|  | Foot Pump (P57-Series) |  | 48 |
| Air | Rotary Air Pumps (PAR-Series) |  | 49 |
|  | Portable Air Pump (PA-Series) |  | 50-51 |
|  | Metal Air Pumps (PA-Series) |  |  |
|  | Double Acting Air Pump (PA-Series) |  |  |
|  | Remote Control Air Pump (PA-Series) |  |  |
| Electric | Single Phase Electric Pumps |  | 52-55 |
|  | PE30 0.5 HP Electric Pumps |  |  |
|  | PE40 1.0 HP Electric Pumps |  |  |
|  | PE50 1.5 HP Electric Pumps |  |  |
|  | 3 Phase Electric Pumps |  | 56-59 |
|  | PE40 1.0 HP 3 Phase Electric Pumps |  |  |
|  | PE60 3.0 HP 3 Phase Electric Pumps |  |  |
|  | Universal Motor Pumps (PU-Series) |  | 63 |
| Gas | PG70 5.5 HP Gas Engine Pump |  | 60-62 |
|  | PG90 10.2 HP Gas Engine Pump |  |  |
|  | PD80 7.3 HP HATZ Diesel Engine Pump |  |  |
| Control Valves and Accessories Page 66 |  |  |  |

### Hand Pumps

Aluminum and Steel

Combo Kits

Most hand pumps are available in easy to order kits; Pump, Gauge, Couplers, Hose

and Cylinder

pg

64

P Series

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

P1201S

P601S

P240L

P350

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

P4301

Product Information

- Metal reservoirs resist cracking

- Internal pressure relief valve for overload protection

- Good for use with cylinders and hydraulic tools or OEM applications

- Two-Speed pumps reduce user fatigue and increase productivity

- P1000AD can be not only be used in the standard horizontal position, but can also be used with the outlet port facing upwards or downwards positions providing more

flexibility in operation

  - P1000RP comes with a 7/16”-20 return-to-tank port for applications where

this is required

P2301

P1201

P1000

P8701M

P2301M

| Model Number | Usable Oil Capacity (in3) | Oil Outlet Port | Pressure Rating/Oil Displacement per Stroke |  | Piston Stroke (in) | Handle Operating Force (lbs) | Reservoir Construction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st Stage (psi/in3) | 2nd Stage (psi/in3) |  |  |  |
| Single Speed - Single Acting |  |  |  |  |  |  |  |
| P240L | 15 | ¼"-18 NPTF | n/a | 7,200 / 0.09 | 0.63 | 56 | Steel |
| P601S | 37 | ⅜"-18 NPTF | n/a | 10,000 / 0.15 | 0.79 | 109 | Steel |
| P1201S | 67 | ⅜"-18 NPTF | n/a | 10,000 / 0.13 | 0.82 | 89 | Steel |
| 2-Speed - Single Acting |  |  |  |  |  |  |  |
| P350 | 21.4 | ¼"-18 NPTF | 200 / 0.18 | 10,000 / 0.05 | 0.38 | 88 | Aluminum |
| P1000 | 61 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Aluminum |
| P1000AD* | 53-62 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Aluminum |
| P1000RP** | 61 | ⅜"-18 NPTF | 200 / 0.81 | 5,000 / 0.14 | 0.82 | 99 | Aluminum |
| P1201 | 67 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Steel |
| P2001 | 122 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 95 | Steel |
| P2301 | 134 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Steel |
| P4301 | 250 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Steel |
| P8701 | 476 | ⅜"-18 NPTF | 290 / 7.71 | 10,000 / 0.29 | 1.69 | 143 | Steel |
| 2-Speed - Double Acting |  |  |  |  |  |  |  |
| P2301M | 134 | ⅜"-18 NPTF | 200 / 0.81 | 10,000 / 0.14 | 0.82 | 99 | Steel |
| P8701M | 476 | ⅜"-18 NPTF | 290 / 7.71 | 10,000 / 0.29 | 1.69 | 143 | Steel |

1-888-332-6419

46	www.BVAhydraulics.com

- Oil capacity varies by position. Pump can be mounted outlet up or down

** Unit is equipped with a 7/16”-20 Tank Return Port

⅜"-18NPTF

G		 	G

¼"- 18NPTF 	A		 	A

G (P1201S)

G (P1201)

A (P1201S)

B

F

F	B

E	D	 	D

A (P1201)

B

0.236Ø	E	C

E	D

C

H    J	F

⅜”-18NPTF

C

G

A

¼"-18NPTF

B

F

H	J

K		H		J K

0.4

H	J

Pumps

⅜” -18NPTF

G

K		A

2°

E 	D		 	G

C

K		0.4 H				J

⅜"-18NPTF

F

Ø.4

A		B

B	F	E	D C

E 	D		H	J

C (P1000 &P1000AD)

C (P1000RP)		K

H	J

G

K	 	A		 	C

G A

F	B	H	J

B

E

F

E	D

⅜” -18NPTF	C	 	D

| Model Number | Dimensions |  |  |  |  |  |  |  |  |  | Weight w/Oil (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | J | K |  |
| Single Speed - Single Acting |  |  |  |  |  |  |  |  |  |  |  |
| P240L | 14.80 | 4.25 | 15.45 | 8.68 | 1.00 | 1.25 | 15.25 | 3.50 | 2.44 | - | 9.0 |
| P601S | 18.94 | 6.46 | 19.53 | 15.35 | 1.57 | 1.26 | 20.79 | 5.20 | 3.74 | 14.45 | 13.0 |
| P1201S | 20.50 | 6.50 | 21.75 | 17.50 | 2.13 | 1.75 | 22.63 | 5.25 | 3.74 | 21.25 | 20.0 |
| 2-Speed - Single Acting |  |  |  |  |  |  |  |  |  |  |  |
| P350 | 13.43 | 5.91 | 13.89 | 9.09 | 2.36 | 1.34 | 14.13 | 3.35 | 2.56 | - | 6.6 |
| P1000 | 21.77 | 6.34 | 23.03 | 14.96 | 3.59 | 1.26 | 19.96 | 4.88 | 3.94 | 16.93 | 13.7 |
| P1000AD* | 21.77 | 6.34 | 23.03 | 14.96 | 3.59 | 1.26 | 19.96 | 4.88 | 3.94 | 16.93 | 13.7 |
| P1000RP** | 21.63 | 6.50 | 20.50 | 15.13 | 3.63 | 1.26 | 22.63 | 4.88 | 3.75 | 15.50 | 13.7 |
| P1201 | 21.73 | 6.73 | 23.07 | 18.38 | 2.13 | 1.73 | 22.63 | 4.92 | 3.74 | 18.27 | 20.5 |
| P2001 | 22.91 | 6.89 | 24.29 | 18.38 | 2.25 | 1.73 | 23.75 | 4.80 | 2.56 | - | 24.2 |
| P2301 | 21.73 | 6.81 | 23.98 | 3.74 | 1.00 | 1.73 | 23.50 | 5.31 | 3.74 | 22.56 | 27.7 |
| P4301 | 21.73 | 7.68 | 22.87 | 3.74 | 1.06 | 1.73 | 22.40 | 5.31 | 3.74 | 21.46 | 37.6 |
| P8701 | 25.79 | 11.61 | 30.35 | 11.54 | 24.17 | 8.25 | 26.94 | 15.24 | 11.18 | - | 76.0 |
| 2-Speed - Double Acting |  |  |  |  |  |  |  |  |  |  |  |
| P2301M | 25.45 | 6.81 | 27.44 | 3.74 | 1.06 | 1.50 | 21.76 | 4.72 | 3.54 | 22.56 | 32.3 |
| P8701M | 25.79 | 11.61 | 30.35 | 11.54 | 24.17 | 9.00 | 26.94 | 15.24 | 11.18 | - | 77.4 |

- Oil capacity varies by position. Pump can be mounted outlet up or down and does not need to be horizontal

** Unit is equipped with a 7/16”-20 Tank Return Port

## 47

### Foot Pump

P57 Series - 10,000 PSI

3.25”

19”

P570FP

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Frac/Oil Industry

1.8”

Our valve seat puller is designed for removal of valve seats. Use the following model number for ordering purposes:

7.32”

15.75”

3/8” - 18NPTF

1.06”

  - HVSP125

pg 82

SP5-3814

7.68”

This combination kit will work well with this product. See our combo section for more details.

4.53”	11.05”

24.02”

HF Kits

Consider using our portable HF kits in conjunction with this pump.

pg29

20.5”

Related Product: Nut Splitters

Safety Instructions

92

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.	pg

The HNS nut splitters pair up with this portable foot pump. Our hydraulic nut splitters come in a spacious tool box so its easy to carry around with you from site-to-site. This tool box is large enough to hold small cylinders, saddles, spacers or whatever tool you need to

bring with you to your next job site.

Model Number    Nut Size Capacity ATF	Stud Dimension

High-Flow Couplers

High-Flow Coupler: CH38F is

HNS105 HNS200

0.4375” - 0.875”

1.1875” - 2.375”

0.625” - 1.4375”

7/16” - 7/8”

included on all models (except

where specified).

HNS308	2.625” - 3.5”	1.75” - 2.25”

pg83

| Usable Oil (In3) | Model Number | Pressure Rating (psi) |  | Oil Displacement per Stroke (In3) |  | Max. Handle Effort (lbs) | Piston Stroke | Weight (Lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st Stage | 2nd Stage | "1st Stage" | 2nd Stage |  |  |  |
| 30.50 | P570FP | 200 | 10,000 | 0.81 | 0.14 | 127.2 | 0.82 | 18.0 |

1-888-332-6419

48	www.BVAhydraulics.com

PARD1701

PARD1703

PARM4003

PAR Series

Rotary Air Pumps

PAR Series - 10,000 PSI Rated

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types of gauges to suit

any need.	pg69

Related Product: Hoses

We offer a variety of hoses, fittings, lengths and materials.

pg

72

1/2” - NPT Air Inlet

49

PARD1701	PAR1703 and PAR4003

Pumps

3/8"-18NPTF (x2)

A

B

C D

E

1/2” - NPT Air Inlet

F

G

B

C D

E

3/8"-18NPTF(x2)

A

F G

| Used with Cylinder | Motor Size (hp) | Usable Oil Capacity (gal) | Model Number | Pressure Rating (psi) | Pressure Rating |  | Oil Output Flow Rate |  | Valve Type | Valve Function | Air Pressure Range (psi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1st Stage (psi) | 2nd Stage (psi) | 1st Stage (in³/min) | 2nd Stage (in³/min) |  |  |  |
| Single Acting | 1.7 | 1 | PARD1701 | 10,000 | 700 | 10,000 | 313 | 22 | Dump | Advance/Return | 60 ~ 80 |
|  | 1.7 | 3 | PARD1703 | 10,000 | 700 | 10,000 | 313 | 22 | Dump | Advance/Return | 60 ~ 80 |
|  | 1.7 | 3 | PAR1703 | 10,000 | 700 | 10,000 | 417 | 22 | Manual PW3 | Advance/Hold/Return | 60 ~ 80 |
|  | 4 | 1 | PARD4001 | 10,000 | 700 | 10,000 | 465 | 70 | Dump | Advance/Return | 60 ~ 100 |
|  | 4 | 3 | PARD4003 | 10,000 | 700 | 10,000 | 465 | 70 | Dump | Advance/Return | 60 ~ 100 |
|  | 4 | 3 | PAR4003 | 10,000 | 700 | 10,000 | 512 | 70 | Manual PW3 | Advance/Hold/Return | 60 ~ 100 |
| Double Acting | 1.7 | 3 | PARM1703 | 10,000 | 700 | 10,000 | 417 | 22 | Manual PW4 | Advance/Hold/Return | 60 ~ 80 |
|  | 4 | 3 | PARM4003 | 10,000 | 700 | 10,000 | 512 | 70 | Manual PW4 | Advance/Hold/Return | 60 ~ 100 |

| Model Number | Air Consumption (cfm) | Max Sound Level (dBA) | Dimensions (in) |  |  |  |  |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C | D | E | F | G |  |
| PARD1701 | 44 | 93 | 19.10 | 9.39 | 5.79 | 4.72 | 8.07 | 12.34 | 15.90 | 42.0 |
| PARD1703 | 44 | 93 | - | 11.95 | 8.35 | 7.28 | 12.01 | 12.34 | 17.96 | 54.0 |
| PAR1703 | 44 | 93 | 11.94 | 10.53 | 8.21 | 7.28 | 12.01 | 12.34 | 18.08 | 55.0 |
| PARD4001 | 95 | 100 | 19.10 | 11.00 | 5.79 | 4.72 | 8.07 | 12.34 | 16.34 | 54.0 |
| PARD4003 | 95 | 100 | - | 13.56 | 8.35 | 7.28 | 12.01 | 12.34 | 18.40 | 66.0 |
| PAR4003 | 95 | 100 | 13.55 | 10.53 | 8.21 | 7.28 | 12.01 | 12.34 | 18.52 | 67.0 |
| PARM1703 | 44 | 93 | 11.94 | 10.53 | 8.21 | 7.28 | 12.01 | 12.34 | 18.08 | 55.0 |
| PARM4003 | 95 | 100 | 13.55 | 10.53 | 8.21 | 7.28 | 12.1 | 12.34 | 18.52 | 67.0 |

### Air Pumps

Air Actuated Power

PA Series

Flow Rate @ 100 psi air pressure

70

Oil Flow (in3/min) 

60

50

40

30

20

PA1500L

PA1500

PA600H

PA600

10

0

0	2,000 4,000 6,000 8,000 10,000

Pressure (psi) 

Pressure (psi)

Model PA1500

Model PA2000 (Turbo)

Product Information

- Metal reservoirs resist cracking

- US Patent No. 5,946,912, a built in bypass device protects the hydraulic system from over pumping damage

- A compact yet powerful Air Turbo Motor US

Patent No. 5,341,723

  - Suggested air inlet range: 110-175 psi / 7.6-12.1 bar

  - Maximum working pressure: 10,000 psi / 700 Bar

Air Consumption= 7.8cfm

PA2000

PA3801

PA7550

PA1500A Pressure Adjustment

PA1500A Pressure can be adjusted externally from 1,000 to 10,000 psi.

PA7550M

PA3801M

PA1500M

| Model Number | Usable Oil Capacity | Oil Outlet Port | Output Flow Rate |  | Input Air Pressure Required (psi) | Air Consumption (cfm) | Reservoir Construction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No Load (in3/min) | Load (in3/min) |  |  |  |
| Single Acting |  |  |  |  |  |  |  |
| PA600 | 36.6 | ⅜"-18 NPTF | 61 | 9 | 110-175 | 7.8 | Steel |
| PA600H | 36.6 | ⅜"-18 NPTF | 61 | 9 | 110-175 | 7.8 | Steel |
| PA1500 | 91.5 | ⅜"-18 NPTF | 66 | 11 | 110-175 | 7.8 | Aluminum |
| PA1500A* | 91.5 | ⅜"-18 NPTF | 66 | 11 | 110-175 | 7.8 | Aluminum |
| PA1500L** | 91.5 | ⅜"-18 NPTF | 61 | 10 | 110-175 | 7.8 | Aluminum |
| PA2000 | 122.1 | ⅜"-18 NPTF | 65 | 12 | 110-175 | 7.8 | Aluminum |
| PA3801 | 231.9 | ⅜"-18 NPTF | 65 | 11 | 110-175 | 7.8 | Steel |
| PA3801L** | 231.9 | ⅜"-18 NPTF | 61 | 10 | 110-175 | 7.8 | Steel |
| PA7550 | 460.6 | ⅜"-18 NPTF | 65 | 11 | 110-175 | 7.8 | Steel |
| Double Acting |  |  |  |  |  |  |  |
| PA1500M | 91.5 | ⅜"-18 NPTF | 66 | 11 | 110-175 | 7.8 | Aluminum |
| PA3801M | 231.9 | ⅜"-18 NPTF | 65 | 11 | 110-175 | 7.8 | Steel |
| PA7550M | 460.6 | ⅜"-18 NPTF | 65 | 11 | 110-175 | 7.8 | Steel |

1-888-332-6419

50	www.BVAhydraulics.com

- This unit is equipped with an external pressure adjustment (1,000-10,000psi)

**These pumps are equipped with a hand held air powered pendant

PA600

¼"-18NPTF

A

B

C

D

0.31"

⅜"-18NPTF

PA Series

E

FRL-145

¼"-18NPTF	⅜"-18NPTF

C

PA1500

0.23"

4.57"

Ø0.20"

9.66"

9.53"

9.05"

1.00"

0.61"

0.61"

Use in compressed air lines upstream of all PA series air/hydraulic pumps to provide clean filtered (5um), lubricated air source to ensure

long air motor life. Assembly is equipped with a

Pumps

metal bowl with sight glass and a 0-145 PSIG pressure gauge. 145 PSIG maximum

B	D

A		 	E

PA1500L

E

D

1/4-20 UNC 2A

0.30"

0.23"

PA1500M

E

D

0.75"

0.35"

0.31"

working pressure. 3/8” NPT Ports. Use Class-1 Turbine Oil (ISO VG32).

4.82"

4.12"

AD-145

Use in compressed air lines upstream of BVA FRL-145 and all PA series air/hydraulic

pumps to remove 99% of free water from the air line to ensure long air motor life.

145 PSIG maximum working pressure. 3/8” NPT Ports.

1/4"-18NPTF	3/8"-18NPTF

1/4"-18NPTF

B	C	B

3/8"-18NPTF

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

A

PA3801 and PA7550

A

1.89”

M6x15L

and materials.

pg 72

1/4”-18NPTF

3/8”-18NPTF

C

2.04”

Air Pump Combo

Most air pumps are available in easy to order kits: pump,

B	D	gauge, coupler, hose

and cylinder

A	E

pg64

| Model Number | Base Mounting Holes | Dimensions (in) |  |  |  |  | Weight w/ Oil (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E |  |
| Single Acting |  |  |  |  |  |  |  |
| PA600 | yes | 16.77 | 9.84 | 7.01 | 6.18 | 5.59 | 14.1 |
| PA600H | yes | 16.77 | 9.84 | 7.01 | 4.49 | 5.75 | 12.5 |
| PA1500 | yes | 10.35 | 5.00 | 7.28 | 5.16 | 5.35 | 18.1 |
| PA1500A* | yes | 10.35 | 5.00 | 7.28 | 5.16 | 5.35 | 18.1 |
| PA1500L** | yes | 10.96 | 6.30 | 8.03 | 5.12 | 5.35 | 19.4 |
| PA2000 | yes | 12.64 | 6.42 | 8.15 | 5.20 | 7.36 | 20.1 |
| PA3801 | no | 12.64 | 5.08 | 7.36 | 5.24 | 7.36 | 27.1 |
| PA3801L** | no | 12.64 | 6.42 | 8.15 | 5.20 | 7.36 | 30.4 |
| PA7550 | no | 12.64 | 5.00 | 8.31 | 6.14 | 8.43 | 36.0 |
| Double Acting |  |  |  |  |  |  |  |
| PA1500M | yes | 10.51 | 5.00 | 7.99 | 5.35 | 5.35 | 19.1 |
| PA3801M | no | 12.64 | 5.04 | 8.03 | 5.31 | 7.36 | 24.0 |
| PA7550M | no | 12.64 | 5.98 | 9.02 | 6.34 | 10.04 | 40.0 |

* Unit is equipped with an external pressure adjustment (1,000-10,000psi)

**These pumps are equipped with a hand held air powered pendant

## 51

### Electric Pumps

PE30, PE40, PE50 Series

- Single Phase

- Meets OSHA regulations for sound (80dBA)

- 2 Stage pumps provide fast run up before the high pressure stage

- Suction strainers screen out contamination to prevent pump damage

- All units are wired as 120 VAC 60 Hz; Optional wiring includes 50 Hz motors and 240 VAC 60 Hz

- 100% Duty Cycle, NEMA 5 Enclosure TEFC motors

- All hand pendants are wired 24 VAC for safety

- Starts under full load

| PE50W4N03A |  | Use with Cylinder Type | Flow Rate |  |  |  | Motor Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st Stage |  | 2nd Stage |  |  |
|  |  |  | in3/min | psi | in3/min | psi |  |
| Dump Manual Valve w/10 ft. Motor Pendant |  |  |  |  |  |  |  |
|  | 2W/2P Advance/Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
| Dump Solenoid Valve w/10 ft. Pendant |  |  |  |  |  |  |  |
|  | 2W/2P Advance /Return (Auto*) |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
|  | 3W/2P Advance /Return** |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
| Manual Valve |  |  |  |  |  |  |  |
|  | 3W/3P Advance /Hold Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
|  | 4W/3P Advance /Hold/Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
| Manual Valve w/10 ft. Motor Pendant |  |  |  |  |  |  |  |
|  | 3W/3P Advance /Hold Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
|  | 4W/3P Advance /Hold/Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
| Solenoid Valve w/10 ft. Pendant & Locking Plate |  |  |  |  |  |  |  |
|  | 3W/3P Advance /Hold Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |
|  | 4W/3P Advance /Hold/Return |  | 293 | 350 | 18 | 10,000 | 0.5 Hp |
|  |  |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 476 | 700 | 61 | 10,000 | 1.5 Hp |

1-888-332-6419

52	www.BVAhydraulics.com

- Advance position holds pressure with motor running or off. Return retracts cylinder with motor off

** Advance position holds pressure with motor running and cylinder Returns when button is released.

PE Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

53

Electric Pumps Flow Chart

800

Oil Flow (in3/min) 

600

400

200

0

0	2,000	4,000	6,000	8,000	10,000

Pressure (psi) 

Model PE30 (0.5hp)	Model PE40 (1hp)

Pumps

Model PE50 (1.5hp)

| Usable Oil Capacity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Gal | 2 Gal | 3 Gal | 5 Gal | 10 Gal | 15 Gal | 25 Gal |
| Model Number | Model Number | Model Number | Model Number | Model Number | Model Number | Model Number |
| Dump Manual Valve w/10 ft. Motor Pendant |  |  |  |  |  |  |
| PE30DMP01A | PE30DMP02A | PE30DMP03A | - | - | - | - |
| - | PE40DMP02A | PE40DMP03A | PE40DMP05A | PE40DMP10A | - | - |
| - | - | PE50DMP03A | PE50DMP05A | PE50DMP10A | PE50DMP15A | PE50DMP25A |
| Dump Solenoid Valve w/10 ft. Pendant |  |  |  |  |  |  |
| PE30H2S01A | PE30H2S02A | PE30H2S03A | - | - | - | - |
| - | PE40H2S02A | PE40H2S03A | PE40H2S05A | PE40H2S10A | - | - |
| - | - | PE50H2S03A | PE50H2S05A | PE50H2S10A | PE50H2S15A | PE50H2S25A |
| PE30DSP01A | PE30DSP02A | PE30DSP03A | - | - | - | - |
| - | PE40DSP02A | PE40DSP03A | PE40DSP05A | PE40DSP10A | - | - |
| - | - | PE50DSP03A | PE50DSP05A | PE50DSP10A | PE50DSP15A | PE50DSP25A |
| Manual Valve |  |  |  |  |  |  |
| PE30M3N01A | PE30M3N02A | PE30M3N03A | - | - | - | - |
| - | PE40M3N02A | PE40M3N03A | PE40M3N05A | PE40M3N10A | PE40M3N15A | PE40M3N25A |
| - | - | PE50M3N03A | PE50M3N05A | PE50M3N10A | PE50M3N15A | PE50M3N25A |
| PE30M4N01A | PE30M4N02A | PE30M4N03A | - | - | - | - |
| - | PE40M4N02A | PE40M4N03A | PE40M4N05A | PE40M4N10A | PE40M4N15A | PE40M4N25A |
| - | - | PE50M4N03A | PE50M4N05A | PE50M4N10A | PE50M4N15A | PE50M4N25A |
| Manual Valve w/10 ft. Motor Pendant |  |  |  |  |  |  |
| PE30W3N01A | PE30W3N02A | PE30W3N03A | - | - | - | - |
| - | PE40W3N02A | PE40W3N03A | PE40W3N05A | PE40W3N10A | PE40W3N15A | PE40W3N25A |
| - | - | PE50W3N03A | PE50W3N05A | PE50W3N10A | PE50W3N15A | PE50W3N25A |
| PE30W4N01A | PE30W4N02A | PE30W4N03A | - | - | - | - |
| - | PE40W4N02A | PE40W4N03A | PE40W4N05A | PE40W4N10A | PE40W4N15A | PE40W4N25A |
| - | - | PE50W4N03A | PE50W4N05A | PE50W4N10A | PE50W4N15A | PE50W4N25A |
| Solenoid Valve w/10 ft. Pendant & Locking Plate |  |  |  |  |  |  |
| PE30S3L01A | PE30S3L02A | PE30S3L03A | - | - | - | - |
| - | PE40S3L02A | PE40S3L03A | PE40S3L05A | PE40S3L10A | PE40S3L15A | PE40S3L25A |
| - | - | PE50S3L03A | PE50S3L05A | PE50S3L10A | PE50S3L15A | PE50S3L25A |
| PE30S4L01A | PE30S4L02A | PE30S4L03A | - | - | - | - |
| - | PE40S4L02A | PE40S4L03A | PE40S4L05A | PE40S4L10A | PE40S4L15A | PE40S4L25A |
| - | - | PE50S4L03A | PE50S4L05A | PE50S4L10A | PE50S4L15A | PE50S4L25A |

* Advance position holds pressure with motor running or off. Return retracts cylinder with motor off

** Advance position holds pressure with motor running and cylinder Returns when button is released.

### Electric Pumps

Electric Power Source

PE30 Series

3/8"-18 NPTF

C

F	5/8”-18UNF

E  D

G	H

B	A

PE50M4N03A

PE40/PE50 Series

Pump Accessories

See the assortment

of accessories designed to work with these pumps.

pg 69

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

C

F	5/8”-18UNF

E

D

G

B

3/8"-18 NPTF

H A

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

PE40/PE50 Series With Casters

methods of operation. Always be prepared.

pg 92

3/8"-18 NPTF

Pressure Settings

Custom pressure settings available upon request. Contact your local

BVA dealer.

C

F E D

5/8”-18UNF

Reservoir Sizing

Recommended reservoir sizing

is Extended Cylinder + Hose	G

B

Volume Oil Capacity x 1.3	H

A

| Reservoir Size | Dimensional Table (in) |  |  |  |  |  |  |  | Pump Weight by Series* (lbs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | PE30 | PE40 | PE50 |
| 1 Gal | 12.7 | 10.3 | 19.1 | 4.7 | 5.7 | 12.0 | 8.1 | 12.4 | 61.0 | - | - |
| 2 Gal | 16.7 | 12.0 | 17.9 | 6.0 | 6.9 | 13.2 | 4.5 | 12.4 | 75.0 | 72.0 | - |
| 3 Gal | 16.7 | 12.0 | 19.7 | 7.2 | 8.2 | 10.5 | 4.5 | 12.4 | 84.0 | 81.0 | 91.0 |
| 5 Gal | 16.7 | 12.0 | 24.8 | 12.3 | 13.2 | 15.5 | 4.5 | 12.4 | - | 102.0 | 112.0 |
| 10 Gal | 20.9 | 16.5 | 31.5 | 19.0 | 19.9 | 22.3 | 8.3 | 16.5 | - | 184.0 | 194.0 |
| 15 Gal | 20.9 | 16.5 | 34.6 | 22.1 | 23.1 | 25.4 | 8.3 | 16.5 | - | 255.0 | 265.0 |
| 25 Gal | 25.6 | 21.3 | 34.6 | 22.1 | 23.1 | 25.4 | 10.6 | 21.3 | - | 362.0 | 372.0 |

1-888-332-6419

54	www.BVAhydraulics.com

- Pump weights include oil.

Single Phase Pump Matrix Chart

If you don’t see a pump that would fit your application needs, you can custom build a pump to your desired specifications. Use this Pump Matrix Chart as a guide for creating your very own custom built Electric Pump. For assistance in building your pump, call your local BVA Representative.

G

A

A

02

S4L

30

E

P

1

Product

Type

2

Motor

Type

3

Flow

Group

4

Valve Type

/Operation

5

Reservoir

Size

6	7

Voltage	Pump

Options

- - Product Type

P	Pump

- - Motor Type

E	Induction Electric Motor, TEFC

Pumps

- Flow Group

30	293/18 in3/min @ 350/10,000 psi, 0.5 Hp

40	311/37 in3/min @ 700/10,000 psi, 1.0 Hp

50	476/61 in3/min @ 700/10,000 psi, 1.5 Hp

- - Valve Type/Operation

CMR WMR

DMP DSP H2S

M2N M3N M4N M3L M4L W2N W3N W4N W3L W4L

S3N S4N S3L S4L

Pump Mounted Manifold Block - Pressure & Return Ports Only CMR Block w/10ft Motor Control Pendant

Dump Valve w/motor pendant 2W/2P

Dump Solenoid Valve w/pendant 3W/2P (Spool Type Valve)

Solenoid Valve 2W/2P - Pendant Control On/Off Only (Poppet Type Valve)

Manual Valve 3W/2P Manual Valve 3W/3P Manual Valve 4W/3P

Manual Valve 3W/3P w/Locking Plate Manual Valve 4W/3P w/Locking Plate

Manual Valve 3W/2P w/10ft Motor Control Pendant Manual Valve 3W/3P w/10ft Motor Control Pendant Manual Valve 4W/3P w/10ft Motor Control Pendant

Manual Valve 3W/3P w/Locking Plate and 10ft Motor Control Pendant Manual Valve 4W/3P w/Locking Plate and 10ft Motor Control Pendant

Solenoid Valve 3W/3P (Spool Type) Solenoid Valve 4W/3P (Spool Type)

Solenoid Valve 3W/3P w/Locking Plate (Spool Type Valve) Solenoid Valve 4W/3P w/Locking Plate (Spool Type Valve)

PE Series

Ordering Example

PE30S4L02A-A

After reviewing this model number, we know that the pump has the

following specifications:

- Electric power source

- ½ Hp flow group

- Solenoid Valve w/Locking Plate 4W/3P

- 2 gallon reservoir tank

- External Pressure Adjustment Block

PE30S4L02A-A

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg

92

Related Product: Hoses

We offer a variety of hoses, fittings, lengths and materials.

pg

72

55

- Reservoir Size

- 1 Gallon

- 2 Gallons

- 3 Gallons

05	5 Gallons

10	10 Gallons

15	15 Gallons

25	25 Gallons

- - Voltage

A	120V 1 ph 60 Hz - US Plug NEMA 5-15P

- 230V 1 ph 50 Hz - European Plug

- 240V 1 ph 60 Hz - No Plug

- - Pump Options

A	External Pressure Adjustment Block (available on any models with 3W/3P or 4W/3P)

G	Gauge w/Gauge Adapter 0-15,000 psi 2-1/2” Wet - Installed “A” Port

M	Intermittent Motor Option (available on solenoid equipped models only EXCEPT H2S option)

P	Pressure Switch (available on solenoid equipped models only)

U	Foot Control Switch (available on solenoid equipped models only)

W	Wireless Control (available on solenoid equipped models only)

Not all options are available in all configurations. Table is for reference only. Please consult your

local BVA dealer if you want to order a custom pump or if you have questions.

3-Phase Electric Pumps

Electric Power Source

PE40DSP02G

PE60S4L06G

Pressure Settings

Custom pressure settings available upon request. Contact your local

BVA dealer.

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

  - All hand pendants are wired with 24 VAC

  - Quiet operation meets OSHA regulations (80dBA)

  - 2 speed operation allows for faster cylinder advance

  - Full load starting capacities

  - Internal suction strainer keeps contamination from damaging pump

  - Rugged durable design meets the demands of industrial applications

  - Precision components to minimize wear and improve overall dependability

  - Motors are rated 50/60 Hz for voltage

  - Reservoirs equipped with sight level gauges

  - PE40 models are standard as 240 VAC 60 Hz

  - PE60 Models are standard as 480 VAC 60 Hz

| methods of operation. 92 Always be prepared. pg |  | Use with Cylinder Type | Flow Rate |  |  |  | Motor Size*** |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st Stage |  | 2nd Stage |  |  |
|  |  |  | in3/min | psi | in3/min | psi |  |
| Dump Solenoid Valve w/10 ft. Pendant |  |  |  |  |  |  |  |
|  | 2W/2P Advance/Return (Auto*) |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |
|  | 3W/2P Advance/Return** |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |
| Manual Valve |  |  |  |  |  |  |  |
|  | 3W/3P Advance/Hold Return |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |
|  | 4W/3P Advance/Hold/Return |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |
| Solenoid Valve w/10 ft. Pendant & Locking Plate |  |  |  |  |  |  |  |
|  | 3W/3P Advance/Hold Return |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |
|  | 4W/3P Advance/Hold/Return |  | 311 | 700 | 37 | 10,000 | 1.0 Hp |
|  |  |  | 592 | 700 | 110 | 10,000 | 3.0 Hp |

1-888-332-6419

56	www.BVAhydraulics.com

- Advance position holds pressure with motor running or off. Return retracts cylinder with motor off

** Advance position holds pressure with motor running and cylinder Returns when button is released.

*** Standard voltage on the 1 Hp pump is 208-240 VAC 60 Hz; 3 Hp pump is 460-480 VAC 60 Hz

PE Series

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Additional Voltages

Additional voltage options are available, see part number

matrix for options

pg

59

57

Pumps

40 Gallon Reservoir Features

4 Point Lifting Eyes

PE60S4L40J

The 40-Gallon Reservoir is fabricated with a baffle separating return oil from suction oil. Comes standard with an aluminum sight and temperature gauge.

| Usable Oil Capacity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 Gal | 3 Gal | 5 Gal | 6 Gal | 10 Gal | 15 Gal | 25 Gal | 40 Gal |
| Model Number |  |  |  |  |  |  |  |
| Dump Solenoid Valve w/10 ft. Pendant |  |  |  |  |  |  |  |
| PE40H2S02G | PE40H2S03G | PE40H2S05G | - | PE40H2S10G | PE40H2S15G | PE40H2S25G | - |
| - | - | - | PE60H2S06J | PE60H2S10J | PE60H2S15J | - | PE60H2S40J |
| PE40DSP02G | PE40DSP03G | PE40DSP05G | - | PE40DSP10G | PE40DSP15G | PE40DSP25G | - |
| - | - | - | PE60DSP06J | PE60DSP10J | PE60DSP15J | - | PE60DSP40J |
| Manual Valve |  |  |  |  |  |  |  |
| PE40M3N02G | PE40M3N03G | PE40M3N05G | - | PE40M3N10G | PE40M3N15G | PE40M3N25G | - |
| - | - | - | PE60M3N06J | PE60M3N10J | PE60M3N15J | - | PE60M3N40J |
| PE40M4N02G | PE40M4N03G | PE40M4N05G | - | PE40M4N10G | PE40M4N15G | PE40M4N25G | - |
| - | - | - | PE60M4N06J | PE60M4N10J | PE60M4N15J | - | PE60M4N40J |
| Solenoid Valve w/10 ft. Pendant & Locking Plate |  |  |  |  |  |  |  |
| PE40S3L02G | PE40S3L03G | PE40S3L05G | - | PE40S3L10G | PE40S3L15G | PE40S3L25G | - |
| - | - | - | PE60S3L06J | PE60S3L10J | PE60S3L15J | - | PE60S3L40J |
| PE40S4L02G | PE40S4L03G | PE40S4L05G | - | PE40S4L10G | PE40S4L15G | PE40S4L25G | - |
| - | - | - | PE60S4L06J | PE60S4L10J | PE60S4L15J | - | PE60S4L40J |

* Advance position holds pressure with motor running or off. Return retracts cylinder with motor off

** Advance position holds pressure with motor running and cylinder Returns when button is released.

*** Standard voltage on the 1 Hp pump is 208-240 VAC 60 Hz; 3 Hp pump is 460-480 VAC 60 Hz

### 3-Phase Electric Pumps

Electric Power Source

PE60S4L40J	C

PE60M4N06G	F

D

Safety Practices

Good industry practice	B

recommends not exceeding	 	B

80% of maximum rated capacities of all our products.

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69	C

F

Pressure Settings

H

Custom pressure settings available upon request.

Contact your local	 	A		A BVA dealer.

| Reservoir Size | Dimensional Table (in.) |  |  |  |  |  | Pump Weight by Series* (lbs.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | F | H | PE40 |
| 2 Gal | 16.65 | 12.01 | 21.00 | 5.98 | 13.16 | 12.36 | 72.0 |
| 3 Gal | 16.65 | 12.01 | 22.25 | 7.24 | 10.51 | 12.36 | 81.0 |
| 5 Gal | 16.65 | 12.01 | 27.25 | 12.28 | 15.53 | 12.36 | 102.0 |
| 10 Gal | 20.87 | 16.54 | 28.25 | 18.98 | 22.25 | 16.54 | 184.0 |
| 15 Gal | 20.87 | 16.54 | 37.25 | 22.13 | 25.40 | 16.54 | 255.0 |
| 25 Gal | 25.59 | 21.26 | 39.25 | 22.13 | 25.40 | 21.26 | 362.0 |

| Reservoir Size | Dimensional Table (in.) |  |  |  |  |  | Pump Weight by Series* (lbs.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | F | H | PE60 |
| 6 Gal | 20.63 | 14.80 | 22.95 | 8.82 | 16.04 | 16.87 | 187.0 |
| 10 Gal | 27.13 | 19.53 | 24.82 | 11.75 | 17.91 | 23.13 | 348.0 |
| 15 Gal | 27.13 | 23.46 | 24.82 | 11.75 | 17.91 | 23.13 | 396.0 |
| 40 Gal | 34.50 | 23.13 | 38.58 | 24.00 | 31.67 | - | 628.0 |

1-888-332-6419

58	www.BVAhydraulics.com

  - Pump weights include oil.

3-Phase Pump Matrix Chart

If you don’t see a pump that would fit your application needs, you can custom build a pump to your desired specifications. Use this Pump Matrix Chart as a guide for creating your very own custom built Electric Pump. For assistance in building your pump, call your local BVA Representative.

# P	E	40	DSP	02	G	A	G

1

Product

Type

2

Motor

Type

3

Flow

Group

4

Valve Type

/Operation

5

Reservoir

Size

6	7

Voltage	Pump

Options

1 - Product Type

P	Pump

- - Motor Type

E	Induction Electric Motor, TEFC

Pumps

- Flow Group

40	311/37 in3/min @ 700/10,000 psi, 1.0 Hp

60	592/110 in3/min @ 700/10,000 psi, 3.0 Hp

- - Valve Type/Operation

CMR

DSP H2S

M2N M3N M4N M3L M4L

S3N S4N S3L S4L

Pump Mounted Manifold Block - Pressure & Return Ports Only

Dump Solenoid Valve w/pendant 3W/2P (Spool Type Valve)

Solenoid Valve 2W/2P - Pendant Control On/Off Only (Poppet Type Valve)

Manual Valve 3W/2P Manual Valve 3W/3P Manual Valve 4W/3P

Manual Valve 3W/3P w/Locking Plate Manual Valve 4W/3P w/Locking Plate

Solenoid Valve 3W/3P (Spool Type) Solenoid Valve 4W/3P (Spool Type)

Solenoid Valve 3W/3P w/Locking Plate (Spool Type Valve) Solenoid Valve 4W/3P w/Locking Plate (Spool Type Valve)

PE Series

Ordering Example

PE40DSP02G

After reviewing this model number, we know that the pump has the

following specifications:

- Electric power source

- 1 Hp flow group

- Dump Solenoid Valve w/Pendant 3W/2P

- 2 gallon reservoir tank

- 208-240V 3 ph 60 Hz Voltage - No Plug

PE40DSP02G

Pump Accessories

See the assortment

of accessories designed

to work with these pumps.

pg 69

Safety Instructions

Visit our DO’S and DON’TS section to review the best methods of operation.

Always be prepared.

pg 92

Related Product: Hoses

We offer a variety of hoses, fittings, lengths and materials.

pg

72

59

- Reservoir Size

- 2 Gallon

- 3 Gallons

- 5 Gallons

- 6 Gallons

10	10 Gallons

15	15 Gallons

25	25 Gallons

40	40 Gallons

6 - Voltage

G	208-240V 3 ph 60 Hz - No Plug

W	380-415V 3 ph 50 Hz - No Plug

J	460-480V 3 ph 60 Hz - No Plug or Wire

7 - Pump Options

A	External Pressure Adjustment Block (available on any models with 3W/3P or 4W/3P)

G	Gauge w/Gauge Adapter 0-15,000 psi 2-1/2” Wet - Installed “A” Port

P	Pressure Switch (available on solenoid equipped models only)

U	Foot Control Switch (available on solenoid equipped models only)

W	Wireless Control (available on solenoid equipped models only)

Not all options are available in all configurations. Table is for reference only. Please consult

your local BVA dealer if you want to order a custom pump or if you have questions.

Gas and Diesel Pumps

PG/PD Series - Portable Power

PG70M4N05

PD80S4L05

PG90S4L10

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Gas Pumps	Diesel Pumps

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg 92

Related Product: Gauges

Reduce the risk of overloading your product by using a gauge.

A variety of graduations and types to suit any need.

pg69

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths

    - Gas engine pumps will provide 10,000 psi at remote locations

    - Industrial grade 5.5 or 10.2 hp Honda Gas Engine

    - Ideal for applications at remote locations

    - Two speed pump designed for rapid cylinder advance

    - Gas pump features a sturdy protective roll cage for use in the toughest environments

    - Manual control valve for use with single or

      - Diesel engine pumps will provide 10,000 psi at remote locations

      - Industrial grade 7.3 hp HATZ Diesel engine

      - Locking solenoid control valve for use with single or double acting cylinders or tools

      - Two speed pump designed for rapid cylinder advance

      - Pump features a sturdy protective roll cage for use in the toughest environments

      - Ideal for application when diesel fuel in

and materials.

pg 72

double acting cylinders or tools

      - Optional electric start

remote environments is required

        - Comes standard with electric start

| Valve Function | Valve Type | Use with Cylinder Type | Flow Rate |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st Stage |  | 2nd Stage |  |
|  |  |  | in3/min | psi | in3/min | psi |
| 3W/3P Advance/Hold Return | Manual |  | 427 | 700 | 100 | 10,000 |
|  | Solenoid |  | 600 | 700 | 200 | 10,000 |
|  | Solenoid |  | 600 | 700 | 200 | 10,000 |
| 4W/3P Advance/Hold Return | Manual |  | 427 | 700 | 100 | 10,000 |
|  | Solenoid |  | 600 | 700 | 200 | 10,000 |
|  | Solenoid |  | 600 | 700 | 200 | 10,000 |

1-888-332-6419

60	www.BVAhydraulics.com

Gas/Diesel Pump Matrix Chart

If you don’t see a pump that would fit your application needs, you can custom build a pump to your desired specifications. Use this Pump Matrix Chart as a guide for creating your very own custom gas/diesel pump. For assistance in building your pump, call your local BVA Representative.

P

G

90

S4L

10

A

G

1

Product

Type

2

Motor

Type

3

Flow

Group

4*

Valve Type

/Operation

5

Reservoir

Size

7

Pump

Options

1 - Product Type

P	Pump

- - Motor Type

G	Gas Motor

D	Diesel Motor

Pumps

- Flow Group

70	427/100 in3/min @ 700/10,000 psi, 5.5 Hp

80	600/200 in3/min @ 700/10,000 psi, 7.3 Hp

90	1070/200 in3/min @ 700/10,000 psi, 10.2 Hp

4* - Valve Type/Operation

CMR

DSP H2S

M2N M3N M4N M3L M4L

S3N S4N S3L S4L

Pump Mounted Manifold Block - Pressure & Return Ports Only

Dump Solenoid Valve w/pendant 3W/2P (Spool Type Valve)

Solenoid Valve 2W/2P - Pendant Control On/Off Only (Poppet Type Valve)

Manual Valve 3W/2P Manual Valve 3W/3P Manual Valve 4W/3P

Manual Valve 3W/3P w/Locking Plate Manual Valve 4W/3P w/Locking Plate

Solenoid Valve 3W/3P (Spool Type) Solenoid Valve 4W/3P (Spool Type)

Solenoid Valve 3W/3P w/Locking Plate (Spool Type Valve) Solenoid Valve 4W/3P w/Locking Plate (Spool Type Valve)

PG/PD Series

Ordering Example

PG90S4L10

After reviewing this model number, we know that the pump has the

following specifications:

- Gasoline power source

- 10.2 Hp flow group

- Solenoid Valve w/Locking Plate 4W/3P

- 10 gallon reservoir tank

PG90S4L10

Roll Cages

All gas and diesel pumps are equipped with roll cages to prevent any damage that might occur on a job site.

Wireless Controller

PG90 and PD80 pumps have an optional wireless controller. Visit the gas/diesel pump matrix and pump accessories page

for more details.

pg 71

Optional Electric Start

PG90 has an optional electric start.

PD80 comes standard with an electric start.

Refer to the Gas/Diesel Pump Matrix.

61

5 - Reservoir Size

5	5 Gallons

10	10 Gallons

15	15 Gallons

25	25 Gallons

40	40 Gallons

6 - Pump Options

A	External Pressure Adjustment Block (available on any models with 3W/3P or 4W/3P)

G	Gauge w/Gauge Adapter 0-15,000 psi 2-1/2” Wet - Installed “A” Port

P	Pressure Switch (available on solenoid equipped models only)

E	Foot Control Switch (available on solenoid equipped models only)

W	Wireless Control (available on solenoid equipped models only)

* Solenoid options are not available on the PG70 Series Gas Pumps

Not all options are available in all configurations. Table is for reference only. Please consult

your local BVA dealer if you want to order a custom pump or if you have questions.

| Motor HP / Type | Usable Oil Capacity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5 Gallons | 10 Gallons | 15 Gallons | 40 Gallons |
|  | Model Number | Model Number | Model Number | Model Number |
| Gas - 5.5 Hp Honda | PG70M3N05 | - | - | - |
| Gas - 10.2 Hp Honda | PG90S3L05 | PG90S3L10 | PG90S3L15 | PG90S3L40 |
| Diesel - 7.3 Hp HATZ | PD80S3L05 | PD80S3L10 | PD80S3L15 | PD80S3L40 |
| Gas - 5.5 Hp Honda | PG70M4N05 | - | - | - |
| Gas - 10.2 Hp Honda | PG90S4L05 | PG90S4L10 | PG90S4L15 | PG90S4L40 |
| Diesel - 7.3 Hp HATZ | PD80S4L05 | PD80S4L10 | PD80S4L15 | PD80S4L40 |

Gas and Diesel Pumps

PG70/PG90/PD80 Series

PD80S4L05

PG90M4N10

Safety Practices	H

Good industry practice recommends not exceeding 80% of maximum rated

capacities of all our products.	G

F    E

Related Product: Gauges

D

Reduce the risk of overloading	B

your product by using a gauge.	 	 A variety of graduations and

types to suit any need.

C	 	A

pg69

Safety Instructions

Visit our DO’S and DON’TS

section to review the best	H

methods of operation. Always be prepared.

pg 92

G

F   E

Related Product: Hoses

We offer a variety of

hoses, fittings, lengths	 	D	B

and materials.

pg 72

C	A

| Model Number* | Dimensions (in) |  |  |  |  |  |  |  | Weight** (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H |  |
| Gas Pumps |  |  |  |  |  |  |  |  |  |
| PG70xxx05 | 26.57 | 21.50 | 20.94 | 10.47 | 11.50 | 12.43 | 15.47 | 24.70 | 162 |
| PG90xxx10 | 28.25 | 23.37 | 27.00 | 13.50 | 12.00 | 12.93 | 17.38 | 24.70 | 303 |
| PG90xxx15 | 28.25 | 23.37 | 27.00 | 13.50 | 12.00 | 12.93 | 17.38 | 24.70 | 372 |
| Diesel Pumps |  |  |  |  |  |  |  |  |  |
| PD80xxx05 | 26.57 | 21.50 | 20.94 | 10.47 | 11.50 | 12.43 | 15.47 | 32.60 | 255 |
| PD80xxx10 | 28.25 | 23.37 | 27.00 | 13.5 | 12.00 | 12.93 | 17.38 | 32.60 | 329 |
| PD80xxx15 | 28.25 | 26.00 | 27.00 | 13.5 | 12.00 | 12.93 | 17.38 | 32.60 | 403 |

###### 1-888-332-6419

62	www.BVAhydraulics.com

- “xxx” in the model number refers to the type of valve you may use on the gas/diesel engine pump. Refer to your local BVA dealer or the Matrix Chart (on page 61) to build your own pump.

** Pump weights include oil.

### Universal Motor Pumps

PU Series - Light-Weight Pumps

  - Portable, light weight and compact design

  - Universal Motor, starts under full load and is available in either 120VAC 50/60 HZ or 240VAC 50/60HZ

  - 10ft 24 VAC pendant control for safety

  - Weighs only 37 lbs. with oil

  - External pressure adjustment is available

  - Designed for intermittent duty operation

  - Gauge port on control valve

PU20M4N01B

PU20DMP01B

Pumps

Hydraulic Gear Pullers

5, 10, 15, 25, 50 & 100 Ton Hydraulic Gear Pullers come with a safety cage design help

keep the jaws tight to the gear and prevent slippage that can cause injury. Pair these with BVA’s PU20M3N01B Lightweight

e-pump for portability.

pg 83

12.99”

1/4”-NPTF

1/4”-NPTF

1/4”-NPTF

Hydraulic Nut Splitters

Available in 3 models, each nut splitter can

split a wide range of nut sizes thereby

reducing the number of splitters needed. The BVA PU20DMP01B dump style pump works well with

these splitters

pg 83

8.07”

3/8”-18NPTF

7.95”

5.51”

4.45”

5.59”

Flange Spreaders

BVA’s Pin Type Flange Spreaders are

available in both 5 & 10 Ton models and well suited for most environments. For portability and speed the PU20M3N01B would be an ideal choice. pg 81

HGP-1 Pump Kit

HGP-1 Pump Kit contains: PU20M3N01B, CF3814, GW2514, CS3838C, and CH38M

12.44”

14.25”

| Valve Function | Flow Rate |  |  |  | Motor Size | Voltage 50/60 Hz |  | Weight (lbs) | Length (in) | Width (in) | Height (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1st Stage |  | 2nd Stage |  |  | 120 VAC | 240 VAC |  |  |  |  |
|  | in3/min | psi | in3/min | psi |  | Model # | Model # |  |  |  |  |
| 2W/2P Advance/Return | 133 | 200 | 28 | 10,000 | 1.0 Hp | PU20DMP01B | PU20DMP01C | 37.0 | 12.44 | 8.07 | 11.81 |
| 3W/3P Advance/Hold Return | 133 | 200 | 28 | 10,000 | 1.0 Hp | PU20M3N01B | PU20M3N01C | 37.0 | 14.25 | 8.07 | 11.81 |
| 4W/3P Advance/Hold/Return | 133 | 200 | 28 | 10,000 | 1.0 Hp | PU20M4N01B | PU20M4N01C | 37.0 | 14.25 | 8.07 | 11.81 |

63

### Combinations

Pump and Cylinder Sets

Each set includes:

  - ¼" ID 6 foot Hydraulic Hose

  - ⅜" High Flow Male Coupler

  - Adapter (SP3 Models Only)

  - 2½" Face Diameter Wet Gauge

| Single Acting Cylinder Type | Cylinder |  |  |  | Hand Pump P350 | Hand Pump P1000 | Hand Pump P2301 | Air Pump PA1500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity (Ton) | Model Number | Stroke (in) | Collapsed Height (in) |  |  |  |  |
| General Purpose Cylinders | 5 | H0505 | 5.08 | 8.70 | SP3-0505 | - | - | - |
|  | 10 | H1002 | 2.01 | 4.84 | - | SP10-1002 | - | SA15-1002 |
|  |  | H1004 | 4.02 | 6.85 | - | SP10-1004 | - | SA15-1004 |
|  |  | H1006 | 5.94 | 9.84 | - | SP10-1006 | - | SA15-1006 |
|  |  | HT1006 | 5.95 | 11.46 | - | SP10-1006T | - | SA15-1006T |
|  |  | H1010 | 9.96 | 13.86 | - | SP10-1010 | - | SA15-1010 |
|  |  | HT1010 | 9.96 | 15.47 | - | SP10-1010T | - | SA15-1010T |
|  | 15 | H1504 | 3.98 | 7.87 | - | SP10-1504 | - | SA15-1504 |
|  |  | H1506 | 5.98 | 10.63 | - | SP10-1506 | - | SA15-1506 |
|  | 25 | H2502 | 2.01 | 6.61 | - | SP10-2502 | - | SA15-2502 |
|  |  | H2504 | 4.02 | 8.62 | - | SP10-2504 | - | SA15-2504 |
|  |  | H2506 | 6.10 | 10.71 | - | SP10-2506 | - | SA15-2506 |
|  |  | H2508 | 8.11 | 12.72 | - | - | SP23-2508 | SA15-2508 |
|  |  | H2514 | 14.25 | 18.74 | - | - | SP23-2514 | SA15-2514 |
|  | 55 | H5506 | 6.26 | 11.14 | - | - | SP23-5506 | SA15-5506 |
|  | 100 | H10002A | 2.01 | 8.62 | - | - | SP23-10002A | - |
| Flat Body Cylinders | 5 | HF0503* | 0.24 | 1.26 | SP3-0503F | - | - | - |
|  |  | HF0503B * | 0.24 | 1.26 | SP3-0503B | - | - | - |
|  | 10 | HF1005 | 0.43 | 1.69 | SP3-1005F | - | - | - |
|  |  | HF1005B | 0.43 | 1.69 | SP3-1005B | - | - | - |
|  | 20 | HF2005 | 0.43 | 2.01 | SP3-2005F | - | - | - |
|  |  | HF2005B | 0.43 | 2.01 | SP3-2005B | - | - | - |
|  | 30 | HF3005 | 0.51 | 2.31 | SP3-3005F | - | - | - |
|  |  | HF3005B | 0.51 | 2.31 | SP3-3005B | - | - | - |
|  | 50 | HF5006 | 0.63 | 2.64 | SP3-5006F | - | - | - |
|  |  | HF5006B | 0.63 | 2.64 | SP3-5006B | - | - | - |
|  | 75 | HF7506 | 0.63 | 3.13 | SP3-7506F | - | - | - |
|  | 100 | HF10006 | 0.63 | 3.37 | SP3-10006F | - | - | - |
|  | 150 | HF15006 | 0.63 | 3.94 | - | SP10-15006F | - | - |

1-888-332-6419

64	www.BVAhydraulics.com

- HF0503 comes with CR38M coupler

** HC1202 comes with CRZ14M coupler

| Single Acting Cylinder Type | Cylinder |  |  |  | Hand Pump P350 | Hand Pump P1000 | Hand Pump P2301 | Air Pump PA1500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity (Ton) | Model | Stroke (in) | Collapsed Height (in) |  |  |  |  |
| Hollow Hole Cylinders | 12 | HC1201T** | 0.31 | 2.36 | SP3-1201T | - | - | - |
|  |  | HC1202T** | 1.61 | 4.74 | SP3-1202T | - | - | - |
|  |  | HC1202XT** | 1.61 | 4.74 | SP3-1202XT | - | - | - |
|  |  | HC1203XT | 2.99 | 7.24 | - | SP10-1203XT | - | - |
|  | 20 | HC2002T | 2.01 | 6.38 | - | SP10-2002T | - | SA15-2002T |
|  | 30 | HC3002T | 2.52 | 7.05 | - | SP10-3002T | - | - |
|  |  | HC3006T | 6.11 | 13.02 | - | SP10-3006T | - | SA15-3006T |
|  | 60 | HC6003T | 3.00 | 9.77 | - | SP10-6003T | - | SA15-6003T |
|  |  | HC6006T | 6.00 | 12.76 | - | - | SP23-6006T | SA15-6006T |
|  | 100 | HC10003T | 3.00 | 10.00 | - | - | SP23-10003T | SA15-10003T |
| Low Profile Cylinders | 10 | HL1001 | 1.50 | 3.48 | SP3-1001L | SP10-1001L | - | SA15-1001L |
|  | 20 | HL2002 | 1.75 | 3.90 | SP3-2002L | SP10-2002L | - | SA15-2002L |
|  | 30 | HL3002 | 2.44 | 4.61 | SP3-3002L | SP10-3002L | - | SA15-3002L |
|  | 50 | HL5002 | 2.36 | 4.80 | - | SP10-5002L | - | SA15-5002L |
|  | 60 | HL6003KK | 2.50 | 5.63 | - | SP10-6003L | SP23-6003L | SA15-6003L |
|  | 100 | HL10002 | 2.24 | 5.55 | - | SP10-10002L | SP23-10002L | SA15-10002L |
| Lock Nut Cylinders | 55 | HLN5502 | 1.97 | 6.50 | - | - | SP23-5502LN | SA15-5502LN |
|  |  | HLN5504 | 3.94 | 8.40 | - | - | SP23-5504LN | SA15-5504LN |
|  |  | HLN5506 | 5.91 | 10.40 | - | - | SP23-5506LN | SA15-5506LN |
| Aluminum Cylinders | 30 | HU3002T | 2.00 | 6.50 | SP3-3002U | - | - | SA15-3002U |
|  |  | HU3006T | 6.00 | 10.50 | - | SP10-3006U | - | SA15-3006U |
|  | 50 | HU5006T | 6.00 | 10.50 | - | - | SP23-5006U | SA15-5006U |

* HF0503 comes with CR38M coupler

** HC1202 comes with CRZ14M coupler

We see here our combo kit SA15-3002L getting set up to lift a load. This combo kit includes a HL3002 cylinder, PA1500 air pump, CS3814 hose/CH38M coupler set, CF3814 gauge adapter and a GW2514 pressure gauge.

### Combinations

Combination Kits

Pump and Cylinder Sets

65

### Flow Control Valves

10,000 PSI Rated

3.48”

2.52”

2.52”

1.26”

1.45”

CVN1 and CVN3

3.88”

3/8” - 18NPTF

1.45”

CVC3

2.77”

2.96”

1.36”

1.50”

2.01”

Ø.32”

TYP OF 2

1.61”

0.76”

CVP3

3/8”-18 NPTF

| Valve Type | Model Number | Ports | Applications | Hydraulic Symbol |
| --- | --- | --- | --- | --- |
| Needle Valve | CVN1 | ¼"-18NPTF | Controls cylinder speed and acts as temporary shut-off valve for load holding. |  |
|  | CVN3 | "-18NPTF |  |  |
| Check Valve | CVC3 | "-18NPTF | Allows flow in one direction only. |  |
| Pilot Operated Check Valve | CVP3 | "-18NPTF | Mounted in line to a cylinder which will hold pressure in the event of pressure loss in the system. Most often used on double-acting cylinders with pilot port, receiving pressure from a tee off of the cylinder retracting line. Pilot pressure ratio 15% or 6.5:1 |  |

1-888-332-6419

66	www.BVAhydraulics.com

Flow Control Valves

1.73"

4.53"

3/8"-18NPTF

7.44"

3/8"-18NPTF

3.05"

10,000 PSI Rated

Ø 1.5"

CVR3

CVM3

4.65"

3.94"

2.76"

2.28"

0.63"

0.63"

1.02"

4.79"

4.65"

2.76"

3.94"

2.28"

0.63"

0.63"

1.02"

4.79"

Control Valves / Accessories

CVMS3

| Valve Type | Model Number | Ports | Applications | Hydraulic Symbol |
| --- | --- | --- | --- | --- |
| Pressure Relief Valve | CVR3 | ”-18NPTF | Controls pressure developed by the pump in hydraulic loop. By-pass occurs when preset pressure reaches 0-10,000 PSI. Turns clockwise to increase pressure, counter-clockwise to decrease. |  |
| Manual Operated Check Valve | CVM3 | ”-18NPTF | Mounted in line to a cylinder and will hold pressure in the cylinder in the event of pressure loss in the system. Pressure is manually released. |  |
|  | CVMS3 | ”-18NPTF | Mounted in line to a cylinder and will hold pressure in the cylinder in the event of pressure loss in the system. This model is equipped with high-pressure bypass. Pressure is manually released. |  |

67

Manifolds

10,000 PSI Rated

1.5"

1/4" - 20 UNC

  - " Depth (x2)

0.82"

2"	1.63"

CMH6

3/8"-18NPTF (x6)

8"

CM7

3/8"-18NPTF (x7) 0.625"

1.25"

2.24"

0.625"

0.25"

2"	3"	2”

Ø¼" (x2)

0.625"

1.25"

1.5"

1.5"

1.5"

1.25"

CM14

3/8"-18NPTF (x7)

2.24"

0.625"

3.25"

0.25"

15.48"

8"

3.25"

Ø 1/4" (x2)

0.625"

1.25"

0.625"

1.25"	4"	4"	4"	1.25"

MFC4

Ø 0.28"

MFC2

Ø 0.28"

0.5"

0.5"

0.5"

11"

0.5"	5"

3/8”-18NPTF (x5)

1.5"

1.62"

2.61"

3"

12"

3.86"

4.13"

1"

3"

3"

FCMH6

2.61"

3/8”-18NPTF (x3)

3.86"

4.13"

| 1.625" |  |  |
| --- | --- | --- |

1"

1.5"	3"

6"

FCMH6

    - Controls the flow of (5) hydraulic hoses from one central location

    - Sturdy design for long product life

    - Convenient carry handle

| Manifold Type | Model Number | Number of Ports | Port Size | Mounting Holes |
| --- | --- | --- | --- | --- |
| 6-Port Hexagon Manifold | CMH6 | 6 | ⅜″-18NPTF | ¼″-20UNC |
| 7″ Long Manifold | CM7 | 7 | ⅜″-18NPTF | ¼″-20UNC |
| 14″ Long Manifold | CM14 | 7 | ⅜″-18NPTF | ¼″-20UNC |
| Manifold with Two Needle Valves | MFC2 | 3 | ⅜″-18NPTF | 0.28” Dia. |
| Manifold with Four Needle Valves | MFC4 | 5 | ⅜″-18NPTF | 0.28” Dia. |
| Spider Valves | FCMH6 | 6 | ⅜″-18NPTF | - |

1-888-332-6419

68	www.BVAhydraulics.com

Hydraulic Pressure Gauge

Digital or Analog

GW4014

GW2514

GD2514

GE2514

GE2514 Back-Lit digital pressure gauge displays in: PSI, BAR, KG/CM2 and MPa

| Gauge Type | Model Number | Certified Gauge | Face Dia. (in) | Thread (in) | Material | Pressure Range |  | Graduation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Major Graduation (psi) | Minor Graduation (psi) | Major Graduation (bar) | Minor Graduation (bar) |
|  |  |  |  |  |  | (psi) | (bar) |  |  |  |  |
| Digital | GE2514 | - | 2.50 | 1/4"-18 NPTF | Stainless | 0 - 10,000 | 0 - 700 | - | - | - | - |
| Dry | GD2514 | - | 2.50 | 1/4"-18 NPTF | Brass | 0 - 15,000 | 0 - 1,034 | 2,000 | 200 | 100 | 20 |
|  | GD4014 | - | 4.00 | 1/4"-18 NPTF | Brass | 0 - 15,000 | 0 - 1,034 | 2,000 | 200 | 100 | 20 |
|  | GD2512 | - | 2.50 | 1/2"-14 NPTF | Brass | 0 - 10,000 | 0 - 700 | 2,000 | 200 | 100 | 20 |
|  | GD4012 | - | 4.00 | 1/2"-14 NPTF | Brass | 0 - 10,000 | 0 - 700 | 2,000 | 200 | 100 | 20 |
| Wet | GW2514 | - | 2.50 | 1/4"-18 NPTF | Brass | 0 - 15,000 | 0 - 1,034 | 2,000 | 200 | 100 | 20 |
|  | GW4014 | - | 4.00 | 1/4"-18 NPTF | Brass | 0 - 15,000 | 0 - 1,034 | 2,000 | 200 | 100 | 20 |
|  | GW4014-CERT | yes | 4.00 | 1/4"-18 NPTF | Stainless | 0 - 20,000 | 0 - 1,379 | 2,000 | 200 | 100 | 20 |
|  | GW2512 | - | 2.50 | 1/2"-14 NPTF | Brass | 0 - 10,000 | 0 - 700 | 2,000 | 200 | 100 | 20 |
|  | GW4012 | - | 4.00 | 1/2"-14 NPTF | Brass | 0 - 10,000 | 0 - 700 | 2,000 | 200 | 100 | 20 |

C

E

A

Control Valves / Accessories

B

Gauge Adapter

10,000 PSI Rated

D	F

CF3814

CF3812E

B

F

C

D		E

F

CF3812

B

C

A

| Model Number | Dimensions (in) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F |
| CF3814 | 5.12 | 1.38 | ¼″-18NPTF | ⅜″-18NPTF | ⅜″-18NPTF | 1.25 |
| CF3812 | 2.81 | 1.38 | ½"-14NPTF | ⅜"-18NPTF | ⅜"-18NPTF | 1.25 |
| CF3812E | 6.10 | 1.38 | ½"-14NPTF | ⅜"-18NPTF | ⅜"-18NPTF | 1.25 |

D	E

A

## 69

### Hydraulic Oil

ISO 46

| Volume (gal) | Model Number | Viscosity Index (min) | Viscosity @ 100°F (S.U.S.) | Viscosity @ 210°F (S.U.S.) | ASTM Grade | Flash Point (°F) | Pour Point (°F) | API Gravity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | F01 | 144 | 234 | 53 | 215 | 356 | -49 | 30.8 |
| 55 | F55 | 144 | 234 | 53 | 215 | 356 | -49 | 30.8 |

Recommended for all BVA cylinders and pumps

Coupler Dust Cover

- Keeps contaminants out of the system

| Model Number | Coupler Compatibility |
| --- | --- |
| CDC38 | CH38, CH38F, CH38M |

Solenoid Valves

Pump Mounted

- Pump mounted solenoid kits converts your manual pump to an electrically controlled pump.

CDC38

F01

A

B

3"

8.54"

7.24"

0.93"

A

PS33L

3/8-16UNC-2LX4

| Use With Cylinder | Model Number | Valve Type | Valve Function | Hydraulic Symbol | Schematic Flow path |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Acting | PS33L | Solenoid 115VAC | Advance/ Hold/Re- turn |  |  |  |  | 7.0 |
| Single Acting | PD33 | Solenoid 115VAC | Advance/ / Return |  |  |  | - | 6.0 |
| Double Acting | PS43L | Solenoid 115VAC | Advance/ Hold/Re- turn |  |  |  |  | 7.0 |

1-888-332-6419

70	www.BVAhydraulics.com

CH38

A

D 	G

E C	H D	C

### Hydraulic Couplers

10,000 PSI Rated

F      J	 	B

CFS38

CR38

A

G

E C 	H  D

A

D

C	E  C	H    D	C

F   J	 	B			J	G F

CH14 	A

B		 G

CRZ14	A

B

D

E C	H D	C

F J

E C	H    D	C

J	G

F

| Coupler Type | Model Numbers |  |  | Dimensions (in) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Complete Set | Male Half | Female Half | A* | B | C | D | E | F | G | H | J |
| High-Flow Coupler | CH38 | CH38M | CH38F | 3.35 | 2.84 | 3/8"-18NPTF | 1.10 | - | 1.00 | 0.75 | 0.63 | 0.63 |
| Flush-Face Coupler | CFS38 | CFS38M | CFS38F | 4.36 | 2.85 | 3/8"-18NPTF | 1.21 | 1.06 | 1.12 | 0.79 | 0.63 | 0.63 |
| Regular-Flow Coupler | CR38 | CR38M | CR38F | 3.15 | 2.20 | 3/8"-18NPTF | 1.04 | - | 0.73 | 0.50 | 0.53 | 0.53 |
| High-Flow Coupler | CH14 | CH14M | CH14F | 2.83 | 2.35 | 1/4"-18NPTF | 0.87 | - | 0.83 | 0.62 | 0.41 |  |
| Zinc Coupler | CRZ14 | CRZ14M | CRZ14F | 2.42 | 1.67 | 1/4"-18NPTF | 0.95 | 0.83 | 0.58 | 0.39 | 0.35 | 0.35 |

* Value A is total length when male and female couplers are connected.

Foot Pedals

FP2

Single or Double

- Hands free pump operation

Control Valves / Accessories

Can be used with all pumps equipped with solenoid valves

FP1

Foot controls with 10’ cord. This option replaces the hand controls on solenoid equipped models.

WR12K

Wireless Hand Held Control

  - Allows for increased mobility and safety

  - Nominal range of 325 feet (greater w/ unobstructed line of sight)

  - Batteries last 100 hours with continuous key press (2 AAA batteries)

  - For hands free pump operation

  - Contact your local BVA dealer for further information

External Pressure Adjustment

Sandwich Mounted

PW-AS1

| Model Number | Pressure Adj. Range (psi) | Interval (psi) | Suitable Model |
| --- | --- | --- | --- |
| PW-AS1 |  |  | PE Series, 1 Gal. Reservoir |
| PW-AS2 | 1,000 |  | PE Series, 2+ Gal. Reservoir |
|  | ~ | 50 |  |
| PW-AS3 |  |  | PU20 Series Pump |
|  | 10,000 |  |  |
| PW-AS4 |  |  | PU20DMP Series |

Pressure can be adjusted

externally (1,000-10,000 psi)

    - Works with single and double acting cylinders

71

CMR2

Pressure Switch

CVS1

Pump Mounted Manifold Block

CMR2

- Converts BVA pumps with mounted valves for use with remote mounted valves

- Comes with 4 mounting bolts

CVS1

Automatically shuts off electric pump when predetermined system pressure is reached

| Model Number | Adjustable Pressure Range | Electrical Rating | Port Size | Weight (lbs) |
| --- | --- | --- | --- | --- |
| CVS1 | 1,000~10,150 psi | 15 amp @ 250V max | ¼"-18NPTF | 1.32 |

Hydraulic Hoses

Rubber or Thermoplastic

  - All hoses are designed with a Maximum Working Pressure of 10,000 psi

  - Heavy Duty Rubber Hoses are made with 2 layers of steel braids and comply with the Material Handling Institutes IJ-100 standard for hoses and carries a 2:1 safety factor

  - Thermoplastic Hoses are made with 2 layers of steel braids with wrapped in a Polyurethane jacket and carries a 4:1 safety factor

| Hose ID (in) | Hose Length (ft) | Hose End 1 | Hose End 2 1/4"-18 NPTF 3/8"-18 NPTF CR38M Coupler CH38M Coupler |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rubber Thermoplastic Model Number |  | Rubber Thermoplastic Model Number |  | Rubber Thermoplastic Model Number |  | Rubber Thermoplastic Model Number |  |
| ¼” | 6 | ¼"-18 NPTF | CS1414 | CT1414 | - | - | - | - | CS1414M | CT1414M |
|  | 2 | ⅜"-18 NPTF | - | - | CS3814A | - | CS3814AMR | - | CS3814AM | - |
|  | 3 | ⅜”-18 NPTF | - | - | CS3814B | CT3814B | CS3814BRM | - | CS3814BM | - |
|  | 6 | ⅜”-18 NPTF | - | - | CS3814 | CT3814 | CS3814MR | - | CS3814M | CT3814M |
|  | 10 | ⅜”-18 NPTF | - | - | CS3814C | CT3814C | CS3814CRM | - | CS3814CM | CT3814CM |
|  | 20 | ⅜”-18 NPTF | - | - | CS3814D | CT3814D | CS3814DRM | - | CS3814DM | CT3814DM |
|  | 30 | ⅜”-18 NPTF | - | - | CS3814G | CT3814E | - | - | CS3814GM | CT3814EM |
| ⅜” | 6 | ⅜”-18 NPTF | - | - | CS3838 | CT3838 | CS3838MR | - | CS3838M | CT3838M |
|  | 10 | ⅜”-18 NPTF | - | - | CS3838C | CT3838C | - | - | CS3838CM | CT3838CM |
|  | 20 | ⅜”-18 NPTF | - | - | CS3838D | CT3838D | - | - | CS3838DM | CT3838DM |
|  | 30 | ⅜”-18 NPTF | - | - | CS3838E | CT3838E | - | - | CS3838EM | CT3838EM |
|  | 50 | ⅜”-18 NPTF | - | - | CS3838F | CT3838F | - | - | CS3838FM | CT3838FM |

1-888-332-6419

72	www.BVAhydraulics.com

PW2

2.97"

6.37"

### Pump Mounted Valves

For PE, PG and PD Pumps Only

PW33L

5.58"

4.10"

3/8-18NPTF

0.93"

6.23"	6.08"

1.75"

2.31"

4.63"

A

0.87"

6.13"

PW3 and PW4

5.66"

5.58"

PW43L

1.75"

3.84"

0.93"

B	A

3.25"

3/8-18NPTF

B	A

0.87"

6.23"	6.08"

Control Valves / Accessories

4.63"

| Valve Operation | Valve Type | Use w/ Cylinder | Model Number |  | Hydraulic Symbol | Schematic Flow path |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3-Way, 2-Position | Single Acting | PW2 |  |  |  |  | - | 4 |
|  | 3-way, |  |  |  |  |  |  |  |  |
|  | 3-Position, Tandem | Single Acting | PW3 |  |  |  |  |  | 4 |
|  | center |  |  |  |  |  |  |  |  |
|  | 4-Way, |  |  |  |  |  |  |  |  |
| Manual | 3-Position, Tandem | Double Acting | PW4 |  |  |  |  |  | 3 |
|  | center |  |  |  |  |  |  |  |  |
|  | 3-Way, | Single Acting |  |  |  |  |  |  |  |
|  | 3-Position, |  |  |  |  |  |  |  |  |
|  | Tandem |  | PW33L |  |  |  |  |  | 6 |
|  | center, |  |  |  |  |  |  |  |  |
|  | Locking |  |  |  |  |  |  |  |  |
|  | 4-Way, | Double Acting |  |  |  |  |  |  |  |
|  | 3-Position, |  |  |  |  |  |  |  |  |
|  | Tandem |  | PW43L |  |  |  |  |  | 6 |
|  | center, |  |  |  |  |  |  |  |  |
|  | Locking |  |  |  |  |  |  |  |  |

## 73

### Remote Mounted Directional Control Valves

Not to be used with Hand Pumps

PR33, PR33C	PR33L , PR33CL

PR43, PR43C

PR43CL

3.80"

4.49"

4.21"

4.49"

6.93"

6.93"

3/8"-18NPTF (x3)

6.93"

6.93"

3/8”-18NPTF (x3)

4.21"

6.16"

3/8”-18NPTF(x4)

4.21"

6.16"

1/4"-18NPTF (x4)

0.75"

1.38" 1/4”-20UNC (x4)

0.75"

1.38"

1/4"-20UNC (x4)

0.75"		T

1.38"

1/4”-20UNC(x4)

0.75"	 1.38"

1/4”-20UNC (x4)"

Valve Operation

Valve Type

Use w/ Cylinder

Model Number

Hydraulic Symbol

Schematic Flow path	Weight

(lbs)

3-Way,  3 Position,

Tandem center

PR33	3.5

3-Way,  3 Position,

Closed center

PR33C	3.5

3-Way,  3 Position,

Tandem center, Locking

Single Acting

PR33L	5.7

- Way,  3 Position,

Closed center, Locking

PR33CL	5.7

Manual

- Way,  3 Position,

Tandem center

PR43	3.5

4-Way,  3 Position,

Tandem center, Locking

PR43L	5.7

4-Way,  3 Position,

Closed center

Double Acting

PR43C	3.5

4-Way,  3 Position,

Closed center, Locking

PR43CL	5.7

1-888-332-6419

74	www.BVAhydraulics.com

All valves have a detent in the center position.

### Fittings

Control Valves / Accessories

10,000 PSI Rated

| Fitting Type |  | Model Number | Dimensions |  |  |  | Display Image |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A (in) | B (in) | C | D |  |  |  |  |  |
| Adapters |  |  |  |  |  |  |  |  |  |  |  |
|  | Adapter ½″-14NPTF - Female ⅜″-18NPTF - Male | FT108 | 1.69 | 1.14 | Male ⅜″-18NPTF | Female ½″-14NPTF | B |  | A D C |  |  |
|  | Adapter ½″-14NPTF - Female ¼″-18NPTF - Male | FT118 | 1.69 | 1.14 | Male ¼″-18NPTF | Female ½″-14NPTF |  |  |  |  |  |
|  | Fitting | CA3814 | 1.38 | 0.91 | Male ¼″-18NPTF | Female ⅜″-18NPTF | B | D | A |  | C |
| Bushings/Connectors |  |  |  |  |  |  |  |  |  |  |  |
|  | Bushing | CB1438 | 0.87 | 0.67 | Male ⅜″-18NPTF | Female ¼″-18NPTF | B | D | A |  | C |
|  | Bushing | CB1412 | 1.10 | 0.87 | Male ½″-14NPTF | Female ¼″-18NPTF |  |  |  |  |  |
|  | Bushing | CB3812 | 1.10 | 0.87 | Male ½″-14NPTF | Female ⅜″-18NPTF |  |  |  |  |  |
|  | 2 way 45° Connector Street Elbow, ⅜″ to ⅜″-NPTF | FT045 | 1.89 | 1.02 | Male ⅜″-18NPTF | Female ⅜″-18NPTF | B | D | A | C |  |
|  | 2 way 90° Connector Street Elbow, ⅜″ to ⅜″-NPTF | FT101 | 1.81 | 1.02 | Male ⅜″-18NPTF | Female ⅜″-18NPTF | B | D | A C |  |  |
|  | 2 way 90° Connector Elbow, ⅜″ to ⅜″ - NPTF | FT107 | 1.40 | 1.02 | Female ⅜″-18NPTF | Female ⅜″-18NPTF | B | D | A | C |  |
|  | 3 Way Connector ⅜″ to ⅜″ - NPTF | FT106 | 1.77 | 1.02 | Female ⅜″-18NPTF | Female ⅜″-18NPTF | B |  | A |  | D C |
|  | 4 Way Connector Cross, ⅜″ to ⅜″ - NPTF | FT105 | 1.77 | 1.02 | Female ⅜″-18NPTF | Female ⅜″-18NPTF | B |  | A | D | C |
| Couplings |  |  |  |  |  |  |  |  |  |  |  |
|  | Coupling Reducing Connector, ⅜″ to ¼″-NPTF | FT102 | 1.18 | 1.02 | Female ¼″-18NPTF | Female ⅜″-18NPTF | B | D | A C |  |  |
|  | Coupling Reducing Con- nector, ½″ to ⅜″ - NPTF | FT112 | 1.89 | 1.14 | Female ½″-14NPTF | Female ⅜″-18NPTF |  |  |  |  |  |
|  | Coupling ⅜″ to ⅜″- NPTF | FT103 | 1.18 | 1.02 | Female ⅜″-18NPTF | Female ⅜″-18NPTF |  |  |  |  |  |
| Hex Nipples |  |  |  |  |  |  |  |  |  |  |  |
|  | Male Connector Hex Nipple, ¼″ to ¼″ - NPTF | FT104 | 1.50 | 0.67 | Male ¼″-18NPTF | Male ¼-18NPTF | B | D | A C |  |  |
|  | Male Connector Hex Nipple, ⅜″ to ⅜″- NPTF | FT114 | 1.50 | 0.75 | Male ⅜″-18NPTF | Male ⅜″-18NPTF |  |  |  |  |  |
|  | Male Connector Hex Nipple, ¼″ to ⅜″- NPTF | FT124 | 1.45 | 0.75 | Male ¼″-18NPTF | Male ⅜″-18NPTF |  |  |  |  |  |

75

76

| Presses | Reference Image Page Number | Presses | Page Number / Reference Image |  |
| --- | --- | --- | --- | --- |
| Movable Head Shop Press | 77 | Shop Press Accessories |  | 80 |
| Fixed Head Shop Presses | 78-79 | Press Guards |  | 80 |
| Tools |  |  | Page Number / Reference Image |  |
| Flange Spreaders |  |  |  | 81 |
| Duck Bill Spreaders |  |  |  | 81 |
| Oil Field Valve Seat Pullers 82 |  |  |  |  |
| 50 Ton Cylinder with Base and Needle Valve Kit 82 |  |  |  |  |
| Hydraulic Bench Vise |  |  |  | 82 |
| Hydraulic Gear Pullers |  |  |  | 83 |
| Mechanical Gear Pullers |  |  |  | 83 |
| Hydraulic Nut Splitters |  |  |  | 83 |
| Maintenance/Bottle Jacks Page 84 |  |  |  |  |

1-888-332-6419

www.BVAhydraulics.com

Shop Presses / Tools

| Capacity (Tons) | Model Number | Pump Type | Pump Model | Cylinder Model | Cylinder Stroke (In) | Bed Positions | Press Dimensions (H x L x W) | Working Space (Min - Max) (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Acting |  |  |  |  |  |  |  |  |
| 55 | IAPR5506 | Air | PA2000 | H5506 | 6 | 7 | 78" X 46.5" X 30" | 15.38" X 51.38" |
|  | IEPR5506 | Electric | PE40W3N02A | H5506 | 6 | 7 | 78" X 46.5" X 30" | 15.38" X 51.38" |
|  | IAPR5513 | Air | PA3801 | H5513 | 13 | 7 | 85" X 46.5" X 30" | 15.38" X 51.38" |
|  | IEPR5513 | Electric | PE40W3N02A | H5513 | 13 | 7 | 85" X 46.5" X 30" | 15.38" X 51.38" |
|  | IEPRS5513 | Electric/Solenoid | PE40S3L02A | H5513 | 13 | 7 | 85" X 46.5" X 30" | 15.38" X 51.38" |
| Double Acting |  |  |  |  |  |  |  |  |
| 55 | IAPDR5506 | Air | PA3801M | HD5506 | 6 | 7 | 78" X 46.5" X 30" | 15.38" X 51.38" |
|  | IEPDR5513 | Electric | PE40W4N02A | HD5513 | 13 | 7 | 85" X 46.5" X 30" | 15.38" X 51.38" |
|  | IEPDRS5513 | Electric/Solenoid | PE40S4L02A | HD5513 | 13 | 7 | 85" X 46.5" X 30" | 15.38" X 51.38" |

55 Ton Shop Press with Movable Work Head

H-Frame

Movable work head

Welded steel construction

- Cylinder moves from side to side and locks into position

- Multi-position bed for work height adjustment

- Pump, cylinder, hose, gauge and fittings are included

K

J

H

D

A (min)

A (max)

F

E

G

C

B

| Model Number | Dimensions (in) |  |  |  |  |  |  |  |  |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A (Min.) | A (Max.) | B | C | D | E | F | G | H | J | K |  |
| Single Acting |  |  |  |  |  |  |  |  |  |  |  |  |
| IAPR5506 | 15.38 | 51.38 | 78 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 950.0 |
| IEPR5506 | 15.38 | 51.38 | 78 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 1015.0 |
| IAPR5513 | 15.38 | 51.38 | 85 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 991.0 |
| IEPR5513 | 15.38 | 51.38 | 85 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 1048.0 |
| IEPRS5513 | 15.38 | 51.38 | 85 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 1050.0 |
| Double Acting |  |  |  |  |  |  |  |  |  |  |  |  |
| IAPDR5506 | 15.38 | 51.38 | 78 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 955.0 |
| IEPDR5513 | 15.38 | 51.38 | 85 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 1020.0 |
| IEPDRS5513 | 15.38 | 51.38 | 85 | 46.5 | 30 | 35.75 | 6 | 4 | 9 | 10 | 15 | 1022.0 |

77

### Shop Presses

- Welded steel construction

- Ideal for the removal or installation of gears, universal joints, pulleys, wrist pins, and other press jobs

- Multi-position bed for work height adjustment

- Excellent for bending, bonding, or straightening applications

Cylinder H2506

Cylinder H5506

Pump

PE40W3N02A

Cylinder H10010

Pump PA1500

Pump P2301

Press IAP2506

Press IMP5506

Press IEP10010

| Capacity (Tons) | Model Number | Pump Type | Pump Model | Cylinder Model | Cylinder Stroke (In) | Bed Positions | Press Dimen- sions (H X L X W) | Working Space (Min - Max) (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single Acting |  |  |  |  |  |  |  |  |
| 25 | IMP2506 | Manual | P1000 | H2506 | 6 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IAP2506 | Air | PA1500 | H2506 | 6 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IEP2506 | Electric | PE30W3N01A | H2506 | 6 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IMP2514 | Manual | P2001 | H2514 | 14 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IAP2514 | Air | PA2000 | H2514 | 14 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IEP2514 | Electric | PE30W3N01A | H2514 | 14 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IEPS2514 | Electric/Solenoid | PE30S3L01A | H2514 | 14 | 8 | 71” X 48” X 28” | 4.5” X 47.25” |
| 55 | IMP5506 | Manual | P2301 | H5506 | 6 | 9 | 79” X 56” X 30” | 14” X 55.5” |
|  | IAP5506 | Air | PA1500 | H5506 | 6 | 9 | 79” X 56” X 30” | 14” X 55.5” |
|  | IEP5506 | Electric | PE40W3N02A | H5506 | 6 | 9 | 79” X 56” X 30” | 14” X 55.5” |
|  | IAP5513 | Air | PA3801 | H5513 | 13 | 9 | 79” X 56” X 30” | 14” X 55.5” |
|  | IEP5513 | Electric | PE40W3N02A | H5513 | 13 | 9 | 79” X 56” X 30” | 14” X 55.5” |
| 100 | IAP10010 | Air | PA7550 | H10010 | 10 | 8 | 89” X 63” X 30” | 11.25” X 51” |
|  | IEP10010 | Electric | PE40W3N02A | H10010 | 10 | 8 | 89” X 63” X 30” | 11.25” X 51” |
|  | IEPS10010 | Electric/Solenoid | PE40S3L02A | H10010 | 10 | 8 | 89” X 63” X 30” | 11.25” X 51” |
| Double Acting |  |  |  |  |  |  |  |  |
| 25 | IMPD2506 | Manual | P2301M | HD2506 | 6 | 6 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IAPD2506 | Air | PA3801M | HD2506 | 6 | 6 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IEPD2506 | Electric | PE30W4N01A | HD2506 | 6 | 6 | 71” X 48” X 28” | 4.5” X 47.25” |
|  | IEPDS2506 | Electric/Solenoid | PE30S4L01A | HD2506 | 6 | 6 | 71” X 48” X 28” | 4.5” X 47.25” |
| 55 | IEPD5513 | Electric | PE40W4N02A | HD5513 | 13 | 9 | 79” X 56” X 30” | 14” X 55.5” |
| 100 | IEPD10010 | Electric | PE40W4N02A | HD10010 | 10 | 8 | 89” X 63” X 30” | 11.25” X 51” |
|  | IEPDS10010 | Electric/Solenoid | PE40S4L02A | HD10010 | 10 | 8 | 89” X 63” X 30” | 11.25” X 51” |

1-888-332-6419

78	www.BVAhydraulics.com

I Series

Made in the U.S.A.

All press frames are made in the USA and meet ISO 9001/2008 and CE

certification.

Press Accessories

See the assortment of accessories designed to work with these product.

pg

80

Press Guards

Add a layer of protection between the operator and the work for extra safety.

pg 80

79

Shop Presses / Tools

K

J

H

D

A (min)

A (max)

F

E

G

C

B

| Model Number | Dimensions (in) |  |  |  |  |  |  |  |  |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A (Min.) | A (Max.) | B | C | D | E | F | G | H | J | K |  |
| Single Acting |  |  |  |  |  |  |  |  |  |  |  |  |
| IMP2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 442.0 |
| IAP2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 447.0 |
| IEP2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 490.0 |
| IMP2514 | 6.25 | 48.25 | 79.03 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 469.0 |
| IAP2514 | 6.25 | 48.25 | 79.03 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 465.0 |
| IEP2514 | 6.25 | 48.25 | 79.03 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 506.0 |
| IEPS2514 | 6.25 | 48.25 | 79.03 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 508.0 |
| IMP5506 | 8.00 | 55.88 | 80.50 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 847.0 |
| IAP5506 | 8.00 | 55.88 | 80.50 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 837.0 |
| IEP5506 | 8.00 | 55.88 | 80.50 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 880.0 |
| IAP5513 | 8.00 | 55.88 | 87.51 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 876.0 |
| IEP5513 | 8.00 | 55.88 | 87.51 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 910.0 |
| IAP10010 | 14.38 | 52.38 | 90.00 | 46.25 | 40.00 | 35.50 | 8.00 | 4.00 | 10.13 | 11.00 | 16.25 | 1142.0 |
| IEP10010 | 14.38 | 52.38 | 90.00 | 46.25 | 40.00 | 35.50 | 8.00 | 4.00 | 10.13 | 11.00 | 16.25 | 1178.0 |
| IEPS10010 | 14.38 | 52.38 | 90.00 | 46.25 | 40.00 | 35.50 | 8.00 | 4.00 | 10.13 | 11.00 | 16.25 | 1180.0 |
| Double Acting |  |  |  |  |  |  |  |  |  |  |  |  |
| IMPD2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 468.0 |
| IAPD2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 459.0 |
| IEPD2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 496.0 |
| IEPDS2506 | 6.25 | 48.25 | 71.00 | 38.75 | 28.00 | 30.00 | 6.00 | 3.00 | 6.50 | 8.00 | 11.50 | 498.0 |
| IEPD5513 | 8.00 | 55.88 | 87.51 | 46.25 | 30.00 | 35.63 | 6.00 | 3.00 | 9.00 | 10.00 | 14.75 | 942.0 |
| IEPD10010 | 14.38 | 52.38 | 90.00 | 46.25 | 40.00 | 35.50 | 8.00 | 4.00 | 10.13 | 11.00 | 16.25 | 1198.0 |
| IEPDS10010 | 14.38 | 52.38 | 90.00 | 46.25 | 40.00 | 35.50 | 8.00 | 4.00 | 10.13 | 11.00 | 16.25 | 1200.0 |

### 25 Ton Shop Press Accessories

CS25 includes:

- Rod forcing cap

- V push adapter

- Threaded adapter

- ¾″ push adapter

- 1″ push adapter

- Arbor plates (in pairs)

55 Ton Shop Press Accessories

100 Ton Shop Press Accessory

CS55 includes:

- Rod forcing cap

- Threaded adapter

- 1½″ push adapter

- 1¼″ push adapter

- V push adapter

- Arbor plates (in pairs)

Press Guards

Add a barrier layer between the operator and the work for added safety. Press guards help keep hands out of work area and shields flying debris from being ejected from the press and prevents them from becoming a hazard.

- Sturdy construction

- Fits all types of shop presses (single and double acting)

- Prevent possible injury/damage from flying debris from

occurring in case of an issue

AP100 includes:

  - Arbor plates (in pairs)

| Model Number | Capacity (tons) | Shop Press Compatibility |
| --- | --- | --- |
| SPG25 | 25 | All Manual, Electric and Air 25 Tons Presses (Single and Double Acting) |
| SPG100 | 55 & 100 | All Manual, Electric and Air 55 and 100 Tons Presses (Single and Double Acting) |

1-888-332-6419

80	www.BVAhydraulics.com

Flange Spreader

Pin Type

SP3-3814

This combination kit works well with our Hand Pump. See our combo section for more details.

HFS05

Kit: SP3-3814

pg64

SP5-3814

This combination kit works well with our Foot Pump. See our combo section for more details.

HFS05

HFS10

- Designed for heavy duty industrial environment

- Adjustable jaw width for a wide range of applications and ease of use

- Spare wedge attachment comes standard

| Capacity (tons) | Model Number | Stroke (in.) | Oil Capacity (in³) | Maximum Flange Thickness (in.) |
| --- | --- | --- | --- | --- |
| 5 | HFS05 | 1.00 | 1.00 | 2 X 2.3 |
| 10 | HFS10 | 2.01 | 4.50 | 2 X 3.6 |

SR10

SR05

Spreaders

Duck Bill Spreader

- Designed to work in confined working spaces

Shop Presses / Tools

Ideal for wedging, prying, and opening

- High strength forged steel

- Spring return for easy operation

3/8"- 18NPTF

B	B

3/8"- 18NPTF

CAPACITY 12 TON

D

D

E

C

SR05

A

C

SR10

A

E

| Cylinder Capacity (tons) | Model Number | Cylinder Effective Area (in²) | Oil Capacity (in³) | Dimensions (in) |  |  |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | A | B | C | D | E |  |
| 0.5 | SR05 | 1.22 | 0.58 | 3.54 | 7.57 | 0.51 | 2.05 | 2.36 | 4.8 |
| 1 | SR10 | 1.09 | 0.86 | 3.90 | 6.57 | 0.47 | 2.17 | 2.20 | 5.1 |

## 81

### Valve Seat Puller

Frac Pump Maintenance

- Pump kit to operate HVSP Series cylinders. Part number: SP12-CSG4

- Pump kit includes: P1201S, CS3814, CH38M, GW4014, and CF3814.

HVSP120

SP5-3814

HVSP125

| Cylinder Capacity (tons) | Model Number | Center Hole ID (in) | Stroke (in) | Collapsed Height (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 120 | HVSP120 | 2.13” | 0.607 | 3.63 | 61.7 |
| 125 | HVSP125 | 2.13” | 0.607 | 3.63 | 54.0 |

50 Ton Cylinder w/ Base & Needle Valve

This combination kit works well with this product. See our combo section for more details.

Safety Practices

Good industry practice recommends not exceeding 80% of maximum rated capacities of all our products.

Hydraulic Truck Jack

- Aluminum Cylinder

- Anodized ram with carry handle

- Shutoff valve

- High/low temp functionality

- CH38F 3/8”-18NPTF high flow female coupler

- 50 Ton @ 9,091 PSI

JU506

| Cylinder Capacity (tons) | Model Number | Collapsed Height (in) | Extended Height (in) | Stroke (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- |
| 50 | JU506 | 11.6 | 17.6 | 6.0 | 28.0 |

### Hydraulic Bench Vise

5 Ton Holding Force

HFS10

Hands free operation with foot pump control

- Air control valve for variable speed and added safety

- Cast steel construction 52,000 PSI tensile strength

- Hydraulic pressure closes jaws, spring return opens jaws

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

methods of operation. Always be prepared.

pg92

| Model Number | Capacity (tons) | Jaw Width (in) | Jaw Clearance (in) | Jaw Opening (in) |
| --- | --- | --- | --- | --- |
| HBV5 | 5 | 8.00 | 3.50 | 8.00 |

1-888-332-6419

82	www.BVAhydraulics.com

HGP503

HGP253

HGP153

HGP103

HGP053

### Hydraulic Gear Pullers

POSILOCK®

- Safety cage design prevents jaws from slipping

- Cylinders are removable for ease of servicing

- Each puller comes with a lift plate and a ram point set

| Capacity (tons) | Model Number | Reach (in) | Spread Range (in) | Ram Stroke (in) | Jaw Style |
| --- | --- | --- | --- | --- | --- |
| 5 | HGP053 | 6” | 0.5” - 8” | 5” | External |
| 10 | HGP103 | 8” | 0.75” - 12” | 6.13” | External |
| 15 | HGP153 | 10” | 1” - 15” | 10” | External |
| 25 | HGP253 | 12” | 2.5” - 18” | 14” | External |
| 25 | HGP253IE | 12” | 2.5” - 18” | 14” | External/Internal |
| 50 | HGP503 | 14” | 3” - 25” | 13.25” | External |
| 100 | HGP1003 | 50” | 7.5” - 70” | 10.25” | External |

HGP-1

This combination kit will work well with the hydraulic gear pullers. See our combo section for more details.

  - Able to use 2 or 3 jaws

  - Reversible ratchet jaws

  - Forged, heat treated high quality steel

  - Designed for heavy duty industrial environment

  - Adjustable jaw width for a wide range of applications for ease of use

  - Easily transportable

Mechanical Gear Pullers

2, 5, 7 Tons

MGP07

| Capacity (Ton) | Model Number | Maximum Spread (in) | Maximum Reach (in) | Screw Length (in) |
| --- | --- | --- | --- | --- |
| 2 | MGP02 | 3-1/4” | 2-5/8” | 4-1/2” |
| 5 | MGP05 | 5” | 4” | 8-1/4” |
| 7 | MGP07 | 10” | 10” | 12” |

Hydraulic Nut Splitter w/ Case

Compact and Heavy Duty

MGP05

MGP02

Shop Presses / Tools

Safety Instructions

Visit our DO’S and DON’TS

section to review the best

HNS105

Spacers included

    - Straight chisel with a 60/40 angle, and works on all type of materials, including

stainless steel

methods of operation. Always be prepared.

SP5-3814

This combination kit works well with

pg92

    - Chisel can be re-sharpened multiple times

    - Comes with a padded tool box to hold the head, cylinder, tips and spacers

| Capacity (tons) | Model Number | Nut Size Capacity ATF | Stud Dimension | Weight (lbs) |
| --- | --- | --- | --- | --- |
| 25 | HNS105 | 0.625” - 1.4375” | 0.4375” - 0.875” | 20.0 |
| 25 | HNS200 | 1.1875” - 2.375” | 7/16” - 7/8” | 21.0 |
| 100 | HNS308 | 2.625” - 3.5” | 1.75” - 2.25” | 95.0 |

POSILOCK® is a registered trade mark of Posilock Puller Inc.

the hydraulic nut splitter set. See our combo section for more details.

High-Flow Couplers

High-Flow Coupler: CH38F is included on all models (except where specified).

## 83

84

| Maintenance Kits / Bottle Jacks Page Number / Reference Image |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rail Car Jack |  |  |  |  |  | 85 |
| Maintenance Kits |  |  |  |  |  | 85 |
| Lifting Bags |  |  |  |  |  | 86 |
| Lifting Bag Accessories |  |  |  |  |  | 87 |
| Manual Bottle Jacks |  |  |  |  |  | 88 |
| Air/Manual Bottle Jacks |  |  |  |  |  | 88 |
| Toe Jacks |  |  |  |  |  | 88 |
| Mini Jacks |  |  |  | 89 |  |  |
| OMNI Jacks |  |  |  | 89 |  |  |
| Orbital Jacks |  |  |  | 89 |  |  |
| Cross Referencing and |  | Engineering Page |  |  | 90 |  |

###### 1-888-332-6419

www.BVAhydraulics.com

Rail Car Jack, JDR80, lifting the back of a rail car for the operator to start mainte-

nance on the car’s wheels.

Rail Car Jack

Double Acting Alum. Cylinder

- High strength aluminum construction suitable for rough working environments.

- Specially designed saddle

- Rod wiper seals out dirt and contamination

- Connect under pressure flush face quick couplers with pressure

relief feature for ease of installation and/or removal

- Max. working pressure: 4200 PSI / 290 Bar

JDR80

Jack Sets

JDR8-PE45

- PE45M4N10A - 1 hp Electric Motor Pump

- 2x  CS38JDRF  -  Extend Hoses (20 Feet)

- 2x  CS38JDRM -  Retract Hoses (20 Feet)

- 2x JDR80 Cylinders

JDR80	PE45M4N10A

JDR8-PG95

  - PG95M4N10 - 10.2 hp Gas Engine Pump

  - 2x  CS38JDRF  -  Extend Hoses (20 Feet)

  - 2x  CS38JDRM -  Retract Hoses (20 Feet)

  - 2x JDR80 Cylinders

JDR80	PG95M4N10

| Model Number | Cylinder Capacity (tons) | Max. Pressure (psi) | Stroke (in) | Cylinder Effective Area (in2) | Oil Capacity (in3) |  | Weight (lbs) | Col- lapsed Ht. A (in) | Extend- ed Ht. B (in) | Cylinder Bore Dia. C (in) | Outside Dia. D (in) | Rod Dia. E (in) | Saddle Dia. F (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Extend | Retract |  |  |  |  |  |  |  |
| JDR80 | 80 | 4200 | 16.2 | 39.44 | 639.00 | 331.00 | 200.0 | 24.76 | 40.96 | 7.09 | 8.50 | 4.92 | 6.00 |

Maintenance Kits / Bottle Jacks

Maintenance Kits

4, 10, 20 Ton

J50100

J50040

J50200

- Sets are portable for ease of use on the job site

- Custom fitted, rugged blow molded storage case

- 10 ton set has wheels for ease of transport

- Designed for pushing, spreading, pressing, and more

| Capacity (tons) | Pump Capacity (psi) | Model Number | Rod |  |  | Number of Attachments | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Collapsed Height (in²) | Extended Height (in³) | Effective Area (in²) |  |  |
| 4 | 8,000 | J50040 | 11.00 | 15.75 | 0.998 | 14 | 46.0 |
| 10 | 10,000 | J50100 | 13.63 | 19.75 | 2.411 | 13 | 81.0 |
| 20 | 10,000 | J50200 | 11.77 | 16.93 | 5.14 | 16 | 206.0 |
| * When extension tubes and/or offset attachments are used, the capacity is reduced. |  |  |  |  |  |  |  |

85

### Lifting Bags

Aramid Reinforced

A lifting bag raising the axle of this train car for maintenance

Lifting bags are useful for many different types of applications

- High pressure air lifting bags require only 1” clearance and can lift up to 86 tons

- 3 or 4 layers of Aramid Fiber on both sides of the bag to create a lifting bag unequaled in strength and durability.

- Made in the USA and is ISO 9001/2008 and CE Certified

- Product replacement 3 year unconditional warranty.

- Marked with a distinct “bulls eye with tonnage rating”

- Conical, interlocking surface to limit slippage and aid in stacking.

- Each bag has either molded carry handles and/or tabs/eyelets

A

B

C

LB-50

Made in the U.S.A.

All lifting bags are made in the USA and meet ISO 9001/2008 and CE certification.

| Model Number | Capacity (tons) | Max. Lifting Height (in) | Max Working Pressure (psi) | Reinforcing Material | Handles or Eyelets | Dimensions (in) |  |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Length A | Width B | Deflated Ht C |  |
| LB-1 | 1.5 | 2.5 | 120 | Aramid | 2 Eyelets | 6.0 | 6.0 | 0.75 | 2.0 |
| LB-3 | 3.3 | 3.0 | 120 | Aramid | 4 Eyelets | 12.0 | 6.0 | 0.75 | 3.0 |
| LB-6 | 6 | 4.5 | 120 | Aramid | 4 Eyelets | 10.0 | 10.0 | 1.0 | 4.0 |
| LB-13 | 13 | 7.5 | 120 | Aramid | 4 Eyelets | 15.0 | 15.0 | 1.0 | 12.0 |
| LB-15 | 15 | 7.5 | 120 | Aramid | 2 Eyelets | 15.0 | 21.0 | 1.0 | 15.0 |
| LB-22 | 22 | 9.5 | 120 | Aramid | 1 Handle | 20.0 | 20.0 | 1.0 | 22.0 |
| LB-32 | 32 | 11.0 | 120 | Aramid | 1 Handle | 21.0 | 25.0 | 1.0 | 28.0 |
| LB-37 | 37 | 9.0 | 120 | Aramid | 2 Handles | 40.0 | 105.0 | 1.0 | 34.0 |
| LB-50 | 50 | 15.0 | 120 | Aramid | 2 Handles | 29.0 | 29.0 | 1.0 | 45.0 |
| LB-70 | 70 | 18.0 | 120 | Aramid | 2 Handles | 34.0 | 34.0 | 1.0 | 65.0 |
| LB-86 | 86 | 24.0 | 120 | Steel | 4 Handles | 42.0 | 42.0 | 1.0 | 118.0 |

1-888-332-6419

86	www.BVAhydraulics.com

Lifting Bag Accessories

Maintenance Kits / Bottle Jacks

Aramid Reinforced

| Reference Image | Model Number | Description |
| --- | --- | --- |
|  | LB-ACVC | Air-Cock Shut-off Valve W/Safety Coupler |
|  | LB-AT80CF | 80 cu ft Air Tank, 3,000psi |
|  | LB-AR1 | Air Bottle Regulator w/safety coupler |
|  | LB-DDC | Dual Deadman Control w/safety coupler |
|  | LB-SDC | Single Deadman Control w/safety coupler |
|  | LB-RH10C | 10 ft Green Regulator Hose w/Safety Couplers |
|  | LB-H16YC | 20 ft Yellow High Pressure Hose w/safety coupler |
|  | LB-H32YC | 32 ft Yellow High Pressure Hose w/safety coupler |
|  | LB-H16BC | 20 ft Blue High Pressure Hose w/safety coupler |
|  | LB-H32BC | 32 ft Blue High Pressure Hose w/safety coupler |
|  | LB-H16RC | 20 ft Red High Pressure Hose w/safety coupler |
|  | LB-H32RC | 32 ft Red High Pressure Hose w/safety coupler |
|  | LB-MCP4 | Master Control Pkg #4 |
|  | LB-MCP3 | Master Control Pkg #3 |

87

### Manual Bottle Jacks

  - Ton

    - Hydraulic system is protected from internal damage by a patented by-pass mechanism (U.S. Patent No.5,946,912)

    - Machined and polished cylinders with die-cut threads provide non-abrasive leak-free performance

    - Fabricated from hard-cast steel with heat-treated critical stress areas

    - Unique cross-type forged release valve assures positive load control

    - A wide, rugged base provides stability and strength

| Capacity (ton) | Model Number | Min. Height (in) | Max. Height (in) | Hydraulic Lifting (in) | Extension Screw (in) | Base Size (in) |  | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Length | Width |  |
| 12 | J10125 | 9.5 | 18.6 | 6.0 | 3.1 | 5.3 | 5.1 | 17.5 |
| 12 | J10129 | 6.0 | 10.6 | 3.0 | 1.6 | 5.3 | 5.1 | 14.9 |
| 20 | J10205 | 9.5 | 18.6 | 6.0 | 3.1 | 6.3 | 5.9 | 27.5 |
| 20 | J10209 | 6.5 | 11.3 | 3.1 | 1.6 | 6.3 | 5.9 | 21.8 |
| 30 | J10300 | 11.0 | 18.0 | 7.0 | - | 8.1 | 6.9 | 53.5 |
| 50 | J10500 | 9.5 | 13.9 | 4.4 | - | 10.9 | 7.4 | 103.5 |

Air/Manual Bottle Jacks

  - Ton

    - Air operation reduces operator fatigue and increases speed

    - A compact/powerful air turbo motor (U.S. Patent No. 5,341,723) is equipped to raise the load to the desired height efficiently, effortlessly, and safely

    - Hydraulic system is protected from internal damage by a patented

by-pass mechanism (U.S. Patent No.5,946,912)

    - Versatile ″Quick Connector″ design provides easy removal of the air

hose from the air motor, allowing compact storage or remote operation

| Capacity (ton) | Model Number | Min. Height (in) | Max. Height (in) | Hydraulic Lifting (in) | Extension Screw (in) | Base Size Length (in) | Base Size Width (in) | Input Air Pressure Range (psi) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | J18124 | 9.5 | 18.5 | 5.5 | 3.5 | 7.6 | 6.5 | 110-175 | 28.6 |
| 20 | J18204 | 9.8 | 18.8 | 5.1 | 3.9 | 8.3 | 7.3 | 110-175 | 37.4 |
| 30 | J18302 | 11.0 | 18.0 | 7.0 | - | 10.3 | 8.3 | 110-175 | 60.5 |
| 50 | J18502 | 12.0 | 19.0 | 7.0 | - | 10.9 | 7.4 | 110-175 | 128.5 |
| 100 | J18992* | 12.0 | 18.1 | 6.1 | - | 12.6 | 9.4 | 110-175 | 205.9 |

*Air Operated Only

### Toe Jacks

Low Height

    - Heat treated and chrome plated medium-carbon steel cylinder

    - Chrome plated rod promotes longer seal life and optimum performance

    - U.S. Patented by-pass device protects hydraulic system from over travel damage

    - Hydraulic system is protected from internal damage by a patented by-pass mechanism (U.S. Patent No.5,946,912)

| Capacity (tons) | Model Number | Toe Saddle |  | Head Saddle |  | Hydraulic Lifting (in) | Base Size (L x W) (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min. Height (in) | Max. Height (in) | Min. Height (in) | Max. Height (in) |  |  |  |
| 3 | J13060 | 0.6 | 5.8 | 9.0 | 14.1 | 5.1 | 6.0 x 4.4 | 36.1 |
| 6 | J13120 | 0.9 | 6.0 | 10.5 | 15.6 | 5.1 | 11.0 x 7.5 | 55.0 |
| 10 | J13200 | 1.1 | 6.3 | 11.0 | 16.1 | 5.1 | 11.6 x 9.0 | 84.0 |
| 15 | J13300 | 1.1 | 6.6 | 13.1 | 18.6 | 5.5 | 12.5 x 10.8 | 116.6 |

1-888-332-6419

88	www.BVAhydraulics.com

J11050

J11100

Mini Jacks

Compact

- Low profile mini jack

- Compact and lightweight

- Convenient for storage

Interchangeable saddle on model J11050 and J11055

These mini jacks are lightweight and will fit in the

palm of your hand. We see here a J11050 Mini Jack.

| Capacity (tons) | Model Number | Minimum Height (in) | Maximum Height (in) | Hydraulic Lifting (in) | Base Size (L x W) (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | J11050 | 2.8 | 3.1 | 0.8 | 4.1 x 4.0 | 4.6 |
| 5 | J11055 | 3.8 | 4.9 | 1.5 | 4.1 x 4.0 | 5.3 |
| 10 | J11100 | 4.8 | 6.3 | 1.6 | 4.9 x 4.4 | 12.5 |
| 20 | J11200 | 5.1 | 6.8 | 1.6 | 5.4 x 4.8 | 16.7 |

J11050

Maintenance Kits / Bottle Jacks

OMNI Jacks

Multi-Position Jack

- A wide, rugged base provides stability and strength while operating in

a vertical or horizontal position

- Hydraulic lift is protected from internal damage by a by-pass mecha- nism U.S. Patent No. 5,946,912

- Machined and polished cylinders with die-cut threads provide non-abrasive leak-free, durable performance

- Fabricated from hard-cast steel with heat-treated critical stress areas

| Capacity (tons) | Model Number | Minimum Height (in) | Maximum Height (in) | Hydraulic Lifting (in) | Extension Screw (in) | Base Size (L x W) (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | J16123 | 9.5 | 18.6 | 6.0 | 3.1 | 5.3 x 5.1 | 18.0 |
| 20 | J16203 | 9.5 | 18.6 | 6.0 | 3.1 | 6.3 x 5.9 | 28.0 |

J16123

J16203

### Orbital Jacks

Rotating Handle

  - Serrated, heat-treated saddle provides generous lifting area and secure grip

  - Heat-treated extension screw allows low pick-up height adjustment and maximum lift height

  - U.S. Patented built-in by-pass device protects hydraulic system from over-pumping damage

J10124

J10204

J10126

J10206

| Capacity | Model Number | Min Height (in) | Max Height (in) | Hydraulic Lifting (in) | Extension Screw (in) | Base Size (in) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | J10124 | 9.5 | 18.6 | 6.0 | 3.1 | 7.5 x 4.6 | 19.6 |
|  | J10126 | 6.1 | 10.6 | 3.0 | 1.5 | 7.5 x 4.6 | 16.5 |
| 20 | J10204 | 9.6 | 18.8 | 5.1 | 4.0 | 7.5 x 5.4 | 29.3 |
|  | J10206 | 6.5 | 11.3 | 2.8 | 2.0 | 7.5 x 5.4 | 22.2 |

89

### BVA Cross Reference Chart*

| Ton | BVA | Enerpac | Power Team | Simplex |
| --- | --- | --- | --- | --- |
| 55 | H5502 | RC-502 | C552C | R552 |
|  | H5504 | RC-504 | C554C | R554 |
|  | H5506 | RC-506 | C556C | R556 |
|  | H5513 | RC-5013 | C5513C | R5513 |
| 75 | H7506 | RC-756 | C756C | R756 |
|  | H7513 | RC-7513 | C7513C | R7513 |
| 100 | H10006 | RC-1006 | C1006C | R1006 |
|  | H10010 | RC-10010 | C10010C | R10010 |
| Threaded Cylinders |  |  |  |  |
| 10 | HT1006 | - | C106CBT | - |
|  | HT1010 | - | C1010CBT | - |
| Aluminum Cylinders |  |  |  |  |
| 20 | HU2002T | RAC-202 | RA202 | - |
|  | HU2004T | RAC-204 | RA204 | - |
|  | HU2006T | RAC-206 | RA206 | - |
| 30 | HU3002 | RAC-302 | RA302 | - |
|  | HU3004 | RAC-304 | RA304 | - |
|  | HU3006 | RAC-306 | RA306 | RAS306B |
| 50 | HU5002 | RAC-502 | RA552 | RAS502A |
|  | HU5004 | RAC-504 | RA554 | - |
|  | HU5006 | RAC-506 | RA556 | RAS508A |
|  | HU5010 | RAC-5010 | RA5510 | - |
| 75 | HU7504 | - | - | - |
|  | HU7506 | - | - | - |
| 700 | HU10002 | RAC-1002 | RA1002 | RAS1002A |
|  | HU10006 | RAC-1006 | RA1006 | RAS1008A |
|  | HU10010 | RAC-10010 | - | - |
| Low Profile Cylinders |  |  |  |  |
| 10 | HL1001 | RCS-101 | RSS101 | RLS101 |
| 20 | HL2002 | RCS-201 | RSS202 | RLS201 |
| 30 | HL3002 | RCS-302 | RSS302 | RLS302 |
| 50 | HL5002 | RCS-502 | RSS502 | RLS502 |
| 100 | HL10002 | RCS-1002 | RSS1002 | RLS1002 |
| Flat Body Cylinders |  |  |  |  |
| 5 | HF0503 | RSM-50 | RLS50 | - |
|  | HF0503B kit | - | - | - |
| 10 | HF1005 | RSM-100 | RLS100 | RFS10 |
|  | HF1005B kit | - | - | - |

Compatibility Check

| Ton | BVA | Enerpac | Power Team | Simplex |
| --- | --- | --- | --- | --- |
| Single Acting Cylinders |  |  |  |  |
| 5 | H0500 | RC-50 | - | R50 |
|  | H0501 | RC-51 | C51C | R51 |
|  | H0503 | RC-53 | C53C | R53 |
|  | H0505 | RC-55 | C55C | R55 |
|  | H0507 | RC-57 | C57C | R57 |
|  | H0509 | RC-59 | C59C | R59 |
| 10 | H1001 | RC-101 | C101C | R101 |
|  | H1002 | RC-102 | C102C | R102 |
|  | H1004 | RC-104 | C104C | R104 |
|  | H1006 | RC-106 | C106C | R106 |
|  | H1008 | RC-108 | C108C | R108 |
|  | H1010 | RC-1010 | C1010C | R1010 |
|  | H1012 | RC-1012 | C1012C | R1012 |
|  | H1014 | RC-1014 | C1014C | R1014 |
| 15 | H1501 | RC-151 | C151C | - |
|  | H1502 | RC-152 | C152C | R152 |
|  | H1504 | RC-154 | C154C | R154 |
|  | H1506 | RC-156 | C156C | R156 |
|  | H1508 | RC-158 | C158C | - |
|  | H1510 | RC-1510 | C1510C | R1510 |
|  | H1512 | RC-1512 | C1512C | - |
|  | H1514 | RC-1514 | C1514C | R1514 |
| 25 | H2501 | RC-251 | C251C | R251 |
|  | H2502 | RC-252 | C252C | R252 |
|  | H2504 | RC-254 | C254C | R254 |
|  | H2506 | RC-256 | C256C | R256 |
|  | H2508 | RC-258 | C258C | R258 |
|  | H2510 | RC-2510 | C2510C | R2510 |
|  | H2512 | RC-2512 | C2512C | R2512 |
|  | H2514 | RC-2514 | C2514C | R2514 |
| 30 | H3008 | RC-308 | - | R308 |

BVA ® is a registered trademark of SF Companies.

Enerpac® is a registered trademark of the Actuant Corporation. Power Team® is a registered trademark of SPX Corporation.

Simplex® is registered of the Actuant Corporation.

1-888-332-6419

90	www.BVAhydraulics.com

Cross-Referencing / Engineering

BVA Cross Reference Chart*

| Ton | BVA | Enerpac | Power Team | Simplex |
| --- | --- | --- | --- | --- |
| 20 | HF2005 | RSM-200 | RLS200 | RFS20 |
|  | HF2005B kit | - | - | - |
| 30 | HF3005 | RSM-300 | RLS300 | RFS30 |
|  | HF3005B kit | - | - | - |
| 50 | HF5006B | RSM-500 | RLS500S | RFS50 |
| 75 | HF7506 | RSM-750 | RLS750S | RFS75 |
| 100 | HF10006 | RSM-1000 | RLS1000S | RFS100 |
| 150 | HF15006 | RSM-1500 | RLS1500S | RFS150 |
| Pull Cylinders |  |  |  |  |
| 2 | HP0205 | BRC-25 | RP25 | RP25 |
| 5 | HP0505 | BRC-46 | RP55 | RP55 |
| 10 | HP1006 | BRC-106 | - | - |
| Hollow Hole Cylinders |  |  |  |  |
| 12 | HC1201T | RCH-120 | RH120 | RC120 |
|  | HC1202T | RCH-121 | RH121 | RC121 |
|  | HC1202XT | RCH-1211 | RH121T | RC1211 |
|  | HC1203XT | RCH-123 | RH123 | RC123 |
| 20 | HC2002T | RCH-202 | RH202 | RC202 |
|  | HC2006T | RCH-206 | RH206 | RC206 |
| 30 | HC3002T | RCH-302 | RH302 | RC302 |
|  | HC3006T | RCH-306 | RH306 | RC306 |
| 60 | HC6003T | RCH-603 | RH603 | RC603 |
|  | HC6006T | RCH-606 | RH606 | RC606 |
| 100 | HC10003T | RCH-1003 | RH1003 | RC1003 |
| Lock Nut Cylinders |  |  |  |  |
| 55 | HLN5502 | CLL-502 | R552L | RLN502 |
|  | HLN5504 | CLL-504 | - | - |
|  | HLN5506 | CLL-506 | R556L | RLN506 |
|  | HLN5508 | CLL-508 | - | - |
|  | HLN5510 | CLL-5010 | R5510L | RLN5010 |
|  | HLN5512 | CLL-5012 | - | - |
| 100 | HLN10002 | CLL-1002 | R1002L | RLN1002 |
|  | HLN10004 | CLL-1004 | - | - |
|  | HLN10006 | CLL-1006 | R1006L | RLN1006 |
|  | HLN10008 | CLL-1008 | - | - |
|  | HLN10010 | CLL-10010 | R10010L | RLN10010 |
|  | HLN10012 | CLL-10012 | - | - |

| Ton | BVA | Enerpac | Power Team | Simplex |
| --- | --- | --- | --- | --- |
| 10 | HD1010 | RR-1010 | RD1010 | RDA1010 |
|  | HD1012 | RR-1012 | - | - |
| 30 | HD3008 | RR-308 | - | RDA308 |
|  | HD3014 | RR-3014 | RD2514 | RDA3014 |
| 55 | HD5506 | RR-506 | RD556 | RDA556 |
|  | HD5513 | RR-5013 | RD5513 | RDA5512 |
|  | HD5520 | RR-5020 | RD5518 | RDA5523 |
| 75 | HD7506 | RR-756 | - | - |
|  | HD7513 | RR-7513 | - | - |
| 100 | HD10006 | RR-1006 | RD1006 | RDA1006 |
|  | HD10013 | RR-10013 | RD10013 | RDA10012 |

Compatibility Check

| BVA | Enerpac | Power Team | Simplex |
| --- | --- | --- | --- |
| Hand Pumps |  |  |  |
| P601S | P-39 | P12 | P41 |
| P1201S | P-391 | P55 | - |
| P350 | P-142 | - | - |
| P1000 | P-392 | P59 | P60 |
| P2001 | P-80 | P157 | P82 |
| P2301 | P-80 | P159 | P140 |
| P4301 | P-801 | P157 | P230 |
| P8701 | P-462 | P460 | P461 |
| P2301M | P-84 | P157D | P230D |
| P8701M | P-464 | P460D | P461D |
| P240L | P-18 | - | - |
| Air Pumps |  |  |  |
| PA600 | PA-133 | PA9 | - |
| PA600H | - | PA9H | - |
| PA1500 | - | PA6 | GA90 |
| PA1500L | PARG-1102N | PA6R | GA90R |
| PA1500M | - | PA6DM | - |
| PA2000 | PATG-1102N | - | - |
| PA3801 | PATG-1105N | PA6-1 | GA9230 |
| PA3801L | - | - | GA9230R |
| PAR1703 | - | PA174 | - |
| PAR4003 | ZA4310MX | PA462 | G5733 |
| PARD1701 | - | PA172 | - |
| PARD1703 | - | PA174 | - |

*For comparison purposes only. Products are not identical. To view complete cross over list go to www.BVAhydraulics.com

## 91

Safety Instructions

- Read and understand all printed materials provided with and on this cylinder

- Do not exceed rated capacities

- Use only on hard level surface capable of sustaining the load

- Do not connect actuators to pumps with higher pressure ratings

- Always inspect hoses and connections for damage prior to use

- Keep hands and feet clear of work area

- LIFTING DEVICE ONLY! Immediately after lifting, support the load with appropriate means

- Use only manufacturer approved hydraulic fluid

- A flow-restricting component shall not be present between the relief valve and the tank

- No alterations shall be made to this cylinder

- Failure to heed these markings may result in injury and/or property damage

- Each double-acting cylinder shall be fitted with a relief valve on the retract circuit, which

bleeds off the smaller effective area of the cylinder

Safety Instructions

Industry Safety Standards

General

▲

Do not override the factory setting of relief valves. Always use a gauge

to check system pressure.

Cylinders

▲

Provide a solid support for the entire cylinder base area. Use cylinder base attachment

for more stability.

▲

The entire cylinder saddle must be in contact with the load. Movement of the cylinder must be parallel with the movement of the load.

▲

Do not use cylinder without saddle. This will cause rod to “mushroom” Saddles distribute load evenly on the rod.

▲

As with jacks, never place any part of your body under the load. Load must be on cribbing before venturing under.

###### Jacks

▲

Provide a level and solid support for the entire jack base area.

▲

Always protect cylinder threads for use with attachments.

▲

Never place any part of your body under the load. Ensure the load is solidly supported before venturing under.

▲

Keep hydraulic equipment away from open

fire and temperatures above 150°F (65°C).

▲

The entire jack saddle must be in contact with load. Movement of the load must be in the same direction as jack rod.

▲

Good industry practice recommends not exceeding 80% of maximum rated capacities.

- Use only on hard, level surfaces capable of sustaining the load.

- Immediately after lifting, support the load with appropriate means

- Failure to heed these warnings may result in personal injury as well as property damage.

80% Rule

Remove the jack handle when not in use.

###### 1-888-332-6419

92	www.BVAhydraulics.com

Hydraulic Pump Safety Instructions

- READ AND UNDERSTAND ALL WARNINGS, INSTRUCTIONS AND OWNER’S MANUAL INFORMATION BEFORE ATTEMPTING TO USE THIS EQUIPMENT

- Burst hazard exists if hose or connection pressure exceeds rated pressure.

- Do not exceed the rated capacity of this device. Monitor pressure and load at all times.

- Verify load with calibrated load cell and indicator, accurate pressure gauge or equivalent devices.

- Inspect hoses and connections for damage prior to use. Never attempt to grasp a leaking pressurized hose.

- Release the hydraulic pressure before disconnecting hoses or couplers, and before servicing the pump or accessory.

- Immediately after lifting, support the load with appropriate means. Never rely on hydraulic pressure to support a load.

- Wear protective clothing and eye protection that meets ANSI Z87.1 and OSHA standards when operating this equipment.

- Ensure that the chosen application is stable to work on and around.

- Do not subject the pump and its components to shock loads.

- Do not connect to application which can return more oil to the reservoir than the pump reservoir can hold.

- Ensure that the rated pressure of all applications and fittings be equal to or greater than the rated pressure of this pump.

- Keep hydraulic equipment away from flames and heat.

- No alterations shall be made to this device.

- Failure to heed these warnings may result in personal injury as well as property damage.

Pumps

▲

Do not use handle extender. Hand pumps should be easy to operate when used correctly.

▲

Fill pump only to recommended level. Fill only when connected cylinder is fully retracted.

###### Hoses and Couplers

▲

Close release valve finger tight. Using force will

damage the valve.

▲

Use only genuine BVA hydraulic oil. The wrong fluid can destroy your seals and pump and will render your warranty null and void.

Electric Pump Safety Instructions

TO AVOID ELECTROCUTION HAZARD:

- Owner/Operator must read, understand and follow all printed material provided with this device before use.

- Connect ONLY to properly grounded electrical source. Connect to GFI outlet ONLY! For use in dry locations.

- Avoid the use of extension cords. If used, extension cord lengths up to 25 ft shall be SJT-14X3 or heavier. For cords up to 50 ft, use SJT-12X3 or heavier.

Cross-Referencing / Engineering

For extension cord lengths up to 100 ft, use SJT-10X3 or heavier. NEVER use a 3 prong adapter.

- Do not operate this device in an explosive atmosphere or in the presence of

conductive liquids.

▲

Clean both coupler parts before connecting. Use dust caps when coupler parts are not connected.

▲

Keep hoses away from the area beneath loads.

▲

Detach cylinder only when fully retracted or use shut-off valves or safety valves to lock-in cylinder pressure.

▲

Do not kink hoses. Bending radius should be at least 4½". Do not drive over or drop heavy objects on the hoses.

▲

Don’t lift hydraulic equipment by the hoses.

## 93

### Maintaining Your Equipment

Ensure a longer life span of your equipment

Hydraulic Fluids

  - Use only ISO 46 BVA Hydraulic Oil P/N’s; F01 (1 gal) or F55 (55 gal) for pumps and cylinders.

  - Viscosity @ 100 deg F 234 (S.U.S.); Viscosity @210 deg F=53 (S.U.S.); ASTM Grade 215; Flash Point 356

deg F; Pour Point -49 deg F, API Gravity =30.8.

Air Line Lubricants

  - Use Class 1 Turbine Oil (ISO VG32) for Air Line Lubricant

Hydraulic and Pneumatic Filtration

  - Hydraulic fluid must be kept clean, cool and free of water.

  - Use Hydraulic Filtration to provide fluid which meets ISO 17/15/12 cleanliness levels.

  - Air Line Filtration for solid particulate contamination removal: Use BVA P/N; FRL-145, 5uM Pneumatic Filter.

  - Air Line Water Removal; Use BVA P/N; AD-145, Ambient Dryer.

Proper Operation of Equipment

  - Refer to individual Product Owner’s Manuals and BVA Catalogs for; general information, specifications,

safety, set up, operation, maintenance, trouble shooting, storage and warnings for all BVA products.

Formulas

| Key to Measurements |  |  |  |
| --- | --- | --- | --- |
| Weight: |  | Length |  |
| 1 pound (lb.) | =0.4536 kg | 1 in | =25.4 mm |
| 1 kg | =2.205 lbs | 1 mm | =0.039 in |
| 1 ton (short, US) | =2,000 lbs | 1 in² | =6.452 cm² |
| 1 ton (metric) | =2,205 lbs | 1 cm² | =0.155 in² |
| Volume: |  | Pressure: |  |
| 1 in³ | =16.387 cm³ | 1 psi (lb/in²) | =0.69 bar |
| 1 cm³ | =0.061 in³ | 1 bar | =14.5 psi |
| 1 liter | =61.02 in³ | 1 kPa | =0.145 psi |
|  | =0.264 gal(US) |  |  |
| 1 gal (US, liquid) | =3.785 liter | Temperature: |  |
|  | =231 in³ | 0°F | =(0°C x 1.8) + 32 |
|  | =3,785 cm³ | 0°C | =(0°F - 32) ÷ 1.8 |

Cylinder Rod Speed

Formula   →   V = A   × 60

Q

Cylinder Rod Speed (sec/in) =

Cylinder Effective Area (in2) × 60 (sec) Pump Flow Rate (in3/min)	1

Force

Formula   →   F = P × A

Force (lbs) = Hydraulic Working Pressure (lb/in2) × Cylinder Effective Area (in2)

Cylinder Oil Capacity

Formula   →   C = A × L

Oil Capacity (in3) = Cylinder Effective Area (in2) × Cylinder Stroke (in)

¼” Hose Oil Capacity

Formula   →   C = 0.5892 × H

Oil Capacity (in3) = 0.5892 × Length of Hose (ft)

⅜” Hose Oil Capacity

Formula   →   C = 1.3608 × H

Oil Capacity (in3) = 1.3608 × Length of Hose (ft)

Cylinder Effective Area

Formula   →  A =	× D2

4

Cylinder Effective Area =	× (Cylinder Bore Diameter)2

4

= 3.14159265359

1-888-332-6419

94	www.BVAhydraulics.com

Model Number Index

Cylinders

H

6

HUD

16

HL

26

HULN

32

HC

36

HP

41

HU

10

HG

18

HF

28

HLNF

33

HDC

38

HT

42

HD

12

HDG

22

HLN

30

HPD

43

Pages 6-43

HUC/HUDC 40

Accessories 34

Pumps

| P Single Acting 46 Hand Pumps | PAR Rotary Air 49 Pumps | PA Double Acting 50 Air Pumps | PG Gas Engine 60 Pumps |
| --- | --- | --- | --- |
| P Double Acting 46 Hand Pumps | PA All Position 50 Air Pumps | PE 1-Phase 52 Electric Pumps | PD Diesel Engine 60 Pumps |
| P57 Foot Pumps 48 | PA Single Acting 50 Air Pumps | PE 3-Phase 56 Electric Pumps | PU Universal Motor 63 Pumps |

Combo Kits

| H | General Purpose Cylinder Combinations | 64 | HL | Low Profile Cylinder Combinations | 65 |
| --- | --- | --- | --- | --- | --- |
| HF | Flat Body Cylinder Combinations | 64 | HLN | Lock Nut Cylinder Combinations | 65 |
| HC | Hollow Hole Cylinder Combinations | 65 | HU | Aluminum Cylinder Combinations | 65 |

Pages 44-63

Pages 64-65

Control Valves & Accessories

Flow Control Valves

66

Gauge Adapters

69

Solenoid Valves

70

Wireless Controller

71

Pressure Switch

72

Directional Control Valves

74

Manifolds	68

Hydraulic Oil

70

Hydraulic Couplers

71

External Pressure Adj

71

Hydraulic Hoses

72

Fittings

75

69

70

72

Pump Mounted 73

Valves

Manifold Blocks

Foot Pedals 71

Coupler Dust Cover

Pressure Gauges

Pages 66-75

| Movable Head 77 Shop Presses | Shop Press 80 Guards | Oil Field Valve 82 Seat Pullers | Hydraulic Gear Pullers 83 |
| --- | --- | --- | --- |
| Fixed Head 78 Shop Presses | Flange Spreaders 81 | Aluminum Cylinder 82 w/ Base/Needle Valve | Mechanical Gear 83 Pullers |
| Shop Press 80 Accessories | Duck Bill 81 Spreaders | Hydraulic Bench Vise 82 | Nut Splitters 83 |

Presses/Tools

Pages 76-83

| Maintenance Kits/Rail Cylinders | 85 | Manual Bottle Jacks | 88 | Mini Jacks | 89 |
| --- | --- | --- | --- | --- | --- |
| Lifting Bags | 86 | Air/Manual Bottle Jacks | 88 | OMNI All-Position Jacks | 89 |
| Lifting Bag Accessories | 87 | Toe Jacks | 88 | Orbital Jacks | 89 |

Maintenance Kits/Bottle Jacks

Pages 84-89

| BVA Cross Reference Chart | 90 | Maintaining Your Equipment | 94 |
| --- | --- | --- | --- |
| Hydralic Formula Reference | 91 | Maintaining Your Equipment | 94 |
| Safety Instructions | 92 |  |  |

Cross-Reference & Engineering

Pages 90-94

© Copyright 2016 SFA Companies

All rights reserved. Any copying or other use of the material in this catalog (text, illustrations, drawings, photos) without

prior express written consent is prohibited. Information in this catalog is subject to change without prior notice.	95

Cylinders

Contact Information

Pages 6-43

Pumps

United States of America

Shinn Fu Company of America

Address:  10939 N. Pomona Ave.

Kansas City, MO 64153

Tel:  888-332-6419

816-891-6390

Fax:  816-448-2142

Email: sales@bvahydraulics.com

Pages 44-63

Combo Kits

Canada

Global Locations

Europe

Pages 64-65

Control Valves & Accessories

Shinn Fu Company of Canada

Address: 6355 Shawson Dr.

Unit 1 Mississauga, Ontario L5T 1S7 Canada

Tel:  1-905-362-2277

Fax:  1-905-362-2280

Email: sales@shinnfucanada.ca

Shinn Fu Company of Europe

Address: Tussenrimer 20, 4704 SR

Roosendaal, The Netherlands

Tel:  31-165-583-300

Fax:  31-165-583-310

Email: sales@sfeu.nl

Pages 66-75

Presses/Tools

Australia

Shinn Fu Company of Australia

Address: 15 Viewtech Place

Rowville, Victoria 3178, Australia

Tel:  61-3-9755-8277

Fax:  61-3-9755-8219

Email: sales@bvaonline.com.au

Taiwan

Shinn Fu Corporation Headquarters

Address: 7F., No.408 Ruei Gaung Rd Nei Hu Disctrict Taipei City 114 Tiawan R.O.C

Tel:  886-2-8751-7888

Fax:  886-2-8741-7999

Email: sales@shinnfu.com

Pages 76-83

Maintenance Kits/Bottle Jacks

Pages 84-89

Cross-Reference & Engineering

Pages 90-94

Catalog Download

Scan this QR code with your phone to download our catalog

OR

Visit our website

www.BVAhydraulics.com

BVA is a division of