E

U

S S

y

r

At CRP, we’re big enough to help, but small enough to care.

WITH MORE THAN 30 YEARS IN THE THERMOPLASTIC HOSE BUSINESS, CRP UNDERSTANDS THE CHALLENGES AND NEEDS OF CUSTOMERS ACROSS ALL INDUSTRIES.

###### Cranbury, NJ

Tabor City, NC

Puebla, Mexico

Cranbury, NJ Headquarters & Warehouse

www.crpreinflex.com

We take enormous pride in our ability to listen and communicate with our customers. That’s why CRP excels at  providing clients with exactly what they need.

Be it fast delivery, competitive pricing, exceptional market knowledge, precise

hose replacements, a vast selection, custom lengths, or a variety

of end fittings. No matter what the issue, our highly trained customer service staff stands ready to solve

every customer’s request. Because of these commitments, CRP is proud

to have achieved a lengthy list of

high profile clients.

ROBUST HIGH PRESSURE HOSE PROGRAM

##### From hose construction materials and colors, to sizes and working pressures, we offer a wide and abundant selection of hose types for a variety of applications.

In addition, we are able to customize hose assemblies for virtually any requirement.

QUALITY CONTROL

##### With many of our high pressure hoses manufactured to DM&Q

ANSI/ISO/ASQ 9001-2000 standards, product quality is certain. And because CRP is

ISO 9001:2015 certified, customers can rest assured that we have a quality management system in place that delivers service satisfaction. In fact, CRP’s Quality Control Technicians randomly pressure test hoses and in most cases our hoses are identical to the original equipment specified.

TABLE OF CONTENTS	PAGE

Airless Paint Spray	4

Air Cascade Systems	12

Emergency Rescue Equipment	14

Fluid Transfer	20

High Pressure Hydraulics	24

Micro-bore Hoses	32

Moisture Resistant	34

Non-Conductive	36

PTFE Hose	40

Sewer Jetting	42

Water Jetting	46

Accessories	48

Notes	64

PROMPT TURNAROUND

##### For many of our customers, a speedy response is crucial. Fortunately, due to our vast selection of stock hose types and fittings, our turnaround time for orders is typically 24 to 48 hours. Of course, if time is of the essence, we may even be able to ship same day.

TOP-NOTCH SERVICE

##### A proud NAHAD member, CRP has been an expert in hose assembly

fabrication for over 30 years. Backed by a team of extraordinary customer service representatives, we excel at understanding and meeting the needs of all our customers. As CRP continues to grow and evolve, one thing will remain constant: CRP will always be big

enough to help, and small enough to care.	3

s

CRP’s customer service team excels in understanding your needs. That’s why our Airless Paint Spray Hoses are shipped out quickly, because we know time is of the essence in your industry. In fact, if required, same day turnaround may even be possible. CRP’s expertise ensures the hoses you receive will work right and fit right the first time and every time,

no matter what pump system. We carry many standard stock assemblies as well as offer custom fabrication options. Virtually anything you need from the pump up to the gun. We are your Airless Paint Spray Hose provider.

AIRLESS PAINT SPRAY HOSE FEATURES:

- Working pressures from 1,000 psi to 10,000 psi

- Excellent flexibility and flex fatigue

- Hose sizes from 1/8” to 1 1/4”

- Fitting options — NPT, NPSM, and JIC in plated or Stainless Steel – male (M) and female (F)

- Lengths from 4”up to 500’

- Wide range of hose whip assemblies

- Suitable for paints, solvents, lacquers, foams, caulks, resins, and roof coatings

4

1006P SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC PAINT HOSE

A high pressure semi-conductive economy thermoplastic hose that will not swell or degrade when in contact with most paints, chemicals, oils, and solvents. Extremely durable and lightweight. Ideal for situations where electrostatic charge must be dissipated.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006P | n | 2 | 1/4 (6.4) | 0.49 (12.5) | 2.5 (63.5) | 3,300 psi | 13,200 psi | 0.06 | 50’ | -40 to 180ºF |

1000AV SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC PAINT HOSE

*Only sold in 25’ and 50’ lengths

A high pressure semi-conductive thermoplastic hose that will not swell or degrade when in contact with most paints, chemicals, oils, and solvents. Extremely durable and lightweight. Ideal for situations where electrostatic charge must be dissipated.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003AV | n | 1 | 1/8 (3.2) | 0.32 (8.1) | 1.25 (31.8) | 3,500 psi | 14,000 psi | 0.03 | 4” up to 500’ | 0 to +180ºF |
| 1004AV | n n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.75 (44.5) | 3,500 psi | 14,000 psi | 0.05 | 4” up to 500’ | 0 to +180ºF |
| 1006AV | n n | 2 | 1/4 (6.4) | 0.49 (12.5) | 2.50 (63.5) | 3,300 psi | 13,200 psi | 0.06 | 4” up to 500’ | 0 to +180ºF |
| 1010AV | n n | 2 | 3/8 (9.5) | 0.72 (18.2) | 3.00 (76.2) | 3,300 psi | 13,200 psi | 0.11 | 4” up to 500’ | 0 to +180ºF |

2000AVS SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC PAINT HOSE

A high pressure, semi-conductive thermoplastic airless hose that will not swell or degrade when in contact with most paints, chemicals, oils, and solvents. Extremely durable and lightweight.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006AVS | n | 2 | 1/4 (6.4) | 0.51 (12.9) | 2.5 (63.5) | 4,000 psi | 16,000 psi | 0.07 | 4” up to 500’ | 0 to 180ºF |

1006AVB SERIES	FABRIC BRAID REINFORCEMENT THERMOPLASTIC LACQUER HOSE

A high pressure, semi-conductive thermoplastic airless hose that will not swell or degrade when used with most lacquer or solvent based coatins. Durable and lightweight.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

AIRLESS PAINT SPRAY

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006AVB35FXF | n | 2 | 1/4 (6.4) | 0.51 (12.9) | 2.5 (63.5) | 3,300 psi | 13,200 psi | 0.06 | N/A | 0 to 180ºF |

*Sold as 35’ assemblies only

5

1000UK SERIES	WIRE REINFORCED THERMOPLASTIC PAINT HOSE

A high pressure, high performance airless paint spray hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003K | n | 1 | 1/8 (3.2) | 0.27 (7.0) | 1.2 (30) | 5,075 psi | 20,300 psi | 0.04 | 4” up to 500’ | -40 to +212ºF |
| 1004UK | n n | 1 | 3/16 (4.8) | 0.37 (9.4) | 1.0 (25) | 5,250 psi | 21,000 psi | 0.08 | 4” up to 500’ | -40 to +212ºF |
| 1006UK | n n | 1 | 1/4 (6.4) | 0.46 (11.7) | 1.5 (37) | 5,000 psi | 18,000 psi | 0.11 | 4” up to 500’ | -40 to +212ºF |
| 1010UK | n n | 1 | 3/8 (9.5) | 0.60 (15.1) | 2.0 (50) | 3,500 psi | 13,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF |
| 1013UK | n n | 1 | 1/2 (12.7) | 0.72 (18.3) | 3.0 (75) | 3,000 psi | 12,000 psi | 0.20 | 4” up to 500’ | -40 to +212ºF |
| 1020UK | n | 1 | 3/4 (19.1) | 1.0 (25.4) | 6.7 (170) | 1,750 psi | 7,000 psi | 0.29 | 4” up to 300’ | -40 to +212ºF |
| 1025UK | n | 2 | 1 (25.4) | 1.3 (32.5) | 9.1 (231) | 2,750 psi | 11,000 psi | 0.65 | 4” up to 300’ | -40 to +212ºF |

1000UKT SERIES	WIRE REINFORCED TWIN LINE THERMOPLASTIC PAINT HOSE

A high pressure, high performance twin line airless paint spray hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006UKT | n | 1 | 1/4 (6.4) | 0.46 (11.7) | 1.5 (37) | 5,000 psi | 18,000 psi | 0.22 | 4” up to 200’ | -40 to +212ºF |
| 1010UKT | n | 1 | 3/8 (9.5) | 0.60 (15.1) | 2.0 (50) | 3,500 psi | 13,000 psi | 0.30 | 4” up to 200’ | -40 to +212ºF |
| 1013UKT | n | 1 | 1/2 (12.7) | 0.72 (18.3) | 3.0 (75) | 3,000 psi | 12,000 psi | 0.40 | 4” up to 200’ | -40 to +212ºF |

2000UKT SERIES	WIRE REINFORCED THERMOPLASTIC PAINT HOSE

A high pressure, high performance twin line hose with steel reinforcement braiding for strength and electrical conductivity. Offers reistantce to paints, chlorinated solvents and other chemicals, Extremely flexible. Cover is pin-pricked.

###### CONSTRUCTION:

Inner: Nylon 6 Liner

Outer: Abrasion and chemical resistant Polyurethane cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2010UKT | n n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 6,000 psi | 24,000 psi | 0.21 | 4” up to 200’ | -40 to +212ºF |
| 2013UKT | n n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 5,000 psi | 17,000 psi | 0.28 | 4” up to 200’ | -40 to +212ºF |

6

AIRLESS PAINT SPRAY

2000UK SERIES	WIRE REINFORCED THERMOPLASTIC PAINT HOSE

A high pressure, high performance Airless Paint Spray Hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner = 2006/2010/2013 Seamless Nylon 12 Liner = 2020/2025

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UK | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.0 (25) | 6,170 psi | 28,000 psi | 0.21 | 4” up to 500’ | -40 to +212ºF |
| 2010UK | n n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 6,000 psi | 24,000 psi | 0.28 | 4” up to 500’ | -40 to +212ºF |
| 2013UK | n n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 5,000 psi | 17,000 psi | 0.34 | 4” up to 300’ | -40 to +212ºF |
| 2020UK | n | 2 | 3/4 (19.1) | 1.07 (27.1) | 6.7 (170) | 3,100 psi | 13,000 psi | 0.50 | 4” up to 300’ | -40 to +212ºF |
| 2025UK | n | 3** | 1 (25.4) | 1.46 (37.1) | 10.0 (254) | 4,750 psi | 19,000 psi | 0.61 | 4” up to 300’ | -40 to +212ºF* |

2100UKN SERIES	WIRE & FABRIC REINFORCED THERMOPLASTIC PAINT  HOSE

*+158ºF for water based fluids

**Wire and Kevlar reinforced

High pressure, high performance Airless Paint Spray Hose with steel and Kevlar braid reinforcement to ensure mechanical strength. Superior resistance to most paints, chemicals, and solvents.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104UKN | n | 2 | 3/16 (4.8) | 0.45 (11.3) | 1.2 (32) | 8,000 psi | 32,000 psi | 0.11 | 4” up to 500’ | -40 to +212ºF* |
| 2106UKN | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF* |
| 2110UKN | n | 2 | 3/8 (9.5) | 0.68 (17.3) | 3.5 (89) | 8,000 psi | 32,000 psi | 0.22 | 4” up to 500’ | -40 to +212ºF* |
| 2113UKN | n | 2 | 1/2 (12.7) | 0.82 (20.8) | 5.0 (127) | 6,500 psi | 26,000 psi | 0.28 | 4” up to 300’ | -40 to +212ºF* |
| 2120UKN | n | 3 | 3/4 (19.1) | 1.16 (29.5) | 8.0 (203) | 6,500 psi | 26,000 psi | 0.45 | 4” up to 300’ | -40 to +212ºF* |

3000UK SERIES	WIRE & FABRIC REINFORCED THERMOPLASTIC PAINT HOSE

*+158ºF for water based fluids

High pressure, high performance Large

Bore Airless Paint Spray Hose with steel braid reinforcement to ensure superior resistance to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

7

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 3013UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 8,500 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |
| 3113UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 10,000 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE,  CALL 800-526-4066.

404UK SERIES	FABRIC REINFORCED THERMOPLASTIC PAINT HOSE

Medium pressure Airless Paint Spray Hose reinforced with polyester braids for paint spraying where anti-static properties are re- quired. Conductivity is by means of a conductive polymer strip. Resistant to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 404UK | n | 1 | 3/16 (4.8) | 0.38 (9.7) | 1.2 (30) | 1,900 psi | 6,960 psi | 0.04 | 4” up to 500’ | -40 to +212ºF |

FLUID HOSE SERIES	SYNTHETIC REINFORCED RUBBER LOW PRESSURE PAINT TRANSFER HOSE

Premium quality all purpose hose for handling a variety of applications such as paint or agricultural spray. Suitable for paints, lacquers, thinners, oil, and solvents.

###### CONSTRUCTION:

Inner: Nylon, RMA Class B Branded Synthetic Yarn

Outer: Synthetic Neoprene

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 55-1942-04 | n | 1 | 1/4 (6.7) | 0.50 (12.7) | 2.5 (63.5) | 750 psi | 3,000 psi | 0.08 | 500’* | -40 to +190ºF |
| 55-1942-10 | n | 1 | 5/16 (7.9) | 0.56 (14.3) | 2.8 (69.9) | 750 psi | 3,000 psi | 0.13 | 500’* | -40 to +190ºF |
| 55-1942-16 | n | 1 | 3/8 (9.5) | 0.68 (17.5) | 3.0 (76.2) | 750 psi | 3,000 psi | 0.15 | 500’* | -40 to +190ºF |
| 55-1942-25 | n | 1 | 1/2 (12.7) | 0.81 (20.6) | 4.5 (114.3) | 750 psi | 3,000 psi | 0.22 | 500’* | -40 to +190ºF |
| 55-1942-37 | n | 1 | 3/4 (19.0) | 1.19 (30.2) | 6.0 (152.4) | 750 psi | 3,000 psi | 0.36 | 500’* | -40 to +190ºF |
| 55-1942-43 | n | 1 | 1 (25.4) | 1.50 (38) | 7.0 (177.8) | 750 psi | 3,000 psi | 0.49 | 500’* | -40 to +190ºF |

*Coils may contain multiple lengths (max. 3)

100K PTFE SERIES	STAINLESS STEEL BRAID REINFORCED PTFE PAINT HOSE

High pressure, high performance PTFE lined hose with stainless steel braid reinforcement for strength and guaranteed electrical conductivity. Superior resistance to most chemicals, paints, and solvents. Lightweight and flexible in comparison to conventional multi-spirals.

###### CONSTRUCTION:

Inner: Seamless PTFE Liner

Outer: Abrasion Resistant Stainless Steel Braid Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 105K | n | 1 | 3/16 (4.8) | 0.31 (7.9) | 2.5 (63.5) | 3,000 psi | 12,000 psi | 0.07 | 4” up to 200’ | -65 to +450ºF |
| 106K | n | 1 | 1/4 (6.4) | 0.37 (9.4) | 3.0 (76) | 3,000 psi | 12,000 psi | 0.10 | 4” up to 200’ | -65 to +450ºF |
| 110K | n | 1 | 3/8 (9.5) | 0.51 (13.0) | 5.0 (127) | 2,000 psi | 8,000 psi | 0.11 | 4” up to 200’ | -65 to +450ºF |
| 113K | n | 1 | 1/2 (12.7) | 0.63 (16.0) | 6.5 (165.1) | 1,750 psi | 7,000 psi | 0.16 | 4” up to 200’ | -65 to +450ºF |
| 120K | n | 1 | 3/4 (19) | 0.88 (2.4) | 8.5 (216) | 1,000 psi | 4,000 psi | 0.20 | 4” up to 50’ | -65 to +450ºF |

8

AIRLESS PAINT SPRAY

200K PTFE SERIES	STAINLESS STEEL BRAID REINFORCED PTFE  PAINT HOSE

Very high pressure, high performance PTFE lined hose with stainless steel braid reinforcement for strength and guaranteed electrical conductivity. Superior resistance to most chemicals, paints, and solvents. Lightweight and flexible in comparison to conventional multi-spirals.

###### CONSTRUCTION:

Inner: Seamless PTFE Liner

Outer: Abrasion Resistant Stainless Steel Braid Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 206K | n | 2 | 1/4 (6.4) | 0.44 (11.4) | 3.0 (76) | 3,990 psi | 15,950 psi | 0.16 | 200” | -65 to +450ºF |
| 210K | n | 2 | 3/8 (9.5) | 0.60 (15.2) | 4.3 (108) | 3,260 psi | 13,050 psi | 0.24 | 4” up to 200’ | -65 to +450ºF |
| 213K | n | 2 | 1/2 (12.7) | 0.74 (18.8) | 5.3 (135) | 2,900 psi | 11,600 psi | 0.32 | 4” up to 100’ | -65 to +450ºF |
| 220K | n | 2 | 3/4 (19.0) | 1.0 (25.4) | 7.9 (200) | 2,180 psi | 8,700 psi | 0.45 | 4” up to 50’ | -65 to +450ºF |
| 225K | n | 2 | 1 (25.4) | 1.3 (33) | 13.0 (330.2) | 1,810 psi | 7,540 psi | 0.50 | 4” up to 50’ | -65 to +450ºF |

200ST FEP SERIES	POLYURETHANE COVERED STEEL BRAID REINFORCED FEP PAINT HOSE

Ultra high pressure, high performance FEP lined hose with steel spiral and braid reinforcement for strength and guaranteed electrical conductivity. Lightweight, flexible, with a tight bend radius and superior resistance to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless FEP Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 206ST | n | 3 | 1/4 (6.4) | 0.49 (12.4) | 2.5 (63.5) | 6,162 psi | 24,650 psi | 0.18 | 4” up to 100’ | -40 to +176ºF |
| 210ST | n | 3 | 3/8 (9.5) | 0.68 (17.3) | 4.5 (114.3) | 5,000 psi | 20,000 psi | 0.31 | 4” up to 50’ | -40 to +176ºF |
| 213ST | n | 3 | 1/2 (12.7) | 0.81 (20.6) | 5.5 (139.7) | 4,710 psi | 18,850 psi | 0.39 | 4” up to 50’ | -40 to +176ºF |
| 220ST | n | 3 | 3/4 (19.0) | 1.14 (29.0) | 8.0 (203.2) | 3,840 psi | 15,230 psi | 0.67 | 4” up to 50’ | -40 to +176ºF |
| 225ST | n | 3 | 1 (25.4) | 1.33 (34.0) | 9.4 (240) | 3,262 psi | 13,050 psi | 0.87 | 4” up to 50’ | -40 to +176ºF |

1000HB SERIES	FABRIC REINFORCED MOISTURE RESISTANT THERMOPLASTIC PAINT HOSE

Moisture resistant, high performance Airless Paint Spray Hose with fabric braid reinforcement. Resis- tant to most chemicals, solvents, and paints. Ideal for the transfer of aggressive mediums that are reactive to moisture.

###### CONSTRUCTION:

Inner: Polyaimide Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

9

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006HB | n | 1 | 1/4 (6.4) | 0.47 (11.8) | 1.4 (35) | 2,900 psi | 11,600 psi | 0.07 | 4” up to 300’ | -40 to +212ºF |
| 1010HB | n | 1 | 3/8 (9.5) | 0.63 (16.0) | 2.2 (55) | 2,537 psi | 10,150 psi | 0.09 | 4” up to 300’ | -40 to +212ºF |
| 1013HB | n | 1 | 1/2 (12.7) | 0.80 (20.3) | 3.0 (75) | 2,030 psi | 8,120 psi | 0.12 | 4” up to 300’ | -40 to +212ºF |
| 1020HB | n | 1 | 3/4 (19.1) | 1.04 (26.5) | 5.7 (145) | 1,305 psi | 5,220 psi | 0.25 | 4” up to 300’ | -40 to +212ºF |

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE,  CALL 800-526-4066.

2000UHB SERIES	WIRE REINFORCED MOISTURE RESISTANT THERMOPLASTIC PAINT HOSE

Moisture resistant high pressure, high performance hose with steel braid reinforcement for strength. Resistant to most chemicals, solvents, and paints. Ideal for the transfer of aggressive mediums that are reactive to moisture.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyester Elastomer Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1004UHB | n | 1 | 3/16 (4.8) | 0.37 (9.4) | 1.0 (25.4) | 5,250 psi | 21,000 psi | 0.08 | 4” up to 300’ | -40 to +212ºF |
| 2006UHB | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.0 (25) | 6,170 psi | 24,680 psi | 0.21 | 4” up to 300’ | -40 to +212ºF |
| 2010UHB | n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 5,100 psi | 20,400 psi | 0.28 | 4” up to 300’ | -40 to +212ºF |
| 2013UHB | n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 4,250 psi | 17,000 psi | 0.34 | 4” up to 300’ | -40 to +212ºF |
| 2020UHB | n | 2 | 3/4 (19.1) | 1.07 (27.1) | 6.7 (170) | 3,100 psi | 12,400 psi | 0.50 | 4” up to 300’ | -40 to +212ºF |
| 2120UHB | n | 3** | 3/4 (19.1) | 1.16 (29.5) | 8.0 (203) | 6,000 psi | 24,000 psi | 0.44 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

NYLON TUBING SERIES

LOW PRESSURE NYLON TUBING

Nylon tubing absorbs very little water so it will not become brittle or swell because of water. Resistant to solvents, alkalies, oils, greases, petroleum products, fungus, and molds.

Generally good resistance to dilute mineral and organic acids. Excellent bend radii.

CONSTRUCTION:

Extruded Nylon

**2 Kevlar and 1 wire reinforced

| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TN75-25 | Opaque | — | 1/4 (6.4) | 0.35 (8.9) | — | 250 psi | 1,000 psi | 0.03 | 100’ | -90 to +212ºF |
| TN75-38 | Opaque | — | 3/8 (9.5) | 0.50 (12.7) | — | 225 psi | 900 psi | 0.04 | 100’ | -90 to +212ºF |

Additional sizes are  available

| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FEP-20 | Opaque | — | 1/4 (6.4) | 0.38 (9.5) | — | 200 psi | 800 psi | 0.03 | 50’ | -275 to +500ºF |
| FEP-38 | Opaque | — | 3/8 (9.5) | 0.50 (12.7) | — | 150 psi | 600 psi | 0.04 | 50’ | -275 to +500ºF |

FEP TUBING SERIES

LOW PRESSURE FEP TUBING

FEP tubing has the lowest coefficient of friction of any solid material. Excellent electrical and weatherability properties. It can be used with virtually all industrial solvents, chemicals, and corrosive materials, even at elevated temperatures. Outstanding age resistance.

CONSTRUCTION:

Extruded FEP

Additional sizes are  available

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

ADAPTERS

PRESSURE GAUGES

HOSE JACKETS

10

11

AIRLESS PAINT SPRAY

At CRP, our highly trained customer service staff will be happy to discuss custom lengths and accommodate all of your Air Cascade Hose needs. And unlike other manufacturers, no order is too big or too small, so there’s never a required minimum. Most importantly, we understand how vital it is for your business that your hoses be shipped as quickly as possible. So rest assured, you can count on us for timeliness when it counts.

AIR CASCADE HOSE FEATURES:

- Working pressures from 1,750 psi to 10,000 psi

- Hose sizes from 1/8” to 1”

- Wide variety of fitting options — NPT, NPSM, and JIC in plated or Stainless Steel – male (M) or female (F)

- Lengths up to 500’

- Suitable for filling breathing tanks and air lines

12

1000UK SERIES	WIRE REINFORCED THERMOPLASTIC AIR CASCADE HOSE

High pressure, high performance hose with steel braid reinforcement for strength. Offers optimum resistance to most chemicals, gases, and air with no absorption.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003K | n | 1 | 1/8 (3.2) | 0.27 (7.0) | 1.2 (30) | 5,075 psi | 20,300 psi | 0.04 | 4” up to 500’ | -40 to +212ºF |
| 1004UK | n n | 1 | 3/16 (4.8) | 0.37 (9.4) | 1.0 (25) | 5,250 psi | 21,000 psi | 0.08 | 4” up to 500’ | -40 to +212ºF |
| 1006UK | n n | 1 | 1/4 (6.4) | 0.46 (11.7) | 1.5 (37) | 5,000 psi | 18,000 psi | 0.11 | 4” up to 500’ | -40 to +212ºF |
| 1010UK | n n | 1 | 3/8 (9.5) | 0.60 (15.1) | 2.0 (50) | 3,500 psi | 13,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF |
| 1013UK | n n | 1 | 1/2 (12.7) | 0.72 (18.3) | 3.0 (75) | 3,000 psi | 12,000 psi | 0.20 | 4” up to 500’ | -40 to +212ºF |
| 1020UK | n | 1 | 3/4 (19.1) | 1.0 (25.5) | 6.7 (170) | 1,750 psi | 7,000 psi | 0.29 | 4” up to 300’ | -40 to +212ºF |
| 1025UK | n | 2 | 1 (25.4) | 1.3 (32.5) | 9.1 (231) | 2,750 psi | 11,000 psi | 0.65 | 4” up to 300’ | -40 to +212ºF |

2000UK SERIES	WIRE REINFORCED THERMOPLASTIC AIR CASCADE HOSE

High pressure, high performance hose with steel braid reinforcement for strength. Offers optimum resistance to most chemicals, gases, and air with no absorption.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner = 2006/2010/2013 Seamless Nylon 12 Liner = 2020/2025

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UK | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.0 (25) | 6,170 psi | 28,000 psi | 0.21 | 4” up to 500’ | -40 to +212ºF |
| 2010UK | n n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 6,000 psi | 24,000 psi | 0.28 | 4” up to 500’ | -40 to +212ºF |
| 2013UK | n n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 5,000 psi | 17,000 psi | 0.34 | 4” up to 300’ | -40 to +212ºF |
| 2020UK | n n | 2 | 3/4 (19.1) | 1.07 (27.1) | 6.7 (170) | 3,100 psi | 13,000 psi | 0.50 | 4” up to 300’ | -40 to +212ºF |
| 2025UK | n | 3** | 1 (25.4) | 1.46 (37.1) | 10.0 (254) | 4,750 psi | 19,000 psi | 0.61 | 4” up to 300’ | -40 to +212ºF* |

2000AB SERIES

FABRIC REINFORCED THERMOPLASTIC BREATHING AND AIR CYLINDER CHARGING HOSE

*+158ºF for water based fluids

**2 Kevlar and 1 Wire Reinforced Braid

High pressure, high performance hose with Aramid yarn braid reinforcement designed for air cylinder charging and breathing air applications. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Food Grade Polyester Elastomer Tube Tube Liner materials comply with FDA Reg. 21 CFR 177.2600 and EU Directive 90/128 Annex II (Sec A&B) and Annex III

Outer: Abrasion Resistant Polyurethane Cover

3

AIR CASCADE SYSTEMS

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2004AB | n n | 1 | 3/16 (4.8) | 0.41 (10.4) | 1.5 (38.1) | 6,000 psi | 24,000 psi | 0.05 | 4” up to 500’ | -40 to +176ºF |
| 2006AB | n n | 1 | 1/4 (6.4) | 0.48 (12.2) | 2.0 (50.8) | 6,000 psi | 24,000 psi | 0.10 | 4” up to 500’ | -40 to +176ºF |

NOTE: This hose is designed for Air Charging Applications and should not be connected between air breathing masks and pressure regulators in breathing apparatus. Hose should not be used with explosive gases such as hydrogen and oxygen.

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	1

##### Reinflex’s Emergency Rescue Equipment Hoses are specifically designed just for the industry. In fact, several leading rescue equipment manufacturers are our satisfied customers. We provide custom sizes, fittings, and color options to cater to each and every tool manufacturer, even those that are often times unavailable anywhere else. Competitively priced, Reinflex’s Emergency Rescue Equipment Hose is suitable for high pressure applications in the toughest of environments.

EMERGENCY  RESCUE  HOSE FEATURES:

  - Working pressure from 1,000 psi to 10,000 psi

  - Hose sizes from 3/16” to 1”

  - Twin-line is welded (not taped or clamped together) for longer service life and durability

  - Many fitting combinations and color options to meet OEM specs

  - Custom lengths up to 500’

i

Low volumetric expansion providing excellent hydraulic response times

m

14

1000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC EMERGENCY RESCUE HOSE

All-purpose medium pressure hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for hydraulic oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F	SPECIFICATION: SAE 100R7

MINIMUM	DYNAMIC	MINIMUM

CRP PART		# OF	NOMINAL I.D.  NOMINAL O.D.	BEND RADIUS	WORKING		BURST	WEIGHT	MAXIMUM TEMPERATURE NUMBER	COLORS   BRAIDS    Inches (MM)	Inches (MM)		Inches (MM)	PRESSURE	PRESSURE	(LBS/FT)		LENGTH	RANGE

1004UKNC	n	1	3/16 (4.8)	0.41 (10.4)	1.5 (38)	3,000 psi	12,000 psi	0.05	4” up to 500’  -40 to +212ºF*

1006UKNC	n	1	1/4 (6.4)	0.49 (12.4)	2.0 (51)	2,750 psi	11,000 psi	0.07	4” up to 500’  -40 to +212ºF*

1010UKNC	n	1	3/8 (9.5)	0.64 (16.3)	2.5 (64)	2,250 psi	9,000 psi	0.10	4” up to 300’  -40 to +212ºF*

1013UKNC	n	1	1/2 (12.7)	0.78 (19.9)	3.2 (82)	2,000 psi	8,000 psi	0.14	4” up to 300’  -40 to +212ºF*

1020UKNC	n	1	3/4 (19.1)	1.03 (26.1)	7.0 (178)	1,250 psi	5,000 psi	0.20	4” up to 200’  -40 to +212ºF*

1025UKNC	n	1	1 (25.4)	1.28 (32.5)	8.0 (203)	1,000 psi	4,000 psi	0.25	4” up to 200’  -40 to +212ºF*

*+158ºF for water based fluids

2000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC EMERGENCY RESCUE HOSE

All-purpose medium pressure hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for hydraulic oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F	SPECIFICATION: SAE 100R8

MINIMUM	DYNAMIC	MINIMUM

CRP PART		# OF	NOMINAL I.D.  NOMINAL O.D.	BEND RADIUS	WORKING		BURST	WEIGHT	MAXIMUM TEMPERATURE NUMBER	COLORS   BRAIDS    Inches (MM)	Inches (MM)		Inches (MM)	PRESSURE	PRESSURE	(LBS/FT)		LENGTH	RANGE

2004UKNC	n	1	3/16 (4.8)	0.41 (10.4)	1.5 (38)	5,000 psi	20,000 psi	0.05	4” up to 500’  -40 to +212ºF*

2006UKNC	n	1	1/4 (6.4)	0.49 (12.4)	2.0 (51)	5,000 psi	20,000 psi	0.07	4” up to 500’  -40 to +212ºF*

2010UKNC	n	1	3/8 (9.5)	0.63 (15.9)	2.5 (64)	4,000 psi	16,000 psi	0.10	4” up to 500’  -40 to +212ºF*

2013UKNC	n	1	1/2 (12.7)	0.78 (19.9)	3.2 (82)	3,500 psi	14,000 psi	0.13	4” up to 300’  -40 to +212ºF*

2020UKNC	n	1	3/4 (19.1)	1.04 (26.5)	5.91 (150)	2,300 psi	9,200 psi	0.24	4” up to 300’  -40 to +212ºF*

*+158ºF for water based fluids

2000UK SERIES	WIRE REINFORCED THERMOPLASTIC EMERGENCY RESCUE HOSE

A high pressure, high performance hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of oils

and chemicals. Extremely durable. Cover is pin- pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6

Outer: Abrasion Resistant Polyurethane Cover

EMERGENCY RESCUE EQUIPMENT

COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F

MINIMUM	DYNAMIC	MINIMUM

CRP PART		# OF	NOMINAL I.D.  NOMINAL O.D.	BEND RADIUS	WORKING		BURST	WEIGHT	MAXIMUM TEMPERATURE NUMBER	COLORS   BRAIDS    Inches (MM)	Inches (MM)		Inches (MM)	PRESSURE	PRESSURE	(LBS/FT)		LENGTH	RANGE

2006UK	n	2	1/4 (6.4)	0.54 (13.7)	1.0 (25)	6,170 psi	28,000 psi	0.21	4” up to 500’ -40 to +212ºF

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO PLACE AN ORDER OR SPEAK WITH A CRP REPRESENTATIVE, CALL 800-526-4066.	15

2100UK SERIES	WIRE & FABRIC REINFORCED THERMOPLASTIC EMERGENCY RESCUE HOSE

High pressure, high performance hydraulic hose reinforced with a braid of steel and a polyaramid yard. Compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters.

Superior flexibility and bend radii.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106UK | nnnn | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF* |
| 2110UK | n | 2 | 3/8 (9.5) | 0.74 (18.8) | 3.8 (95) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |
| 3113UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 10,000 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

1000UKTNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC EMERGENCY RESCUE HOSE

All-purpose high pressure non-conductive twin line hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyaramid fiber. Compatible with hydraulic oils, mineral oils, phosphate esters, and many industrial gases.

Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATIONS: SAE 100R7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1010UKTNC | n | 1 | 3/8 (9.5) | 0.64 (16.3) | 2.5 (64) |  | 2,250 psi | 9,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |
| 1013UKTNC | n | 1 | 1/2 (12.7) | 0.78 (19.9) | 3.2 (82) |  | 2,000 psi | 8,000 psi | 0.28 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2000UKTNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC EMERGENCY RESCUE HOSE

All-purpose high pressure non-conductive twin line hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyaramid fiber. Compatible with hydraulic oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UKTNC | n | 1 | 1/4 (6.4) | 0.46 (11.7) | 2.0 (51) |  | 5,000 psi | 20,000 psi | 0.14 | 4” up to 200’ | -40 to +212ºF* |
| 2010UKTNC | n | 1 | 3/8 (9.5) | 0.63 (15.9) | 5.0 (127) |  | 4,000 psi | 16,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

16

2100UKT SERIES	WIRE & FABRIC REINFORCED TWIN LINE THERMOPLASTIC EMERGENCY RESCUE HOSE

High pressure, high performance hydraulic hose reinforced with a braid of steel and a polyaramid yard. Compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters.

Superior flexibility and bend radii.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106UKT |  | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.30 | 4” up to 200’ | -40 to +212ºF* |

NOTE: Other colors available  upon request.	*+158ºF for water based fluids

2100UKNC SERIES

FABRIC REINFORCED THERMOPLASTIC EMERGENCY RESCUE HOSE

All-purpose high pressure non-conductive hose suitable for many hydraulic and pneumatic systems. Reinforced with two braids of polyaramid fiber.

Compatible with hydraulic oils, grease, fuel oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibilty and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

7

EMERGENCY RESCUE EQUIPMENT

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104KNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.06 | 4” up to 300’ | -40 to +212ºF* |
| 2106UKNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.11 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	1

2100TNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC EMERGENCY RESCUE HOSE

All purpose high pressure non-conductive hose suitable for many hydraulic and pneumetic systems. Reinforced with two braids of polyamid fiber.

Compatable with hydraulic oils, grease, fuel oils, phosphat esters, and many industrial gases. Excellent flexiblity and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104TNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 300’ | -40 to +212ºF* |
| 2106KNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100RH SERIES

WIRE REINFORCED RUBBER EMERGENCY RESCUE HOSE

All purpose high pressure rubber hose for many hydraulic and pneumetic systems. Reinforced with two braids of high tensile wire. Compatable with hydraulic oils, grease, fuel oils, phosphat esters, and many industrial gases. Excellent flexiblity and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Synethic Rubber

Outer: Abrasion Resistant Rubber Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106RH | n | 2 | 1/4 (6.4) | 0.58 (14.7) | 4.0 (101.6) | 10,000 psi | 20,000 psi | 0.27 | 4” up to 300’ | -40 to +212ºF* |
| 2110RH | n | 2 | 3/8 (9.5) | 0.71 (18.0) | 4.0 (101.6) | 10,150 psi | 20,300 psi | 0.32 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

HOSE JACKETS

QUICK COUPLERS

18

19

EMERGENCY RESCUE EQUIPMENT

With more than 30 years experience, it’s no wonder Reinflex is often called upon by the air and fluid transfer industry. Our Air and Fluid Transfer Hoses are competitively priced and ideal for low pressure applications. Full reels are available or we can create custom assemblies specifically designed to suit

your application needs. The hoses are compatible with most re-usable fittings.

AIR AND FLUID TRANSFER HOSE FEATURES:

- Working pressure from 1,750 psi to 5,000 psi

- Hose sizes from 3/16” to 1”

- Custom assemblies

- Available in continuous lengths up to 200’

- Minimal change in length at operating pressures

- Suitable for a wide range of industrial and agricultural applications, including suction, syphon kits and paint pick up

20

AIR & FLUID TRANSFER

1004T SERIES	SYNTHETIC REINFORCED RUBBER LOW PRESSURE AIR & FLUID TRANSFER HOSE

Ideal for flexible air transfer for abrasives material blasting systems and other industrial, construction or agricultural applications. EPDM hose and tube with spiral yarn reinforcement.

###### CONSTRUCTION:

Inner: EPDM

Outer: EPDM

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | COIL SIZE | TEMPERATURE RANGE |
| 1004TYG | n n | 1 | 3/16 (4.8) | 0.44 (11.2) | N/A | 200 psi | N/A | 0.14 | 800’* | -40 to +190ºF |
| 1004TYY | n n | 1 | 3/16 (4.8) | 0.44 (11.2) | N/A | 200 psi | N/A | 0.14 | 800’* | -40 to +190ºF |

*Coils may contain multiple lengths (max. 3)

AIR HOSE SERIES	SYNTHETIC REINFORCED RUBBER LOW PRESSURE AIR & FLUID TRANSFER HOSE

Ideal for a wide range of applications including industrial, construction, and agricultural. Reinforced synthetic rubber with spiral synthetic yarn.

Medium oil resistance.

###### CONSTRUCTION:

Inner: Synthetic Rubber RMA Class C

Outer: Synthetic Rubber

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 55-1982-04 | n | 1 | 1/4 (6.4) | 0.50 (12.7) | N/A | 200 psi | 800 psi | 0.09 | 500’* | -40 to +190ºF |
| 55-1982-10 | n | 1 | 5/16 (7.9) | 0.58 (14.7) | N/A | 200 psi | 800 psi | 0.12 | 500’* | -40 to +190ºF |
| 55-1982-16 | n | 1 | 3/8 (9.5) | 0.67 (17.0) | N/A | 200 psi | 800 psi | 0.15 | 500’* | -40 to +190ºF |
| 55-1982-22 | n | 1 | 1/2 (12.7) | 0.81 (20.6) | N/A | 200 psi | 800 psi | 0.22 | 500’* | -40 to +190ºF |

*Coils may contain multiple lengths (max. 3)

FLUID HOSE SERIES	SYNTHETIC REINFORCED RUBBER LOW PRESSURE AIR & FLUID TRANSFER HOSE

Premium quality all purpose hose for handling a variety of applications such as paint or agricultural spray. Suitable for paints, lacquers, thinners, oil, and solvents.

###### CONSTRUCTION:

Inner: Nylon, RMA Class B Branded Synthetic Yarn

Outer: Synthetic Neoprene

1

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 55-1942-04 | n | 1 | 1/4 (6.7) | 0.50 (12.7) | 2.5 (63.5) | 750 psi | 3,000 psi | 0.08 | 500’* | -40 to +190ºF |
| 55-1942-10 | n | 1 | 5/16 (7.9) | 0.56 (14.3) | 2.8 (69.9) | 750 psi | 3,000 psi | 0.13 | 500’* | -40 to +190ºF |
| 55-1942-16 | n | 1 | 3/8 (9.5) | 0.68 (17.5) | 3.0 (76.2) | 750 psi | 3,000 psi | 0.15 | 500’* | -40 to +190ºF |
| 55-1942-25 | n | 2 | 1/2 (12.7) | 0.88 (22.4) | 4.5 (114.3) | 750 psi | 3,000 psi | 0.22 | 500’* | -40 to +190ºF |
| 55-1942-37 | n | 2 | 3/4 (19.0) | 1.19 (30.2) | 6.0 (152.4) | 750 psi | 3,000 psi | 0.36 | 500’* | -40 to +190ºF |
| 55-1942-43 | n | 2 | 1 (25.4) | 1.50 (38) | 7.0 (177.8) | 750 psi | 3,000 psi | 0.49 | 500’* | -40 to +190ºF |

*Coils may contain multiple lengths (max. 3)

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	2

1100UKH SERIES

FABRIC REINFORCED THERMOPLASTIC AIR & FLUID HOSE (100R7)

All-purpose medium pressure 100R7 hose that is compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters, as well as many industrial gases. Excellent flexibility and flex fatigue resistance. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1104UKH | n | 1 | 3/16 | (4.8) | 0.41 | (10.4) | 1.5 |  | (38) | 3,000 psi | 12,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 1106UKH | n | 1 | 1/4 | (6.4) | 0.49 | (12.4) | 2.0 |  | (51) | 2,750 psi | 11,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 1110UKH | n | 1 | 3/8 | (9.5) | 0.64 | (16.3) | 2.5 |  | (64) | 2,250 psi | 9,000 psi | 0.10 | 4” up to 500’ | -40 to +212ºF* |
| 1113UKH | n | 1 | 1/2 | (12.7) | 0.78 | (19.9) | 3.2 |  | (82) | 2,000 psi | 8,000 psi | 0.14 | 4” up to 500’ | -40 to +212ºF* |
| 1120UKH | n | 1 | 3/4 | (19.1) | 1.03 | (26.1) | 7.0 |  | (178) | 1,250 psi | 5,000 psi | 0.20 | 4” up to 300’ | -40 to +212ºF* |
| 1125UKH | n | 1 | 1 | (25.4) | 1.28 | (32.5) | 8.0 |  | (203) | 1,000 psi | 4,000 psi | 0.25 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

1200UKH SERIES

FABRIC REINFORCED THERMOPLASTIC AIR & FLUID HOSE (100R8)

All-purpose medium pressure 100R8 hose that is compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters, as well as many industrial gases. Excellent flexibility and flex fatigue resistance. Cover is pin-pricked standard.

MSHA cover available upon request.

CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1204UKH | n | 1 | 3/16 | (4.8) | 0.35 | (8.9) | 1.2 |  | (31) | 5,000 psi | 20,000 psi | 0.04 | 4” up to 500’ | -40 to +212ºF* |
| 1206UKH | n | 1 | 1/4 | (6.4) | 0.45 | (11.4) | 2.0 |  | (51) | 5,000 psi | 20,000 psi | 0.06 | 4” up to 500’ | -40 to +212ºF* |
| 1210UKH | n | 1 | 3/8 | (9.5) | 0.61 | (15.5) | 2.4 |  | (61) | 4,000 psi | 16,000 psi | 0.09 | 4” up to 500’ | -40 to +212ºF* |
| 1213UKH | n | 1 | 1/2 | (12.7) | 0.78 | (19.8) | 3.2 |  | (82) | 3,500 psi | 14,000 psi | 0.14 | 4” up to 500’ | -40 to +212ºF* |
| 1220UKH | n | 1 | 3/4 | (19.1) | 1.06 | (26.9) | 6.0 |  | (152) | 2,300 psi | 9,200 psi | 0.21 | 4” up to 300’ | -40 to +212ºF* |
| 1225UKH | n | 1 | 1 | (25.4) | 1.35 | (34.3) | 8.0 |  | (203) | 2,000 psi | 8,000 psi | 0.31 | 4” up to 300’ | -40 to +212ºF* |

NYLON TUBING SERIES

LOW PRESSURE NYLON TUBING

Nylon tubing absorbs very little water so it will not become brittle or swell because of water. Resistant to solvents, alkalies, oils, greases, petroleum products, fungus, and molds.

Generally good resistance to dilute mineral and organic acids. Excellent bend radii.

CONSTRUCTION:

Extruded Nylon

*+158ºF for water based fluids

| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TN75-25 | Opaque | N/A | 1/4 (6.4) | 0.35 (8.9) | N/A | 250 psi | 1,000 psi | 0.03 | 100’ | -90 to +212ºF |
| TN75-38 | Opaque | N/A | 3/8 (9.5) | 0.50 (12.7) | N/A | 225 psi | 900 psi | 0.04 | 100’ | -90 to +212ºF |

Additional sizes are  available

22

FEP TUBING SERIES

LOW PRESSURE FEP TUBING

FEP tubing has the lowest coefficient of friction of any solid material. Excellent electrical and weatherability properties. It can be used with virtually all industrial solvents, chemicals, and corrosive materials, even at elevated temperatures. Outstanding age resistance.

###### CONSTRUCTION:

Extruded FEP

| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FEP-20 | Opaque | — | 1/4 (6.4) | 0.38 (9.5) | — | 200 psi | 800 psi | 0.03 | 50’ | -275 to +500ºF |
| FEP-38 | Opaque | — | 3/8 (9.5) | 0.50 (12.7) | — | 150 psi | 600 psi | 0.04 | 50’ | -275 to +500ºF |

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

Additional sizes are available

ADAPTERS

HOSE JACKETS

AIR & FLUID TRANSFER

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE  ON  ALL  HOSES. TO PLACE AN ORDER OR SPEAK WITH A CRP REPRESENTATIVE, CALL 800-526-4066.

23

Reinflex’s High Pressure Hydraulic Hoses provide excellent hydraulic response times due to their low volumetric expansion. Unlike some other hose manufacturers, Reinflex offers a TRUE 4 to 1 safety factor on burst pressure. Our thermoplastic hose is lightweight, compact, and won’t deteriorate like rubber hoses. We carry hard to find and custom sizes that will fill in your gaps and prevent your customers from shopping elsewhere. And because CRP is customer service focused, no order is too big or too small, even if you’re ordering

just one hose. CRP Industries is a member of NAHAD.

HIGH PRESSURE HYDRAULIC HOSE FEATURES:

- Low to very high working pressures (up to 10,000 psi)

- UV resistant—does not deteriorate

- Standard fitting options including NPT, NPSM, and JIC in plated or Stainless Steel—male (M) or female (F); other fitting types are available

- Hose sizes from 3/32” up to 1”ID

- Suitable for general hydraulics, bolt torquing, and jacking

- Custom colors available

- Twin line hoses are welded, not taped or clamped

24

HIGH PRESSURE HYDRAULICS

1000MK SERIES	FABRIC REINFORCED THERMOPLASTIC MICRO-BORE HYDRAULIC HOSE

High pressure mini bore hose reinforced with synthetic yarn. Ideal for use with petroleum or synthetic hydraulic fluids and gases. Superior electrical conductivity. Resistant to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless Poly Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1002MK | n | 1 | 5/64 (2.0) | 0.2 (5.0) | 0.8 (20) | 6,825 psi | 27,300 psi | 0.04 | 4” up to 500’ | -40 to +150ºF |
| 1002MKP | n | 1 | 5/64 (2.0) | 0.2 (5.0) | 0.8 (20) | 6,890 psi | 27,550 psi | 0.04 | 4” up to 500’ | -40 to +150ºF |
| 1003MK | n | 1 | 1/8 (3.0) | 0.28 (7.0) | 1.6 (40) | 5,800 psi | 23,200 psi | 0.07 | 4” up to 500’ | -40 to +150ºF |
| 1004MKP | n | 1 | 5/32 (4.0) | 0.32 (8.0) | 1.4 (35) | 5,980 psi | 23,925 psi | 0.09 | 4” up to 500’ | -40 to +150ºF |
| 1004MK | n | 1 | 5/32 (4.0) | 0.34 (8.6) | 1.6 (40) | 6,380 psi | 25,520 psi | 0.09 | 4” up to 500’ | -40 to +150ºF |

1000UKH SERIES	FABRIC BRAID REINFORCED LOW TEMP THERMOPLASTIC HYDRAULIC HOSE (100R18)

All-purpose medium pressure 100R18 hose that is compatabile with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters, as well as many industrial gases. Offers improved flexiblity at low temperatures and can be used for lift truck applications. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Black Abrasion & Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1004UKH | n | 1 | 3/16 | (4.8) | 0.38 | (9.6) | 0.98 |  | (25) | 3000 PSI | 12,000 PSI | 0.04 | 4” up to 500’ | -67 to +212ºF* |
| 1006UKH | n | 2 | 1/4 | (6.4) | 0.48 | (12.2) | 1.4 |  | (35) | 3000 PSI | 12,000 PSI | 0.06 | 4” up to 500’ | -67 to +212ºF* |
| 1010UKH | n | 2 | 3/8 | (9.5) | 0.65 (16.6) |  | 1.8 (45) |  |  | 3000 PSI | 12,000 PSI | 0.11 | 4” up to 500’ | -67 to +212ºF* |
| 1013UKH | n | 2 | 1/2 | (12.7) | 0.91 (23.1) |  | 2.8 (70) |  |  | 3000 PSI | 12,000 PSI | 0.20 | 4” up to 500’ | -67 to +212ºF* |
| 1020UKH | n | 2 | 3/4 | (19.1) | 1.24 (31.4) |  | 5.1 |  | (130) | 3000 PSI | 12,000 PSI | 0.33 | 4” up to 500’ | -67 to +212ºF* |

*+158ºF for water based fluids

1100UKH SERIES

FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE (100R7)

All-purpose medium pressure 100R7 hose that is compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters, as well as many industrial gases. Excellent flexibility and flex fatigue resistance. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Abrasion and Chemical Resistant Polyurethane Cover

5

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1104UKH | n | 1 | 3/16 | (4.8) | 0.41 | (10.4) | 1.5 |  | (38) | 3,000 psi | 12,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 1106UKH | n | 1 | 1/4 | (6.4) | 0.49 | (12.4) | 2.0 |  | (51) | 2,750 psi | 11,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 1110UKH | n | 1 | 3/8 | (9.5) | 0.64 | (16.3) | 2.5 |  | (64) | 2,250 psi | 9,000 psi | 0.10 | 4” up to 500’ | -40 to +212ºF* |
| 1113UKH | n | 1 | 1/2 | (12.7) | 0.78 | (19.9) | 3.2 |  | (82) | 2,000 psi | 8,000 psi | 0.14 | 4” up to 500’ | -40 to +212ºF* |
| 1120UKH | n | 1 | 3/4 | (19.1) | 1.03 | (26.1) | 7.0 |  | (178) | 1,250 psi | 5,000 psi | 0.20 | 4” up to 300’ | -40 to +212ºF* |
| 1125UKH | n | 1 | 1 | (25.4) | 1.28 | (32.5) | 8.0 |  | (203) | 1,000 psi | 4,000 psi | 0.25 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	2

1200UKH SERIES

FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE (100R8)

All-purpose medium pressure 100R8 hose that is compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters, as well as many industrial gases. Excellent flexibility and flex fatigue resistance. Cover is pin-pricked standard.

MSHA cover available upon request.

CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1204UKH | n | 1 | 3/16 | (4.8) | 0.35 | (8.9) | 1.2 |  | (31) | 5,000 psi | 20,000 psi | 0.04 | 4” up to 500’ | -40 to +212ºF* |
| 1206UKH | n | 1 | 1/4 | (6.4) | 0.45 | (11.4) | 2.0 |  | (51) | 5,000 psi | 20,000 psi | 0.06 | 4” up to 500’ | -40 to +212ºF* |
| 1210UKH | n | 1 | 3/8 | (9.5) | 0.61 | (15.5) | 2.4 |  | (61) | 4,000 psi | 16,000 psi | 0.09 | 4” up to 500’ | -40 to +212ºF* |
| 1213UKH | n | 1 | 1/2 | (12.7) | 0.78 | (19.8) | 3.2 |  | (82) | 3,500 psi | 14,000 psi | 0.14 | 4” up to 500’ | -40 to +212ºF* |
| 1220UKH | n | 1 | 3/4 | (19.1) | 1.06 | (26.9) | 6.0 |  | (152) | 2,300 psi | 9,200 psi | 0.21 | 4” up to 300’ | -40 to +212ºF* |
| 1225UKH | n | 1 | 1 | (25.4) | 1.35 | (34.3) | 8.0 |  | (203) | 2,000 psi | 8,000 psi | 0.31 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

1000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC HYDRAULIC HOSE

All-purpose medium pressure non-conductive hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | BEND RADIUS Inches (MM) |  |  | MINIMUM WORKING PRESSURE | DYNAMIC BURST PRESSURE | MINIMUM WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1004UKNC | n | 1 | 3/16 | (4.8) | 0.41 | (10.4) | 1.5 |  | (38) | 3,000 psi | 12,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 1006UKNC | n | 1 | 1/4 | (6.4) | 0.49 | (12.4) | 2.0 |  | (51) | 2,750 psi | 11,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 1010UKNC | n | 1 | 3/8 | (9.5) | 0.64 | (16.3) | 2.5 |  | (64) | 2,250 psi | 9,000 psi | 0.10 | 4” up to 300’ | -40 to +212ºF* |
| 1013UKNC | n | 1 | 1/2 | (12.7) | 0.78 | (19.9) | 3.2 |  | (82) | 2,000 psi | 8,000 psi | 0.14 | 4” up to 300’ | -40 to +212ºF* |
| 1020UKNC | n | 1 | 3/4 | (19.0) | 1.03 | (26.1) | 7.0 |  | (178) | 1,250 psi | 5,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |
| 1025UKNC | n | 1 | 1 | (25.4) | 1.28 | (32.5) | 8.0 |  | (203) | 1,000 psi | 4,000 psi | 0.25 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

1000UKT SERIES	WIRE REINFORCED TWIN LINE THERMOPLASTIC HYDRAULIC HOSE

High pressure, high performance twin line hose reinforced with a steel braid. Guaranteed electrical conductivity. Resistant to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Standard-Plated or Stainless Steel, M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006UKT |  | 1 | 1/4 (6.4) | 0.46 (11.7) | 1.5 (37) | 5,000 psi | 18,000 psi | 0.22 | 4” up to 200’ | -40 to +212ºF |
| 1010UKT |  | 1 | 3/8 (9.5) | 0.60 (15.1) | 2.0 (50) | 3,500 psi | 13,000 psi | 0.30 | 4” up to 200’ | -40 to +212ºF |
| 1013UKT |  | 1 | 1/2 (12.7) | 0.72 (18.3) | 3.0 (75) | 3,000 psi | 12,000 psi | 0.40 | 4” up to 200’ | -40 to +212ºF |

26

HIGH PRESSURE HYDRAULICS

1000UKTNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC HYDRAULIC HOSE

All-purpose medium pressure non-conductive twin line hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1010UKTNC | n | 1 | 3/8 (9.5) | 0.64 (16.3) | 2.5 (64) |  | 2,250 psi | 9,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |
| 1013UKTNC | n | 1 | 1/2 (12.7) | 0.78 (19.9) | 3.2 (82) |  | 2,000 psi | 8,000 psi | 0.28 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2000UK SERIES	WIRE AND FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE

A high pressure, high performance hydraulic hose with wire and Kevlar braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2025UK | n | 3 | 1 (25.4) | 1.46 (37.1) | 10.0 (254) | 4,750 psi | 19,000 psi | 0.61 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

2000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC HYDRAULIC HOSE

All-purpose medium pressure non-conductive hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

7

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2004UKNC | n | 1 | 3/16 | (4.8) | 0.41 | (10.4) | 1.5 |  | (38) | 5,000 psi | 20,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 2006UKNC | n | 1 | 1/4 | (6.4) | 0.49 | (12.4) | 2.0 |  | (51) | 5,000 psi | 20,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 2010UKNC | n | 1 | 3/8 | (9.5) | 0.63 | (15.9) | 2.5 |  | (64) | 4,000 psi | 16,000 psi | 0.10 | 4” up to 500’ | -40 to +212ºF* |
| 2013UKNC | n | 1 | 1/2 | (12.7) | 0.78 | (19.9) | 3.2 |  | (82) | 3,500 psi | 14,000 psi | 0.13 | 4” up to 300’ | -40 to +212ºF* |
| 2020UKNC | n | 1 | 3/4 | (19.1) | 1.04 | (26.5) | 5.91 |  | (150) | 2,300 psi | 9,200 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	2

2000UKTNC SERIES	FABRIC BRAID REINFORCED TWIN LINE THERMOPLASTIC HYDRAULIC HOSE

All-purpose medium pressure non-conductive twin line welded hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UKTNC | n | 1 | 1/4 (6.4) | 0.46 (11.7) | 2.0 (51) |  | 5,000 psi | 20,000 psi | 0.14 | 4” up to 200’ | -40 to +212ºF* |
| 2010UKTNC | n | 1 | 3/8 (9.5) | 0.63 (15.9) | 5.0 (127) |  | 4,000 psi | 16,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100UK SERIES	WIRE & FABRIC BRAID REINFORCED THERMOPLASTIC HYDRAULIC HOSE

High pressure, high performance hydraulic hose reinforced with a braid of steel and polyaramid yarn. Compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters. Superior flexibility and bend radii.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106UK | nnnn | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF* |
| 2110UK | n | 2 | 3/8 (9.5) | 0.74 (18.8) | 3.8 (95) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100UKN SERIES	WIRE & FABRIC BRAID REINFORCED THERMOPLASTIC HYDRAULIC HOSE

High pressure, high performance hose with steel and Kevlar braid reinforcement to ensure mechanical strength. Superior resistance to most

paints, chemicals, and solvents. Cover is pin-pricked for gas applications.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2113UKN | n | 2 | 1/2 (12.7) | 0.82 (20.8) | 5.0 (127) | 6,500 psi | 26,000 psi | 0.28 | 4” up to 300’ | -40 to +212ºF* |
| 2120UKN | n | 3 | 3/4 (19.1) | 1.16 (29.5) | 8.0 (203) | 6,500 psi | 26,000 psi | 0.45 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

28

HIGH PRESSURE HYDRAULICS

2100UKT SERIES	WIRE & FABRIC BRAID TWIN LINE THERMOPLASTIC HYDRAULIC HOSE

High pressure, high performance twin line welded hose reinforced with a braid of steel and polyaramid yarn. Compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters. Superior flexibility and bend radii.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106UKT |  | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.30 | 4” up to 500’ | -40 to +212ºF* |

NOTE: Other colors available  upon request.	*+158ºF for water based fluids

2100TNC SERIES	FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE

All purpose high pressure non conductive hose suitable for many hydraulic and pneumetic systems. Reinforced with two braids of polyamid fiber.

Compatable with hydraulic oils, grease, fuel oils, phosphat esters, and many industrial gases. Excel- lent flexiblity and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104TNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 300’ | -40 to +212ºF* |
| 2106KNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

3000UK SERIES	WIRE & FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE

High pressure, high performance hose with steel and Kevlar braid reinforcement to ensure strength. Superior resistance to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

9

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 3013UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 8,500 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |
| 3113UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 10,000 psi | 32,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	2

2100UKNC SERIES	FABRIC REINFORCED THERMOPLASTIC HYDRAULIC HOSE

All-purpose high pressure non-conductive hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyamid fibers. Compatible with hydraulic oils, grease, fuel oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104KNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.06 | 4” up to 300’ | -40 to +212ºF* |
| 2106UKNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.11 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100RH SERIES

WIRE REINFORCED RUBBER HYDRAULIC HOSE

All purpose high pressure rubber hose for many hydraulic and pneumetic systems. Reinforced with two braids of high tensile wire. Compatable with hydraulic oils, grease, fuel oils, phosphat esters, and many industrial gases. Excellent flexiblity and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Synethic Rubber

Outer: Abrasion Resistant Rubber Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106RH | n | 2 | 1/4 (6.4) | 0.58 (14.7) | 4.0 (101.6) | 10,000 psi | 20,000 psi | 0.27 | 4” up to 300’ | -40 to +212ºF* |
| 2110RH | n | 2 | 3/8 (9.5) | 0.71 (18.0) | 4.0 (101.6) | 10,150 psi | 20,300 psi | 0.32 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

HOSE JACKETS

PRESSURE GAUGES

QUICK COUPLERS

30

31

HIGH PRESSURE HYDRAULICS

32

1000MK SERIES	FABRIC REINFORCED THERMOPLASTIC MICRO-BORE  HOSE

High pressure micro-bore hose reinforced with synthetic yarn. Ideal for use with petroleum or synthetic hydraulic fluids and gases. Resistant to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless Poly Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Standard-Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1002MK | n | 1 | 5/64 (2.0) | 0.2 (5.0) | 0.8 (20) | 6,825 psi | 27,300 psi | 0.04 | 4” up to 500’ | -40 to +150ºF |
| 1002MKP | n | 1 | 5/64 (2.0) | 0.2 (5.0) | 0.8 (20) | 6,890 psi | 27,550 psi | 0.07 | 4” up to 500’ | -40 to +150ºF |
| 1003MK | n | 1 | 1/8 (3.0) | 0.28 (7.0) | 1.6 (40) | 5,800 psi | 23,200 psi | 0.07 | 4” up to 500’ | -40 to +150ºF |
| 1004MKP | n | 1 | 5/32 (4.0) | 0.32 (8.0) | 1.4 (35) | 5,980 psi | 23,925 psi | 0.09 | 4” up to 500’ | -40 to +150ºF |
| 1004MK | n | 1 | 5/32 (4.0) | 0.34 (8.6) | 1.6 (40) | 6,380 psi | 25,520 psi | 0.09 | 4” up to 500’ | -40 to +150ºF |

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

BALL VALVES	PRESSURE GAUGES

MICRO-BORE HOSES

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE  ON  ALL  HOSES. TO PLACE AN ORDER OR SPEAK WITH A CRP REPRESENTATIVE, CALL 800-526-4066.

33

”

n

Reinflex is one of only a few suppliers of moisture resistant hose. Our hydro-bloc thermoplastic high pressure hose

is an excellent alternative to PTFE and is ideal for a wide variety of applications using simple to medium moisture sensitive products. We carry a wide range of fittings as well as offering custom fabrication options, so you can enjoy the convenience of one-stop shopping for all of your moisture resistant hose needs.

MOISTURE RESISTANT FEATURES:

  - Working pressures from low to high (up to 6,000 psi)

  - Hose sizes from 1/4” to 3/4”

  - Wide range of fittings including NPT, NPSM, and JIC

  - Custom fabrications available

  - Lengths up to 300’

  - Suitable for a wide variety of applications including caulking, foam, and polyurea

“

To expedite delivery, our orders are often drop shipped from CRP. So sometimes we don’t see products before the customer does

— but we trust CRP to get it right.

34

MOISTURE RESISTANT

1000HB SERIES	FABRIC REINFORCED THERMOPLASTIC MOISTURE RESISTANT HOSE

Moisture resistant high performance hose with fabric braid reinforcement. Resistant to most chemicals, solvents, and paint. Ideal for the transfer of aggressive mediums that are reactive to moisture.

###### CONSTRUCTION:

Inner: Polyaimide Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1006HB | n | 1 | 1/4 (6.4) | 0.47 (11.8) | 1.4 (35) |  | 2,900 psi | 11,600 psi | 0.07 | 4” up to 300’ | -40 to +212ºF |
| 1010HB | n | 1 | 3/8 (9.5) | 0.63 (16.0) | 2.2 (55) |  | 2,537 psi | 10,150 psi | 0.09 | 4” up to 300’ | -40 to +212ºF |
| 1013HB | n | 1 | 1/2 (12.7) | 0.80 (20.3) | 3.0 (75) |  | 2,030 psi | 8,120 psi | 0.12 | 4” up to 300’ | -40 to +212ºF |
| 1020HB | n | 1 | 3/4 (19.1) | 1.04 (26.5) | 5.7 (145) |  | 1,305 psi | 5,220 psi | 0.25 | 4” up to 300’ | -40 to +212ºF |

2000UHB SERIES	WIRE REINFORCED THERMOPLASTIC MOISTURE RESISTANT HOSE

Moisture resistant high pressure, high performance hose with steel braid reinforcement for strength and guaranteed electrical conductivity in the order of 1.0 ohms/meter. Resistant to most chemicals, solvents, and paint. Ideal for the transfer of aggressive mediums that are reactive to moisture.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyester Elastomer Non-Pin Pricked Cover

5

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1004UHB | n | 1 | 3/16 (4.8) | 0.37 (9.4) | 1.0 (25.4) | 5,250 psi | 21,000 psi | 0.21 | 4” up to 300’ | -40 to +212ºF |
| 2006UHB | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.0 (25) | 6,170 psi | 24,680 psi | 0.21 | 4” up to 300’ | -40 to +212ºF |
| 2010UHB | n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 5,100 psi | 20,400 psi | 0.28 | 4” up to 300’ | -40 to +212ºF |
| 2013UHB | n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 5,000 psi | 17,000 psi | 0.34 | 4” up to 300’ | -40 to +212ºF |
| 2020UHB | n | 2 | 3/4 (19.1) | 1.07 (27.1) | 6.7 (170) | 3,100 psi | 12,400 psi | 0.50 | 4” up to 300’ | -40 to +212ºF |
| 2120UHB | n | 3 | 3/4 (19.1) | 1.16 (29.5) | 8.0 (203) | 6,000 psi | 24,000 psi | 0.44 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	3

36

NON-CONDUCTIVE HOSE

1000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC HOSE

All-purpose medium pressure hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1004UKNC | n | 1 | 3/16 (4.8) | 0.41 (10.4) | 1.5 (38) |  | 3,000 psi | 12,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 1006UKNC | n | 1 | 1/4 (6.4) | 0.49 (12.4) | 2.0 (51) |  | 2,750 psi | 11,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 1010UKNC | n | 1 | 3/8 (9.5) | 0.64 (16.3) | 2.5 (64) |  | 2,250 psi | 9,000 psi | 0.10 | 4” up to 300’ | -40 to +212ºF* |
| 1013UKNC | n | 1 | 1/2 (12.7) | 0.78 (19.9) | 3.2 (82) |  | 2,000 psi | 8,000 psi | 0.14 | 4” up to 300’ | -40 to +212ºF* |
| 1020UKNC | n | 1 | 3/4 (19.1) | 1.03 (26.1) | 7.0 (178) |  | 1,250 psi | 5,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |
| 1025UKNC | n | 1 | 1 (25.4) | 1.28 (32.5) | 8.0 (203) |  | 1,000 psi | 4,000 psi | 0.25 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

1000UKTNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC HOSE

All-purpose high pressure non-conductive twin line hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyaramid fiber. Compatible with hydraulic oils, grease, fuel oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibilty and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1010UKTNC | n | 1 | 3/8 (9.5) | 0.64 (16.3) | 2.5 (64) |  | 2,250 psi | 9,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |
| 1013UKTNC | n | 1 | 1/2 (12.7) | 0.78 (19.9) | 3.2 (82) |  | 2,000 psi | 8,000 psi | 0.28 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2000UKNC SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC HOSE

All-purpose medium pressure hose reinforced with a single braid of polyester fiber. Ideal where non-conductivity is required. Suitable for grease, fuel oils, mineral oils, and phosphate esters. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

7

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2004UKNC | n | 1 | 3/16 | (4.8) | 0.41 | (10.4) | 1.5 |  | (38) | 5,000 psi | 20,000 psi | 0.05 | 4” up to 500’ | -40 to +212ºF* |
| 2006UKNC | n | 1 | 1/4 | (6.4) | 0.49 | (12.4) | 2.0 |  | (51) | 5,000 psi | 20,000 psi | 0.07 | 4” up to 500’ | -40 to +212ºF* |
| 2010UKNC | n | 1 | 3/8 | (9.5) | 0.63 | (15.9) | 2.5 |  | (64) | 4,000 psi | 16,000 psi | 0.10 | 4” up to 500’ | -40 to +212ºF* |
| 2013UKNC | n | 1 | 1/2 | (12.7) | 0.78 | (19.9) | 3.2 |  | (82) | 3,500 psi | 14,000 psi | 0.13 | 4” up to 300’ | -40 to +212ºF* |
| 2020UKNC | n | 1 | 3/4 | (19.1) | 1.04 | (26.5) | 5.91 |  | (150) | 2300 psi | 9200 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	3

2000UKTNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC HOSE

All-purpose high pressure non-conductive twin line hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyaramid fiber. Compatible with hydraulic oils, grease, fuel oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  | SPECIFICATION: SAE 100R8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UKTNC | n | 1 | 1/4 (6.4) | 0.46 (11.7) | 2.0 (51) |  | 5,000 psi | 20,000 psi | 0.14 | 4” up to 200’ | -40 to +212ºF* |
| 2010UKTNC | n | 1 | 3/8 (9.5) | 0.63 (15.) | 5.0 (127) |  | 4,000 psi | 16,000 psi | 0.20 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100UKNC SERIES	FABRIC REINFORCED THERMOPLASTIC HOSE

All-purpose high pressure non-conductive hose suitable for many hydraulic and pneumatic systems. Reinforced with a single braid of polyaramid fiber. Compatible with hydraulic oils, grease, fuel oils, mineral oils, phosphate esters, and many industrial gases. Excellent flexibility and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104KNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.06 | 4” up to 300’ | -40 to +212ºF* |
| 2106UKNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.11 | 4” up to 200’ | -40 to +212ºF* |

*+158ºF for water based fluids

2100TNC SERIES	FABRIC REINFORCED TWIN LINE THERMOPLASTIC HOSE

All purpose high pressure non-conductive hose suitable for many hydraulic and pneumetic systems. Reinforced with two braids of polyamid fiber.

Compatable with hydraulic oils, grease, fuel oils, phosphat esters, and many industrial gases. Excellent flexiblity and flex fatigue resistance.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2104TNC | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.2 (30.5) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 300’ | -40 to +212ºF* |
| 2106KNC | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

ADAPTERS

PRESSURE GAUGES

HOSE JACKETS

38

Non-Conductive

39

NON-CONDUCTIVE HOSE

From lightweight to steel wire reinforced, Reinflex’s PTFE and FEP Hoses are the ultimate moisture barrier and are non-clogging. Unlike many PTFE hoses on the market, ours are True Bore to ensure that the system performance meets flow rate specifications. In addition, Reinflex’s PTFE and FEP Hoses are available in very long lengths — another benefit that is atypical for the industry. We carry a multitude of end fittings, including plated and stainless steel that allow for quick shipping turnaround. Of course, custom lengths and fittings are also available depending on your application needs.

PTFE HOSE FEATURES:

- Working pressures from low to high (up to 6,162 psi)

- Hose sizes from 3/16” to 1”

- Fitting options include NPT, NPSM, and JIC in Stainless Steel, male (M) or female (F). (No need for adapters.)

- Lengths from 4” up to 200’

- Suitable for foams, caulks, and products that require heating or are moisture sensitive

40

100K SERIES	STAINLESS STEEL BRAID REINFORCED PTFE HOSE

High pressure, high performance PTFE hose with stainless steel braid reinforcement for strength and guaranteed electrical conductivity. Superior resistance to most chemicals, paints, and solvents. Lightweight and flexible in comparison to conventional multi-spirals.

###### CONSTRUCTION:

Inner: Seamless PTFE Liner

Outer: Abrasion Resistant Stainless Steel Braid Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 105K | n | 1 | 3/16 (4.7) | 0.31 (7.9) | 2.5 (64) | 3,000 psi | 12,000 psi | 0.07 | 4” up to 200’ | -65 to +450ºF |
| 106K | n | 1 | 1/4 (6.4) | 0.37 (9.4) | 3.0 (76) | 3,000 psi | 12,000 psi | 0.10 | 4” up to 200’ | -65 to +450ºF |
| 110K | n | 1 | 3/8 (9.5) | 0.51 (13.0) | 5.0 (127) | 2,000 psi | 8,000 psi | 0.11 | 4” up to 200’ | -65 to +450ºF |
| 113K | n | 1 | 1/2 (12.7) | 0.63 (16.0) | 6.5 (165) | 1,750 psi | 7,000 psi | 0.16 | 4” up to 200’ | -65 to +450ºF |
| 120K | n | 1 | 3/4 (19) | 0.88 (2.4) | 8.5 (216) | 1,000 psi | 4,000 psi | 0.20 | 4” up to 50’ | -65 to +450ºF |

200K SERIES	STAINLESS STEEL BRAID REINFORCED PTFE HOSE

Very high pressure, high performance PTFE lined hose with stainless steel braid reinforcement for strength and guaranteed electrical conductivity. Superior resistance to most chemicals, paints, and solvents. Lightweight and flexible in comparison to conventional multi-spirals.

###### CONSTRUCTION:

Inner: Seamless PTFE Liner

Outer: Abrasion Resistant Stainless Steel Braid Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 206K | n | 2 | 1/4 (6.4) | 0.44 (11.4) | 3.0 (76) | 3,990 psi | 15,950 psi | 0.16 | 4” up to 200’ | -65 to +450ºF |
| 210K | n | 2 | 3/8 (9.5) | 0.60 (15.2) | 4.25 (108) | 3,260 psi | 13,050 psi | 0.24 | 4” up to 200’ | -65 to +450ºF |
| 213K | n | 2 | 1/2 (12.7) | 0.74 (18.8) | 5.32 (135) | 2,900 psi | 11,600 psi | 0.32 | 4” up to 100’ | -65 to +450ºF |
| 220K | n | 2 | 3/4 (19.0) | 1.0 (25.4) | 7.9 (200) | 2,180 psi | 8,700 psi | 0.45 | 4” up to 50’ | -65 to +450ºF |
| 225K | n | 2 | 1 (25.4) | 1.3 (33) | 13.0 (330) | 1,810 psi | 7,540 psi | 0.50 | 4” up to 50’ | -65 to +450ºF |

200ST SERIES	STEEL BRAID REINFORCED FEP CORE THERMOPLASTIC HOSE

Ultra high pressure, high performance hose with steel braid and spiral reinforcement for strength and guaranteed electrical conductivity. Lightweight, flexible, and a tight bend radius, with superior resistance to most chemicals, paints, and solvents.

###### CONSTRUCTION:

Inner: Seamless FEP Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

1

PTFE HOSE

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 206ST | n | 3 | 1/4 (6.4) | 0.49 (12.4) | 2.5 (64) | 6,162 psi | 24,650 psi | 0.18 | 4” up to 100’ | -40 to +176ºF |
| 210ST | n | 3 | 3/8 (9.5) | 0.68 (17.3) | 4.5 (114) | 5,000 psi | 20,000 psi | 0.31 | 4” up to 50’ | -40 to +176ºF |
| 213ST | n | 3 | 1/2 (12.7) | 0.81 (20.6) | 5.5 (140) | 4,710 psi | 18,850 psi | 0.39 | 4” up to 50’ | -40 to +176ºF |
| 220ST | n | 3 | 3/4 (19.0) | 1.14 (29.0) | 8.0 (203) | 3,840 psi | 15,230 psi | 0.67 | 4” up to 50’ | -40 to +176ºF |
| 225ST | n | 3 | 1 (25.4) | 1.33 (34.0) | 9.4 (240) | 3,262 psi | 13,050 psi | 0.87 | 4” up to 50’ | -40 to +176ºF |

SANITARY  AND  CONVOLUTED  PTFE  HOSE  ASSEMBLIES  ARE  AVAILABLE. CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	4

##### Backed by OE quality, Reinflex’s Sewer Jetting Hoses meet the requirements for all sewer equipment. It is ideal for high pressure applications in even the most harsh sewer environments. Lightweight fabric or steel wire reinforced, our sewer jetting hose is available in many standard stocking assemblies, allowing us to drop ship anywhere quickly. For those requiring hose in long lengths, we are also able to provide speedy turnaround. In addition,

we can design custom fittings and fabrications based on your application needs, and of course, no job is too big or too small.

SEWER JETTING HOSE FEATURES:

  - Working pressures from low to very high (6,170 psi)

  - Fitting options — NPT, NPSM, and JIC in plated or Stainless Steel – male (M) or female (F)

  - Lengths from 4” up to 600’

  - Hose sizes (ID) from 1/8” to 1”

  - Provides excellent abrasion and fungus resistance

  - Meets Waste Equipment Technology Association (WETA) specifications

42

1000SJ SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC SEWER JETTING HOSE

Sewer Line Jetting Hose with high tensile fabric braid reinforcement. Provides excellent abrasion, cut, and fungus resistance. Meets Waste Equipment Technology Association specifications.

###### CONSTRUCTION:

Inner: Yellow Olefin Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) |  | NOMINAL O.D. Inches (MM) |  | MINIMUM BEND RADIUS Inches (MM) |  | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003SJ | n | 2 | 1/8 | (3.2) | 0.32 | (8.1) | 1.3 | (32) | 4,000 psi | 10,000 psi | 0.04 | 4” up to 600’ | -40 to +150ºF |
| 1004SJ | n | 2 | 3/16 | (4.8) | 0.41 | (10.4) | 1.8 | (45) | 4,000 psi | 10,000 psi | 0.05 | 4” up to 600’ | -40 to +150ºF |
| 1006SJ | n | 2 | 1/4 | (6.4) | 0.50 | (12.7) | 2.5 | (64) | 4,000 psi | 10,000 psi | 0.06 | 4” up to 600’ | -40 to +150ºF |
| 1010SJ | n | 2 | 3/8 | (9.5) | 0.65 | (16.5) | 3.0 | (76) | 4,000 psi | 10,000 psi | 0.11 | 4” up to 600’ | -40 to +150ºF |
| 1020SJ | n | 1 | 3/4 | (19.1) | 1.18 | (29.9) | 5.0 | (127) | 2,500 psi | 6,250 psi | 0.28 | 4” up to 600’ | -40 to +150ºF |
| 1025SJ | n | 1 | 1 | (25.4) | 1.46 | (37.1) | 6.0 | (152) | 2,500 psi | 6,250 psi | 0.40 | 4” up to 600’ | -40 to +150ºF |

1000AV SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC SEWER JETTING HOSE

A high pressure semi-conductive thermoplastic hose that will not swell or degrade when

in contact with most chemicals, oils, and solvents. Extremely durable. Ideal for where electrostatic charge must be dissipated and high temperature applications.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003AV | n | 1 | 1/8 (3.2) | 0.32 (8.1) | 1.3 (32) | 3,500 psi | 14,000 psi | 0.03 | 4” up to 500’ | 0 to 180ºF |
| 1004AV | n | 2 | 3/16 (4.8) | 0.43 (10.9) | 1.8 (45) | 3,500 psi | 14,000 psi | 0.05 | 4” up to 500’ | 0 to 180ºF |
| 1006AV | n | 2 | 1/4 (6.4) | 0.49 (12.5) | 2.5 (64) | 3,300 psi | 13,200 psi | 0.05 | 4” up to 500’ | 0 to 180ºF |
| 1010AV | n | 2 | 3/8 (9.5) | 0.72 (18.2) | 3.0 (76) | 3,300 psi | 13,200 psi | 0.11 | 4” up to 500’ | 0 to 180ºF |

2000AV SERIES	FABRIC BRAID REINFORCED THERMOPLASTIC SEWER JETTING HOSE

Large Bore Sewer Line Jetting Hose with high tensile fabric braid reinforcement. Provides excellent abrasion, cut, and fungus resistance meets Waste Equipment Technology Association specifications.

###### CONSTRUCTION:

Inner: Olefin Tube

Outer: Abrasion and Chemical Resistant Polyurethane Cover

3

SEWER JETTING

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2013AV | n | 2 | 1/2 (12.7) | 0.87 (22.1) | 3.0 (76) | 4,000 psi | 12,000 psi | 0.20 | 4” up to 600’ | -40 to +150ºF |
| 2020AV | n | 2 | 3/4 (19.1) | 1.18 (29.9) | 5.0 (127) | 3,000 psi | 7,500 psi | 0.29 | 4” up to 600’ | -40 to +150ºF |
| 2025AV | n | 2 | 1 (25.4) | 1.48 (37.6) | 8.0 (203) | 3,000 psi | 7,500 psi | 0.41 | 4” up to 600’ | -40 to +150ºF |

#### CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	4

1000UK SERIES	WIRE REINFORCED THERMOPLASTIC SEWER JETTING HOSE

A high pressure, high performance hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of chlorinated solvents, and other chemicals. Extremely durable. Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 1003K | n | 1 | 1/8 (3.2) | 0.27 (7.0) | 1.2 (30) | 5,075 psi | 20,300 psi | 0.04 | 4” up to 500’ | -40 to +212ºF |
| 1004UK | n n | 1 | 3/16 (4.8) | 0.37 (9.4) | 1.0 (25) | 5,250 psi | 21,000 psi | 0.08 | 4” up to 500’ | -40 to +212ºF |
| 1006UK | n n | 1 | 1/4 (6.4) | 0.46 (11.7) | 1.5 (37) | 5,000 psi | 18,000 psi | 0.11 | 4” up to 500’ | -40 to +212ºF |
| 1010UK | n n | 1 | 3/8 (9.5) | 0.60 (15.1) | 2.0 (50) | 3,500 psi | 13,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF |
| 1013UK | n n | 1 | 1/2 (12.7) | 0.72 (18.3) | 3.0 (75) | 3,000 psi | 12,000 psi | 0.20 | 4” up to 500’ | -40 to +212ºF |

2000UK SERIES	WIRE REINFORCED THERMOPLASTIC SEWER JETTING HOSE

A high pressure, high performance Sewer Jetting Hose with steel braid reinforcement for strength and guaranteed electrical conductivity. Offers optimum resistance to a wide range of paints, chlorinated solvents, and other chemicals. Extremely durable.

Cover is pin-pricked standard.

###### CONSTRUCTION:

Inner: Seamless Nylon 6 Liner = 2006/2010/2013 Seamless Nylon 12 Liner = 2020/2025

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2006UK | n | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.0 (25) | 6,170 psi | 28,000 psi | 0.21 | 4” up to 500’ | -40 to +212ºF |
| 2010UK | n n | 2 | 3/8 (9.5) | 0.67 (17.0) | 1.5 (37) | 6,000 psi | 22,000 psi | 0.28 | 4” up to 500’ | -40 to +212ºF |
| 2013UK | n n | 2 | 1/2 (12.7) | 0.80 (20.2) | 2.5 (62) | 5,000 psi | 17,000 psi | 0.34 | 4” up to 300’ | -40 to +212ºF |
| 2020UK | n | 2 | 3/4 (19.1) | 1.07 (27.1) | 6.7 (170) | 3,100 psi | 13,000 psi | 0.50 | 4” up to 300’ | -40 to +212ºF |
| 2025UK | n | 3** | 1 (25.4) | 1.46 (37.1) | 10.0 (254) | 4,750 psi | 19,000 psi | 0.61 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

**Wire and kevlar reinforced

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

ADAPTERS

BALL VALVES

PRESSURE GAUGES

QUICK COUPLERS

44

45

SEWER JETTING

W

Superior OE quality is just one feature that sets Reinflex’s Water Jetting Hose apart from its competitors. This extremely high pressure hose is also competitively priced. What’s more, many of our competitors don’t even carry the wide variety that we do. Many standard stocking assemblies, as well as custom fabrications, are available depending on your application needs in jackets ranging from lightweight to steel wire reinforced. Keep in mind that no order is too big or small for CRP to fill. That’s one of the benefits of dealing with a company that prides itself on being large enough to help, yet small enough to care.

WATER JETTING HOSE FEATURES:

    - Working pressure from low to very high (10,000 psi)

    - Fitting options include NPT, NPSM, and JIC in plated or Stainless Steel — male (M) or female (F)

    - Lengths from 4” up to 500’

    - Hose sizes from 1/4” to 1/2”

    - Suitable for oil and chemical plant applications

46

2100UK SERIES	WIRE & FABRIC BRAID REINFORCED THERMOPLASTIC WATER JETTING HOSE

High pressure, high performance hydraulic hose reinforced with a braid of steel and a polyaramid yard. Compatible with hydraulic oils, grease, fuel oils, mineral oils, and most phosphate esters. Superior flexibility and bend radii.

###### CONSTRUCTION:

Inner: Polyester Elastomer Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 2106UK | nnnn | 2 | 1/4 (6.4) | 0.54 (13.7) | 1.5 (38) | 10,000 psi | 40,000 psi | 0.15 | 4” up to 500’ | -40 to +212ºF* |
| 2110UK | n | 2 | 3/8 (9.5) | 0.74 (18.8) | 3.8 (95) | 10,000 psi | 40,000 psi | 0.24 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

3000UK SERIES	WIRE & FABRIC REINFORCED THERMOPLASTIC WATER JETTING HOSE

High pressure, high performance hose with steel and Kevlar braid reinforcement to ensure strength and guaranteed electrical conductivity resistance in the order of 1.0 ohms/meter. Superior resistance to most fluids and solvents.

###### CONSTRUCTION:

Inner: Seamless Nylon 12 Liner

Outer: Abrasion and Chemical Resistant Polyurethane Cover

| COUPLING CAPABILITIES: NPT, NPSM, JIC Plated or Stainless Steel – M or F |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRP PART NUMBER | COLORS | # OF BRAIDS | NOMINAL I.D. Inches (MM) | NOMINAL O.D. Inches (MM) | MINIMUM BEND RADIUS Inches (MM) | DYNAMIC WORKING PRESSURE | MINIMUM BURST PRESSURE | WEIGHT (LBS/FT) | MAXIMUM LENGTH | TEMPERATURE RANGE |
| 3013UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 8,500 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |
| 3113UK | n | 3 | 1/2 (12.7) | 0.90 (22.9) | 5.0 (127) | 10,000 psi | 34,000 psi | 0.33 | 4” up to 300’ | -40 to +212ºF* |

*+158ºF for water based fluids

### Reinflex also carries all the hose accessories you need. See pages 48-62 for details.

ADAPTERS

BALL VALVES

QUICK COUPLERS

7

WATER JETTING

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	4

##### When it comes to accessories, no one can match Reinflex’s extensive stock of OEM grade adapters, ball valves, fittings, pressure gauges, and quick disconnects. Many of our adapters are plated in stainless steel and all of our accessories’ male ends are chamfered. You can also count on our excellent customer service staff to see to it that all of your accessories are delivered quickly each and every time.

OUR COMPLETE ACCESSORIES LINE INCLUDES:

- High Pressure Adapters

- Ball Valves

- Hose Jacketing

- Pressure Gauges

- Quick Disconnects

48

9

ACCESSORIES

Adapters

When you are using a Reinflex high pressure hose, you want to be sure that any connections you make are equally

reliable. That’s where Reinflex high pressure adapters come in. Available in both plated steel and stainless steel, Reinflex adapters come in a variety of fitments and insure leak free connections and optimal flow. For added convenience and value, we even sell a pre-boxed assortment containing 17 of the most popular adapters in the industry.

plated  steel

HEX NIPPLE	MALE NPT TO MALE NPT, TAPERED ENDS 30° CHAMFER

PART #	SIDE A    SIDE B

| HN 12 | 1/8” | 1/8” |
| --- | --- | --- |
| HN 25 | 1/4” | 1/4” |
| HN 38 | 3/8” | 3/8” |
| HN 50 | 1/2” | 1/2” |
| HN 75 | 3/4” | 3/4” |
| HN 100 | 1” | 1” |

HEX REDUCING NIPPLE	MALE NPT TO MALE NPT, TAPERED ENDS 30° CHAMFER

PART #	SIDE A    SIDE B

HRN 1225	1/8”	1/4”

HRN 2538	1/4”	3/8”

HRN 2550	1/4”	1/2”

HRN 3850	3/8”	1/2”

HRN 3875	3/8”	3/4”

HRN 5075	1/2”	3/4”

HRN 5010	1/2”	1”

HRN 7510	3/4”	1”

MALE CONNECTOR	JIC MALE TO MALE NPT, 30° CHAMFER END

PART #	SIDE A    SIDE B

HNJ 0412	JIC#4	1/8”

HNJ 0425	JIC#4	1/4”

HNJ 0438	JIC#4	3/8”

HNJ 0450	JIC#4	1/2”

HNJ 0525	JIC#5	1/4”

HNJ 0625	JIC#6	1/4”

HNJ 0638	JIC#6	3/8”

HNJ 0650	JIC#6	1/2”

HNJ 0825	JIC#8	1/4”

HNJ 0838	JIC#8	3/8”

HNJ 0850	JIC#8	1/2”

HNJ 1275	JIC#12  3/4”

HNJ 1610	JIC#16    1”

PIPE SWIVEL UNION	FEMALE NPSM SWIVEL TO FEMALE NPSM SWIVEL

PART #	SIDE A    SIDE B

| PSU 2525 | 1/4” | 1/4” |
| --- | --- | --- |
| PSU 3838 | 3/8” | 3/8” |
| PSU 5050 | 1/2” | 1/2” |
| PSU 7575 | 3/4” | 3/4” |
| PSU 1010 | 1” | 1” |

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	4

HEX COUPLING

FEMALE NPT TO FEMALE NPT

PART #	SIDE A    SIDE B

HPC 1212	1/8”	1/8”

HPC 1225	1/8”	1/4”

HPC 2525	1/4”	1/4”

HPC 2538	1/4”	3/8”

HPC 2550	1/4”	1/2”

HPC 3838	3/8”	3/8”

HPC 3850	3/8”	1/2”

HPC 5050	1/2”	1/2”

HPC 5075	1/2”	3/4”

HPC 7575	3/4”	3/4”

HPC 7510	3/4”	1”

HPC 1010	1”	1”

ADAPTER

MALE NPT TO FEMALE NPT, 30° CHAMFER MALE END

| PART # | SIDE A | SIDE B |
| --- | --- | --- |
| PA 1212 | 1/8” | 1/8” |
| PA 1225 | 1/8” | 1/4” |
| PA 1238 | 1/8” | 3/8” |
| PA 2525 | 1/4” | 1/4” |
| PA 2538 | 1/4” | 3/8” |
| PA 2550 | 1/4” | 1/2” |
| PA 2575 | 1/4” | 3/4” |
| PA 3838 | 3/8” | 3/8” |
| PA 3850 | 3/8” | 1/2” |
| PA 3875 | 3/8” | 3/4” |
| PA 5050 | 1/2” | 1/2” |
| PA 5075 | 1/2” | 3/4” |
| PA 5010 | 1/2” | 1” |
| PA 7575 | 3/4” | 3/4” |
| PA 7510 | 3/4” | 1“ |
| PA 1010 | 1” | 1” |

HEX REDUCER BUSHING

MALE TO FEMALE, 30° CHAMFER MALE END

| PART # | SIDE A | SIDE B |
| --- | --- | --- |
| HRB 1225 | 1/8” | 1/4” |
| HRB 1238 | 1/8” | 3/8” |
| HRB 1250 | 1/8” | 1/2” |
| HRB 2538 | 1/4” | 3/8” |
| HRB 2550 | 1/4” | 1/2” |
| HRB 2575 | 1/4” | 3/4” |
| HRB 2510 | 1/4” | 1” |
| HRB 3850 | 3/8” | 1/2” |
| HRB 3875 | 3/8” | 3/4” |
| HRB 3810 | 3/8” | 1” |
| HRB 5075 | 1/2” | 3/4” |
| HRB 5010 | 1/2” | 1” |
| HRB 7510 | 3/4” | 1” |

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

50

TO PLACE AN ORDER OR SPEAK WITH A CRP REPRESENTATIVE, CALL 800-526-4066.

51

FEMALE ADAPTER UNION

FEMALE TO FEMALE SWIVEL, 30° CHAMFER MALE END

ACCESSORIES

| PART # | SIDE A | SIDE B |
| --- | --- | --- |
| FA 1212 | 1/8” | 1/8” |
| FA 1225 | 1/8” | 1/4” |
| FA 2512 | 1/4” | 1/8” |
| FA 2525 | 1/4” | 1/4” |
| FA 2538 | 1/4” | 3/8” |
| FA 3825 | 3/8” | 1/4” |
| FA 3838 | 3/8” | 3/8” |
| FA 3850 | 3/8” | 1/2” |
| FA 5038 | 1/2” | 3/8” |
| FA 5050 | 1/2” | 1/2” |
| FA 5075 | 1/2” | 3/4” |
| FA 7550 | 3/4” | 1/2” |
| FA 7575 | 3/4” | 3/4” |
| FA 7510 | 3/4” | 1” |
| FA 1075 | 1” | 3/4” |
| FA 1010 | 1” | 1” |

MALE ADAPTER UNION

MALE NPT TO FEMALE NPSM SWIVEL, 30° CHAMFER MALE END

| PART # | SIDE A | SIDE B |
| --- | --- | --- |
| MFA 1212 | 1/8” | 1/8” |
| MFA 1225 | 1/8” | 1/4” |
| MFA 2512 | 1/4” | 1/8” |
| MFA 2525 | 1/4” | 1/4” |
| MFA 2538 | 1/4” | 3/8” |
| MFA 3825 | 3/8” | 1/4” |
| MFA 3838 | 3/8” | 3/8” |
| MFA 3850 | 3/8” | 1/2” |
| MFA 5038 | 1/2” | 3/8” |
| MFA 5050 | 1/2” | 1/2” |
| MFA 5075 | 1/2” | 3/4” |
| MFA 7550 | 3/4” | 1/2” |
| MFA 7575 | 3/4” | 3/4” |
| MFA 7510 | 3/4” | 1” |

FEMALE JIC ADAPTER

FEMALE JIC SWIVEL TO MALE NPT

PART #	SIDE A    SIDE B

FAMJ 0425	JIC#4	1/4”

FAMJ 0525	JIC#5	1/4”

FAMJ 0625	JIC#6	1/4”

FAMJ 0638	JIC#6	3/8”

FAMJ 0838	JIC#8	3/8”

FAMJ 0850	JIC#8	1/2”

FAMJ 1275	JIC#12  3/4”

FEMALE JIC ADAPTER

FEMALE JIC SWIVEL TO FEMALE

PART #	SIDE A    SIDE B

FAFJ 0425	JIC#4	1/4”

FAFJ 0625	JIC#6	1/4”

FAFJ 0638	JIC#6	3/8”

FAFJ 0650	JIC#6	1/2”

FAFJ 0838	JIC#8	3/8”

FAFJ 0850	JIC#8	1/2”

FAFJ 1275	JIC#12  3/4”

MALE SWIVEL ELBOW

MALE NPT TO FEMALE NPSM SWIVEL, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

MPE 2525	1/4”	1/4”

MPE 2538	1/4”	3/8”

MPE 3825	3/8”	1/4”

MPE 3838	3/8”	3/8”

MPE 3850	3/8”	1/2”

MPE 5038	1/2”	3/8”

MPE 5050	1/2”	1/2”

MPE 5075	1/2”	3/4”

MPE 7550	3/4”	1/2”

MPE 7575	3/4”	3/4”

MPE 1075	1”	3/4”

MPE 1010	1”	1”

FEMALE SWIVEL ELBOW

FEMALE TO 90° FEMALE SWIVEL, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

FPE 2525	1/4”	1/4”

FPE 2538	1/4”	3/8”

FPE 3825	3/8”	1/4”

FPE 3838	3/8”	3/8”

FPE 3850	3/8”	1/2”

FPE 5038	1/2”	3/8”

FPE 5050	1/2”	1/2”

FPE 5075	1/2”	3/4”

FPE 7550	3/4”	1/2”

FPE 7575	3/4”	3/4”

FPE 1010	1”	1”

HEX HEAD PLUG

SOLID MALE END TO PLUG

PART #	SIDE A

HHP 12	1/8”

HHP 25	1/4”

HHP 38	3/8”

HHP 50	1/2”

HHP 75	3/4”

HHP 100	1”

HEX CAP

FEMALE NPT CAP

PART #	SIDE A

HC 12	1/8”

HC 25	1/4”

HC 38	3/8”

HC 50	1/2”

HC 75	3/4”

HC 100	1”

SQUARE HEAD PLUG

MALE NPT PLUG

PART #	SIDE A

SPP 12	1/8”

SPP 25	1/4”

SPP 38	3/8”

SPP 50	1/2”

SPP 75	3/4”

52

SPP 100	1”

53

ACCESSORIES

JIC CAP PLUG

MALE JIC PLUG

PART #	SIDE A

JC 12	JIC#2

JC 25	JIC#4

JC 30	JIC#5

JC 38	JIC#6

JC 50	JIC#8

JC 62	JIC#10

JC 75	JIC#12

UNION TEES

MALE NPT BY MALE NPT BY MALE NPT, 30° CHAMFER MALE END

JC 100	JIC#16

PART #	SIDE A SIDE B SIDE C

UPT 12	1/8”	1/8”	1/8”

UPT 25	1/4”	1/4”	1/4”

UPT 38	3/8”	3/8”	3/8”

UPT 50	1/2”	1/2”	1/2”

UPT 75	3/4”	3/4”	3/4”

FEMALE TEE

FEMALE NPT BY MALE NPT BY MALE NPT

UPT 100	1”	1”	1”

PART #	SIDE A SIDE B SIDE C

FPT 12	1/8”	1/8”	1/8”

FPT 25	1/4”	1/4”	1/4”

FPT 38	3/8”	3/8”	3/8”

FPT 50	1/2”	1/2”	1/2”

FPT 75	3/4”	3/4”	3/4”

FPT 100	1”	1”	1”

UNION TEE

FEMALE NPSM SWIVEL

PART #	SIDE A SIDE B SIDE C

UPTF 12	1/8”	1/8”	1/8”

UPTF 25	1/4”	1/4”	1/4”

UPTF 38	3/8”	3/8”	3/8”

UPTF 50	1/2”	1/2”	1/2”

UPTF 75	3/4”	3/4”	3/4”

FEMALE CROSS

FEMALE NPT ALL ENDS

UPTF 100	1”	1”	1”

PART #	SIDE A SIDE B SIDE C SIDE D

FC 12	1/8”	1/8”	1/8”	1/8”

FC 25	1/4”	1/4”	1/4”	1/4”

FC 38	3/8”	3/8”	3/8”	3/8”

FC 50	1/2”	1/2”	1/2”	1/2”

FC 75	3/4”	3/4”	3/4”	3/4”

FC 100	1”	1”	1”	1”

LIVE GUN SWIVEL

MALE NPT TO FEMALE NPSM SWIVEL, 30° CHAMFER MALE END

PART #

FSG2525

SIDE A SIDE B

1/4”

1/4”

FREE WORKING SWIVELS

30° CHAMFER MALE END

MALE NPT TO FEMALE NPT PART #	SIDE A SIDE B

FS 2525   1/4”   1/4”    Steel FS 3838   3/8”   3/8”    Steel FS 5050   1/2”   1/2”    Steel FS 7575   3/4”   3/4”    Steel FS 1010        1”    1”	Steel

FEMALE NPT BOTH SIDES PART #	SIDE A SIDE B

FSF 2525 1/4” 1/4” FSF 3838 3/8” 3/8” FSF 5050 1/2” 1/2” FSF 7575 3/4” 3/4” FSF 1010 1” 1”

MALE ELBOW

MALE NPT END TO 90° MALE NPT END, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

| ME 2525 | 1/4” | 1/4” |
| --- | --- | --- |
| ME 2538 | 1/4” | 3/8” |
| ME 3838 | 3/8” | 3/8” |
| ME 3850 | 3/8” | 1/2” |
| ME 5050 | 1/2” | 1/2” |
| ME 5075 | 1/2” | 3/4” |
| ME 7575 | 3/4” | 3/4” |
| ME 1010 | 1” | 1” |

TEE

ONE FEMALE NPT END, TWO MALE NPT ENDS, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B   SIDE C

| PT 25 | 1/4” | 1/4” | 1/4” |
| --- | --- | --- | --- |
| PT 38 | 3/8” | 3/8” | 3/8” |
| PT 50 | 1/2” | 1/2” | 1/2” |
| PT 75 | 3/4” | 3/4” | 3/4” |

MALE RUN TEE

TWO FEMALE NPT ENDS, ONE MALE NPT ENDS, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B   SIDE C

MPST 25	1/4”	1/4”	1/4”

MPST 38	3/8”	3/8”	3/8”

MPST 50	1/2”	1/2”	1/2”

MPST 75	3/4”	3/4”	3/4”

MPST 100	1”	1”	1”

HEX NIPPLE

MALE NPT TO MALE NPT, 30° CHAMFER MALE END

stainless  steel  –  316  grade

PART #	SIDE A    SIDE B

HN 12 SS	1/8”	1/8”

HN 25 SS	1/4”	1/4”

HN 38 SS	3/8”	3/8”

HN 50 SS	1/2”	1/2”

HN 75 SS	3/4”	3/4”

HN 100 SS	1”	1”

54

5

ACCESSORIES

HEX REDUCING NIPPLE

MALE NPT TO MALE NPT, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

HRN 1225 SS	1/8”	1/4”

HRN 2538 SS	1/4”	3/8”

HRN 2550 SS	1/4”	1/2”

HRN 3850 SS	3/8”	1/2”

HRN 3875 SS	3/8”	3/4”

HRN 5075 SS	1/2”	3/4”

HRN 7510 SS	3/4”	1”

HEX REDUCING BUSHING

MALE NPT TO FEMALE NPT, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B

| HRB 1225 SS | 1/8” | 1/4” |
| --- | --- | --- |
| HRB 2538 SS | 1/4” | 3/8” |
| HRB 3850 SS | 3/8” | 1/2” |
| HRB 5075 SS | 1/2” | 3/4” |
| HRB 7510 SS | 3/4” | 1” |

MALE ADAPTER UNION

MALE NPT TO FEMALE NPSM SWIVEL, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B

MFA 2525 SS	1/4”	1/4”

MFA 2538 SS	1/4”	3/8”

MFA 3825 SS	3/8”	1/4”

MFA 3838 SS	3/8”	3/8”

MFA 3850 SS	3/8”	1/2”

MFA 5050 SS	1/2”	1/2”

MFA 7575 SS	3/4”	3/4”

FEMALE ADAPTER UNION

FEMALE NPT TO FEMALE NPSM SWIVEL

PART #	SIDE A   SIDE B

| FA 2525 SS | 1/4” | 1/4” |
| --- | --- | --- |
| FA 3838 SS | 3/8” | 3/8” |

ADAPTER

MALE NPT TO FEMALE NPT RIGID, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B

| PA 1212 SS | 1/8” | 1/8” |
| --- | --- | --- |
| PA 1225 SS | 1/8” | 1/4” |
| PA 1238 SS | 1/8” | 3/8” |
| PA 2525 SS | 1/4” | 1/4” |
| PA 2538 SS | 1/4” | 3/8” |
| PA 3838 SS | 3/8” | 3/8” |
| PA 3850 SS | 3/8” | 1/2” |
| PA 3875 SS | 3/8” | 3/4” |
| PA 5075 SS | 1/2” | 3/4” |
| PA 5010 SS | 1/2” | 1” |

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	5

HEX COUPLING

FEMALE NPT TO FEMALE NPT

PART #	SIDE A    SIDE B

HPC 1212 SS	1/8”	1/8”

HPC 1225 SS	1/8”	1/4”

HPC 2525 SS	1/4”	1/4”

HPC 2538 SS	1/4”	3/8”

HPC 3838 SS	3/8”	3/8”

HPC 3850 SS	3/8”	1/2”

HPC 5050 SS	1/2”	1/2”

HPC 7575 SS	3/4”	3/4”

HPC 7510 SS	3/4”	1”

FEMALE TEE

FEMALE BY FEMALE BY FEMALE

PART #	SIDE A    SIDE B    SIDE C

FPT 25 SS	1/4”	1/4”	1/4”

FPT 38 SS	3/8”	3/8”	3/8”

FPT 50 SS	1/2”	1/2”	1/2”

FPT 75 SS	3/4”	3/4”	3/4”

UNION TEE

MALE NPT TO MALE NPT TO MALE NPT, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B    SIDE C

| UPT 25 SS | 1/4” | 1/4” | 1/4” |
| --- | --- | --- | --- |
| UPT 38 SS | 3/8” | 3/8” | 3/8” |

MALE ELBOW

90° MALE NPT TO MALE NPT, 30° CHAMFER MALE END

PART #	SIDE A   SIDE B

| ME 1225 SS | 1/8” | 1/4” |
| --- | --- | --- |
| ME 2525 SS | 1/4” | 1/4” |
| ME 2538 SS | 1/4” | 3/8” |
| ME 3838 SS | 3/8” | 3/8” |
| ME 5050 SS | 1/2” | 1/2” |
| ME 7575 SS | 3/4” | 3/4” |
| ME 1010 SS | 1” | 1” |

FEMALE ELBOW

90° FEMALE NPT TO FEMALE NPT

PART #	SIDE A   SIDE B

FE 2525 SS	1/4”	1/4”

FE 2538 SS	1/4”	3/8”

FE 5050 SS	1/2”	1/2”

FE 7575 SS	3/4”	3/4”

56

7

ACCESSORIES

MALE SWIVEL ELBOW

90° MALE NPT TO FEMALE NPSM SWIVEL, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

MPE 2525 SS	1/4”	1/4”

MPE 3838 SS	3/8”	3/8”

MPE 5050 SS	1/2”	1/2”

MPE 7575 SS	3/4”	3/4”

MALE/FEMALE ELBOW

90° MALE NPT TO FEMALE NPT, 30° CHAMFER MALE END

PART #	SIDE A    SIDE B

MFE 2525 SS	1/4”	1/4”

MFE 3838 SS	3/8”	3/8”

MFE 5050 SS	1/2”	1/2”

MFE 7575 SS	3/4”	3/4”

HEX HEAD PLUG

MALE NPT PLUG

PART #	SIZE

HHP 25 SS	1/4”

HHP 38 SS	3/8”

MALE CONNECTOR

MALE JIC TO MALE, 30° CHAMFER

PART #	SIDE A    SIDE B

HNJ 0425 SS	JIC#4	1/4”

HNJ 0525 SS	JIC#5	1/4”

FEMALE SWIVEL ELBOW

90° FEMALE TO FEMALE SWIVEL

PART #	SIDE A    SIDE B

FPE 7575 SS	3/4”	3/4”

MALE RUN TEE

TWO FEMALE ENDS, ONE MALE NPT END, 30° CHAMFER

PART #	SIDE A SIDE B SIDE C

MPST 25 SS	1/4”	1/4”	1/4”

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	5

ADAPTER KIT

STARTER ASSORTMENT OF ADAPTERS

| PART # | QTY | SIDE A | SIDE B | SIDE C | SEE PG. |
| --- | --- | --- | --- | --- | --- |
| HN 25 | 24 | 1/4” | 1/4” |  | 46 |
| HN 38 | 9 | 3/8” | 3/8” |  | 46 |
| HN 50 | 3 | 1/2” | 1/2” |  | 46 |
| HRN 2538 | 10 | 1/4” | 3/8” |  | 46 |
| HRN 3850 | 4 | 3/8” | 1/2” |  | 46 |
| FPT 25 | 5 | 1/4” | 1/4” | 1/4” | 53 |
| MFA 2525 | 4 | 1/4” | 1/4” |  | 48 |
| MFA 2538 | 4 | 1/4” | 3/8” |  | 48 |
| HC 25 | 6 | 1/4” |  |  | 49 |
| HC 38 | 4 | 3/8” |  |  | 49 |
| HC 50 | 2 | 1/2” |  |  | 49 |
| UPT 25 | 2 | 1/4” | 1/4” | 1/4” | 50 |
| FA 2525 | 4 | 1/4” | 1/4” |  | 52 |
| FA 3838 | 4 | 3/8” | 3/8” |  | 52 |
| HRB 2538 | 4 | 1/4” | 3/8” |  | 47 |
| HRB 3850 | 4 | 3/8” | 1/2” |  | 47 |
| PA 2538 | 2 | 1/4” | 1/4” |  | 52 |

58

9

ACCESSORIES

Ball Valves

If you need ball valves that offer the same quality as the Reinflex hoses in your applications and installations, look no further. The

male and female ball valve assemblies shown below work with paint, petroleum-based hydraulic fluids, fuel oil, and air.

Reinflex’s male-thread ball valves use carbon steel for the body, hard chrome plated carbon steel for the ball, and zinc plated steel for the stem. Connections are male NPT, full bore, and the valves are rated to a working pressure of 7,500 PSI. Our female-thread ball valves have carbon steel or stainless steel bodies, hard chrome plated carbon steel or stainless steel balls, zinc plated steel or stainless steel stems, and Viton or PTFE seals. Connection is female NPT, full bore. For working pressures, please see chart below.

If you require other ball valve construction types, talk to a CRP service specialist who will be happy to discuss the other options available.

BALL VALVES MALE THREADS

BODY: FORGED CARBON STEEL STEM: FORGED CARBON STEEL BALL: CHROME PLATED STEEL

| PART # | SIDE A | SIDE B | CONNECTIONS | WORKING PRESSURE | BALL SEALS | STEM SEALS |
| --- | --- | --- | --- | --- | --- | --- |
| BV 2525 | 1/4” | 1/4” | Male NPT | 7,250 psi | Delrin | Viton |
| BV 3838 | 3/8” | 3/8” | Male NPT | 7,250 psi | Delrin | Viton |

BALL VALVES FEMALE THREADS

BODY: FORGED CARBON STEEL STEM: FORGED CARBON STEEL BALL: CHROME PLATED STEEL

| PART # | SIDE A SIDE B | CONNECTIONS | WORKING PRESSURE | BALL SEALS | STEM SEALS |
| --- | --- | --- | --- | --- | --- |
| BVF 2525 | 1/4” 1/4” | Female NPT | 7,250 psi | Delrin | Viton |
| BVF 3838 | 3/8” 3/8” | Female NPT | 7,250 psi | Delrin | Viton |
| BVF 5050 | 1/2” 1/2” | Female NPT | 7,250 psi | Delrin | Viton |
| BVF 7575 | 3/4” 3/4” | Female NPT | 5,800 psi | Delrin | Viton |
| BVF 1010 | 1” 1” | Female NPT | 5,800 psi | Delrin | Viton |
| BVF 2525 T | 1/4” 1/4” | Female NPT | 4,500 psi | Viton | PTFE |
| BVF 3838 T | 3/8” 3/8” | Female NPT | 4,500 psi | Viton | PTFE |
| BVF 5050 T | 1/2” 1/2” | Female NPT | 4,500 psi | Viton | PTFE |
| BVF 7575 T | 3/4” 3/4” | Female NPT | 3,600 psi | Viton | PTFE |
| BVF 1010 T | 1” 1” | Female NPT | 3,600 psi | Viton | PTFE |

NOTE: Replacement Handles and Seal Kits Available.

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	5

# Hose Jacketing

##### Reinflex is proud to offer Expando PT hose jacketing. Expando PT is a self-fitting protective oversleeve, with expandable

braid construction, designed to offer maximum resistance to abrasion and impact damage. Its open weave construction allows easy identification of contents. An additional advantage offered by these lightweight oversleeves is that each size fits several diameters.

Expando PT is easy to apply, has an operating temperature range from -40ºF to +125ºF, and is up to 4 times more abrasion resistant than competitive products. Additional features like flexibility, choice of colors, and resistance to most industrial fluids round out Expando PT’s advantages.

HOSE JACKETING

SELF–FITTING PROTECTIVE OVERSLEEVE

| PART # (COLOR) | SLEEVE SIZE | EXPANDED SIZE | FEET PER SPOOL | WEIGHT PER 100 FT. (LBS.) |
| --- | --- | --- | --- | --- |
| JACKET 012 (color) | 1/8” | 1/4” | 2,000 | 0.16 |
| JACKET 025 (color) | 1/4” | 7/16” | 1,000 | 0.24 |
| JACKET 050 (color) | 1/2” | 3/4” | 500 | 0.77 |
| JACKET 075 (color) | 3/4” | 1 1/4” | 500 | 1.20 |
| JACKET 125 (color) | 1 1/4” | 1 3/4” | 250 | 1.60 |
| JACKET 175 (color) | 1 3/4” | 2 3/4” | 250 | 2.70 |
| JACKET 200 (color) | 2” | 3 1/2” | 250 | 3.30 |
| JACKET 250 (color) | 2 1/2” | 4 1/2” | 250 | 4.60 |

*Always specify hose jacket color when ordering.

60

1

ACCESSORIES

Pressure Gauge

Reinflex’s Pressure Gauge line up comes in a variety of pressure ranges, dial and connection sizes, and mounting configurations.

For added durability and accuracy in more extreme environments, glycerine filled gauges are also available.

PRESSURE GAUGE

GLASS-FACED

| PART # PRESSURE CONNECTION RANGE (PSI) (MALE) | MOUNT | DIAL DIAMETER | METAL FINISH | LIQUID FILLED |
| --- | --- | --- | --- | --- |
| PG15-200B 0 to 15 1/8” | Back | 2” | Black | No |
| PG30-150 0 to 30 1/8” | Bottom | 1 1/2” | Black | No |
| PG60-200 0 to 60 1/4” | Bottom | 2” | Black | No |
| PG60-200B 0 to 60 1/4” | Back | 2” | Black | No |
| PG100-150 0 to 100 1/8” | Bottom | 1 1/2” | Black | No |
| PG100-150B 0 to 100 1/8” | Back | 1 1/2” | Black | No |
| PG100-200 0 to 100 1/4” | Bottom | 2” | Black | No |
| PG100-200B 0 to 100 1/4” | Back | 2” | Black | No |
| PG100-200-8 0 to 100 1/8” | Bottom | 2” | Black | No |
| PG160-200 0 to 160 1/4” | Bottom | 2” | Black | No |
| PG160-200B 0 to 160 1/4” | Back | 2” | Black | No |
| PG200-150 0 to 200 1/8” | Bottom | 1 1/2” | Black | No |
| PG200-150B 0 to 200 1/8” | Back | 1 1/2” | Black | No |
| PG200-200 0 to 200 1/4” | Bottom | 2” | Black | No |
| PG100-250 0 to 100 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG300-250 0 to 300 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG1000-250 0 to 1,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG1500-250 0 to 1,500 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG2000-250 0 to 2,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG3000-250 0 to 3,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG5000-250 0 to 5,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG6000-250 0 to 6,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |
| PG10K-250 0 to 10,000 1/4” | Bottom | 2 1/2” | Stainless | Yes |

PRESSURE TEST GAUGE KIT

INCLUDES PSI GAUGE, TEE, SWIVEL AND HEX NIPPLE

PART #		PRESSURE CONNECTION MOUNT		DIAL	METAL	LIQUID RANGE (PSI)	(MALE)	DIAMETER  FINISH	FILLED

PG5000-KIT   0 to 5,000	1/4”	Bottom	2 1/2”	Stainless	Yes

(Gauge) Plated Steel (Fittings)

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	6

# Quick Disconnects

##### The fastest and simplest way to connect and disconnect hoses are these quick couplers. For use in spray

applications, the working pressure rating for the 1/4” coupler is 5,000 psi; for the 3/8” option, 4,000 psi; and for the 1/2”, 4,000 psi. Viton O-rings allow use with most paints and solvents. We can also source non-standard couplers, call for details.

QUICK COUPLERS

62

FEMALE ENDS	MALE ENDS

PART #

QC 25F QC 38F QC 50F

SIZE

1/4”

3/8”

1/2”

PART #

QC 25M QC 38M QC 50M

SIZE

1/4”

3/8”

1/2”

NIPPLES

FEMALE ENDS	MALE ENDS

| PART # | SIZE | PART # | SIZE |
| --- | --- | --- | --- |
| QC 25FN | 1/4” | QC 25MN | 1/4” |
| QC 38FN | 3/8” | QC 38MN | 3/8” |
| QC 50FN | 1/2” | QC 50MN | 1/2” |

DOUBLE SHUT OFF SERIES

ALL FEMALE ENDS

QUICK COUPLERS	NIPPLES

| PART # | SIZE | PART # | SIZE |
| --- | --- | --- | --- |
| QCDS 25F | 1/4” | QCDS 25FN | 1/4” |
| QCDS 38F | 3/8” | QCDS 38FN | 3/8” |

10,000 PSI QUICK COUPLERS

COUPLERS

PART #	SIZE	BRAND

QCS18M10KP      1/8” Male NPT	Pioneer QCS25M10KP      1/4” Male NPT	Pioneer QCS38M10KP      3/8” Male NPT	Pioneer

NIPPLES

PART #	SIZE	BRAND

QCS18MN10KP     1/8” Male NPT	Pioneer QCS25MN10KP     1/4” Male NPT	Pioneer QCS25FN10KP 1/4” Female NPT Pioneer QCS38FN10KP      3/8” Female NPT      Pioneer

3

NOTES

High Pressure Hose Safety

CRP Industrial is very concerned about your safety when working with High Pressure Hose and related equipment. We have compiled this list of best practices for your reference and invite you to suggest any other safety measures that you have found valuable in your experience with high pressure hose and equipment.

Best Practices for High Pressure Hose Safety

- Stay within the stated working pressure of hose or equipment — never exceed this pressure.

- Work within the “minimum bend radius” guidelines suggested. Exceeding this radius could cause a kink that will lead to hose failure.

- Stay within recommended operating temperature limits of both hose and equipment. Higher temperatures increase pressure, which can cause failure.

- Inspect hoses prior to every use for signs of wear such as cuts, abrasions, coupling integrity, etc.

- Do not interline Reinflex hose with that of another manufacturer. Differences in manufacturing tolerances and quality standards could result in the failure of the weaker hose.

- Route hose away from areas of high temperature such as exhaust manifolds, heaters, or burners.

- Never run over hose with a vehicle or equipment, as this can compromise the integrity of the hose in a way that might not be visible.

- To avoid uncontrolled and potentially dangerous static discharges, be sure that all equipment is properly grounded. If you have any comments or questions about these guidelines or hose safety in general, please call your CRP representative.

CUSTOM SIZES AND OTHER COUPLING CAPABILITIES AVAILABLE ON ALL HOSES.

TO  PLACE AN  ORDER OR  SPEAK WITH A  CRP REPRESENTATIVE, CALL 800-526-4066.	6

NOTES

64

65

NOTES

NOTES

66

67

NOTES

CRP Industries Inc. 35 Commerce Drive

Cranbury, NJ 08512 USA

Toll Free 800.526.4066

Phone 609.578.4100

Telefax 609.578.4112 industrialcs@crpindustries.com

USA I MEXICO

www.crpreinflex.com

CATIND18